Modifications of 9202/076 Issue 1 – DCR # ??

- Change #1 :

For MH1RT ASIC family, Atmel proposes its packages with lid connected to ground.

Reason of change :

The note ‘lid is connected to ground’ is not specified in the detail specification 9202/076.

Description of change:

In §1.7.1 MQFPF196, §1.7.2 MQFPF256, §1.7.3 MQFPT352, §1.7.4 MCGA349 and §1.7.5 MCGA472,

Add note 3. lid is connected to ground.

- Change #2 :

§1.4.1 ESCC component number

‘XYZ’ has to stay a code with 3 digits allocated to Atmel.

Reason of change :

In 9202/076, ‘X’ is a single letter allocated by ESCC executive, and only 2 digits (‘YZ’) are allocated to Atmel to identify the ASIC.

Atmel needs 3 digits to identify its ASIC : ‘X’ indicates the country, and ‘YZ’ is a chrono. For instance ‘FPK’ is Atmel identification for ‘HBRISC2’.

Description of change:

replace

- Manufacturer Specific ASIC Identification: XYZ (as applicable) where:

X : Single letter allocated by the ESCC Executive to the manufacturer and each complete

code registered with the ESCC Executive Secretariat.

YZ : Individual 2 character code allocated by the Manufacturer to a specific ASIC design.

By:

- Manufacturer Specific ASIC Identification: XYZ.

- Change #3 :

§1.5. Maximum ratings : Clarification expected

Reason of change :

To avoid confusion between supply pin and other input pins

Description of change:

Add in line ‘supply current’, row ’remarks’: ‘Each supply pin’

- Change #4 :

§1.5. Maximum ratings : Mismatch about notes

Reason of change :

Note 3 is not suitable for ‘thermal resistance’

Note 4 is not complete.

Description of change:

- Add note 3

3. Rth(j-c) is given for the smallest available matrix which corresponds to the worst case (highest junction-to-case resistance).

- Rename Note 3 in Note 4 (suitable for ‘soldering temperature’), add note 4 is applicable for MQFP packages.

- Change #5 :

§1.7.4 MCGA349

‘A’ is missing in bottom view description.

- Change #6 :

§2.3.1. Room temperature electrical measurements : VIL, VIH, VT

Reason of change :

Information is missing to be homogeneous with Atmel MH1RT datasheet.

Description of change:

§2.3.1.1 (VDD=2.5V)

Add:

| Low level input voltage | VIL | - | CMOS buffers VDD=2.3V |  | 0.3 VDD | V |
| --- | --- | --- | --- | --- | --- | --- |
| High level input voltage | VIH | - | CMOS buffers VDD=2.7V | 0.7 VDD |  | V |
| Threshold Schmitt trigger input voltage | VT+ | - | Note 5 | 1.06 | 1.61 | V |
| Threshold Schmitt trigger input voltage | VT- | - | Note 5 | 0.78 | 1.25 | V |
| Hysteresis | Delta VT | - | Note 5 | 0.25 |  | V |

§2.3.1.2 (VDD=3V)

Add:

| Low level input voltage | VIL | - | CMOS buffers VDD=2.7V |  | 0.8 | V |
| --- | --- | --- | --- | --- | --- | --- |
| High level input voltage | VIH | - | CMOS buffers VDD=3.3V | 2 |  | V |
| Threshold Schmitt trigger input voltage | VT+ | - | Note 5 | 1.25 | 1.93 | V |
| Threshold Schmitt trigger input voltage | VT- | - | Note 5 | 0.90 | 1.42 | V |
| Hysteresis | Delta VT | - | Note 5 | 0.31 |  | V |

§2.3.1.3 (VDD=3.3V)

Add:

| Low level input voltage | VIL | - | CMOS buffers VDD=3V |  | 0.8 | V |
| --- | --- | --- | --- | --- | --- | --- |
| High level input voltage | VIH | - | CMOS buffers VDD=3.6V | 2 |  | V |
| Threshold Schmitt trigger input voltage | VT+ | - | Note 5 | 1.40 | 2.08 | V |
| Threshold Schmitt trigger input voltage | VT- | - | Note 5 | 0.99 | 1.51 | V |
| Hysteresis | Delta VT | - | Note 5 | 0.37 |  | V |

§2.3.1.4 (VDD=2.5, 3 or 3.3V VCC=5V)

Add:

| Low level input voltage | VIL | - | PICV, PICV5 buffers VDD min |  | 0.8 | V |
| --- | --- | --- | --- | --- | --- | --- |
| High level input voltage | VIH | - | PICV, PICV5 buffers VDD max | 2 |  | V |
| Threshold Schmitt trigger input voltage | VT+ | - | Note 5 | 1.40 | 2.08 | V |
| Threshold Schmitt trigger input voltage | VT- | - | Note 5 | 0.99 | 1.51 | V |
| Hysteresis | Delta VT | - | Note 5 | 0.37 |  | V |

- Change #7 :

§2.3.2. Notes to electrical measurements

Reason of change :

Notes 6 and 7 are no more applicable in final version

Description of change:

Remove notes 6 and 7.