Page 1 of 16

SWITCHES, THERMOSTATIC, BIMETALLIC, SPST WITH SNAP ACTION, WITH OPENING OR CLOSING CONTACT

BASED ON TYPE TH47

ESCC Detail Specification No. 3702/001

| Issue 6 Draft G | November 2017 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2017. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBD | Specification upissued to incorporate changes per DCR. |
| Draft 6A S.Thacker (TW) 17/03/2017 | Draft 6A created per converted format to go with converted Generic 3702 Draft 5A. |
| Draft 6B S.Thacker (TW) 24/05/2017 | Draft 6B created per Comepa review of draft 6A per email dated 22/05/17. Changes from draft 6A are highlighted yellow plus some comments have been added in the review margin for Comepa’s information & consideration Changes resulting from changes to 3702 per telecof dated 01/06/17 have been added to this revision (in blue) |
| Draft 6C S.Thacker (TW) 05/07/2017 | Draft 6C produce per Comepa review of draft 6B+ per email dated 30/06/17. Changes from draft 6B+ are highlighted yellow plus some comments have been added in the review margin for Comepa’s information & consideration |
| Draft 6D S.Thacker (TW) 26/07/2017 | Draft 6D produce per Comepa review of draft 6C per email dated 21/07/17. Changes from draft 6C are highlighted yellow plus some comments have been added in the review margin for Comepa’s information & consideration |
| Draft 6E S.Thacker (TW) 11/08/2017 | Draft 6E produce per Comepa review of draft 6D per email dated 03/08/17. Changes from draft 6D are highlighted yellow plus some comments have been added in the review margin for Comepa’s information & consideration |
| Draft 6F S.Thacker (TW) 12/10/2017 | Draft 6F produce per Comepa review of draft 6E plus 3702 draft 5C per email dated 10/10/17. All technical changes from issue 6E are highlighted yellow. |
| Draft 6G S.Thacker (TW) 17/11/2017 | Draft 6G produce per ESA/Comepa/WG review of draft 6F plus 3702 draft 5E per telecon dated 17/11/17. All technical changes from issue 5 are highlighted yellow. |

TABLE OF CONTENTS

1	GENERAL	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants and Range of Components	6

1.5	MAXIMUM RATINGS	8

1.6	PHYSICAL DIMENSIONS	9

1.6.1	Case without Fixings (Variant 01)	9

1.6.2	Case with 2 Hole Flange Mount (Variant 02)	10

1.6.3	Case with 5mm M4 Mounting Stud (Variant 03)	11

1.7	FUNCTIONAL DIAGRAM	11

1.8	MATERIALS AND FINISHES	12

1.8.1	Case	12

1.8.2	Terminal Material and Finish	12

2	REQUIREMENTS	12

2.1	GENERAL	12

2.1.1	Deviations from the Generic Specification	12

2.2	MARKING	12

2.3	ROBUSTNESS OF TERMINATIONS	12

2.4	Thermal Measurements	13

2.5	Electrical Measurements	13

2.6	PARAMETER DRIFT VALUES	14

2.7	INTERMEDIATE AND END-POINT THERMAL AND ELECTRICAL MEASUREMENTS	14

APPENDIX A	16

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the Component Type Variants and Range of Components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3702.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply. In addition the definitions specified in the Generic Specification shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 370200101B026020HY

Detail Specification Reference: 3702001

Component Type Variant Number: 01 (as required)

Characteristic code: contact type (opening): B (as required)

Characteristic code: nominal functioning temperature, TNF (+26°C): 026 (as required)

Characteristic code: nominal restoring temperature, TNR (+20°C): 020 (as required)

Characteristic code: switching temperature tolerance, Tol (±3°C): H (as required)

Characteristic code: grade: Y (as required)

#### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number shall be as follows:

Contact Type expressed by means of the following codes:

| Contact Type | Code |
| --- | --- |
| Switch with Opening Contact: A component where the switch contact will open when the temperature rises to a value equal to or greater than the upper of the two switching temperatures | B |
| - Switch with Closing Contact: - A component where the switch contact will close when the temperature rises to a value equal to or greater than the upper of the two switching temperatures | C |

Nominal functioning temperature, TNF, and nominal restoring temperature, TNR, each expressed by means of the following codes (see Para. 1.4.2 Note 2). The unit quantity shall be °C.

| Nominal Functioning and Restoring Temperature TNF, TNR (°C) | Code |
| --- | --- |
| +XXX | XXX |
| +XX | 0XX |
| +X | 00X |
| 0 | 000 |
| -X | 60X |
| -XX | 6XX |

The switching temperature tolerance, Tol, relative to TNF and TNR, expressed by the following codes (see Para. 1.4.2 Note 2):

| Temperature Tolerance Tol (± °C) | Code |
| --- | --- |
| 2 | B |
| 3 | H |
| 5 | J |

Component grade expressed by the following codes (see Para. 1.4.2 Note 2):

| Grade Description | Code |
| --- | --- |
| A switch designed to operate within the specified switching temperatures tolerance that also maintains a minimum actual temperature differential | Y |
| A switch designed to operate within the specified switching temperatures tolerance that also maintains both a minimum and a maximum actual temperature differential | Z |

### Component Type Variants and Range of Components

The Component Type Variants and Range of Components applicable to this specification are as follows:

| Variant Number | Mounting Details (1) | Thermal Switching Characteristics (TNF, TNR, Tol, Nom Diff, Act Diff) | Weight Max (g) |
| --- | --- | --- | --- |
| 01 | Case without Fixings | See Note 2 | 7.5 |
| 02 | Case with 2 Hole Flange Mount | See Note 2 | 7.5 |
| 03 | Case with 5mm M4 Mounting Stud | See Note 2 | 8 |

NOTES:

See Para. 1.6.

The following thermal switching characteristic details apply to the available range of components:

Grade Y:

| Nominal Functioning Temperature TNF (°C) | Nominal Restoring Temperature TNR (°C) | Temperature Tolerance Tol (°C) | Nominal Temperature Differential Nom Diff TNF-TNR (°C) | Actual Temperature Differential Act Diff TF-TR (°C) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |
| -41 to -31 | -45 to -40 | 5 | 9 to 14 | 4 | Nom Diff + 2Tol |
| -30 to +99 | -44 to +94 | 3 | 5 to 14 | 4 | Nom Diff + 2Tol |
|  | -40 to +94 | 2 | 5 to 10 | 4 | Nom Diff + 2Tol |
| +100 to +150 | +85 to +140 | 5 | 10 to 15 | 4 | Nom Diff + 2Tol |

Grade Z:

| Nominal Functioning Temperature TNF (°C) | Nominal Restoring Temperature TNR (°C) | Temperature Tolerance Tol (°C) | Nominal Temperature Differential Nom Diff TNF-TNR (°C) | Actual Temperature Differential Act Diff TF-TR (°C) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |
| -35 to -31 | -45 to -41 | 5 | 10 | 4 | 18 |
|  |  | 3 |  | 4 | 14 |
|  | -44 to -40 | 3 | 9 | 4 | 13 |
|  | -43 to -39 | 3 | 8 | 4 | 12 |
|  | -42 to -38 | 3 | 7 | 4 | 11 |
| -30 to +99 | -40 to +89 | 3 | 10 | 4 | 13 |
|  |  | 2 |  | 4 | 12 |
|  | -39 to +90 | 3 | 9 | 4 | 12 |
|  |  | 2 |  | 4 | 11 |
|  | -38 to +91 | 3 | 8 | 4 | 11 |
|  |  | 2 |  | 4 | 10 |
|  | -37 to +92 | 3 | 7 | 4 | 10 |
|  |  | 2 |  | 4 | 9 |
|  | -36 to +93 | 3 | 6 | 4 | 9 |
|  |  | 2 |  | 4 | 8 |
|  | -35 to +94 | 3 | 5 | 4 | 8 |
|  |  | 2 |  | 4 | 7.5 |
| +100 to +150 | +90 to +140 | 5 | 10 | 4 | 17 |
|  |  | 3 |  | 4 | 13 |
|  | +91 to +141 | 3 | 9 | 4 | 12 |
|  | +92 to +142 | 3 | 8 | 4 | 11 |
|  | +93 to +143 | 3 | 7 | 4 | 10 |

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated Current | IR | 4 | A | V ≤ 30V resistive 100000 switching cycles Note 1 |
| Overload Current | Ioverload | 6 | A | V ≤ 30V resistive |
| Rated Voltage | VR | 120 | V | I ≤ 0.3A resistive 100000 switching cycles Note 1 |
| Operating Temperature Range | Top | -50 to +155 | °C |  |
| Storage Temperature Range | Tstg | -75 to +175 | °C |  |
| Soldering Temperature | Tsol | +260 | °C | Note 2 |

Notes:

            - Derate as follows:

            - Duration 10 seconds maximum, greater than 1.5mm from the body; the same terminal shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS

### Case without Fixings (Variant 01)

| Symbol | Dimension (mm) |  |
| --- | --- | --- |
|  | Min | Max |
| ØA | 15.3 | 15.5 |
| B | 7.3 | 7.5 |
| C | - | 11 |
| D | 2.5 | - |
| E | 6.25 | 6.45 |
| F | 3.8 | 4 |
| G | - | 10 |
| ØH | 1.9 | 2.1 |

### Case with 2 Hole Flange Mount (Variant 02)

| Symbol | Dimension (mm) (Note 1) |  |
| --- | --- | --- |
|  | Min | Max |
| ØA | 15.3 | 15.5 |
| B | 7.8 | 8 |
| C | - | 11.5 |
| D | 2.5 | - |
| I | 16 | 16.2 |
| J | 30 | 30.2 |
| K | 23.7 | 23.9 |
| ØL | 3.5 | 3.7 |

NOTES:

Dimensions E, F, G, ØH from Para. 1.6.1 also apply.

### Case with 5mm M4 Mounting Stud (Variant 03)

| Symbol | Dimension (mm) |  |
| --- | --- | --- |
|  | Min | Max |
| ØA | 15.3 | 15.5 |
| B | 8.1 | 8.3 |
| C | - | 11.8 |
| D | 2.5 | - |
| M | M4 |  |
| N | 4.9 | 5.1 |

NOTES:

Dimensions E, F, G, ØH from Para. 1.6.1 also apply.

## FUNCTIONAL DIAGRAM

NOTES:

Contact opens or closes with increasing temperature (see Para. 1.4.1.1(a)).

The terminals are isolated from the case.

## MATERIALS AND FINISHES

### Case

The case shall Nickel-iron alloy, with electroless nickel plating, hermetically sealed, welded construction. Neither electro-deposited tin nor any paint shall be used.

### Terminal Material and Finish

Terminal material shall be Type D with Type 3 finish in accordance with the requirements of ESCC Basic Specification No. 23500.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests - Chart F3

Seal, Fine Leak: The maximum helium measured leak rate shall be: R = 1 x10-8 atm.cm3/s maximum.

#### Deviations from Qualification and Periodic Testing - Chart F4

Seal, Fine Leak: The maximum helium measured leak rate shall be: R = 1 x10-8 atm.cm3/s maximum.

Solderability: For test method 2, soldering iron size B shall apply.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700. The information to be marked shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ROBUSTNESS OF TERMINATIONS

The test conditions for Robustness of Terminations, tested as specified in the Generic Specification, shall be as follows:

Test Ua1 (tensile): applied force: 20N, in the axis of the terminals.

## Thermal Measurements

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Functioning Temperature | TF | ESCC No. 3702 Note 1 | TNF – Tol | TNF + Tol | °C |
| Restoring Temperature | TR | ESCC No. 3702 Note 1 | TNR – Tol | TNR + Tol | °C |
| Actual Temperature Differential | Act Diff | = TF – TR | Note 2 | Note 2 | °C |

NOTES:

The following temperature gradient shall apply for measurements:

0.33°C/ minute minimum; 1°C/ minute maximum.

See Para. 1.4.2 Note 2 for Act Diff minimum and maximum limits.

## Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Voltage Proof | VP | ESCC No. 3702 T ≥ T2 or T ≤ T3 (Note 1) Between contacts & case: Between open contacts: | 1000 100 | - - | Vrms |
| Insulation Resistance | RI | ESCC No. 3702 T ≥ T2 or T ≤ T3 (Note 1) V = 100Vdc Between contacts & case: Between open contacts: | 500 500 | - - | MΩ |
| Contact Resistance | RC | ESCC No. 3702 T ≥ T2 or T ≤ T3 (Note 1) | - | 50 | mΩ |

NOTES:

See the Generic Specification for definition of T2 and T3.

## PARAMETER DRIFT VALUES

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.4 Thermal Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Functioning Temperature | TF | ±1 | TNF – Tol | TNF + Tol | °C |
| Restoring Temperature | TR | ±1 | TNR – Tol | TNR + Tol | °C |

## INTERMEDIATE AND END-POINT THERMAL AND ELECTRICAL MEASUREMENTS

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4 Thermal Measurements and Para. 2.5 Electrical Measurements.

| Test Reference per ESCC No. 3702 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Rapid Change of Temperature |  |  |  |  |  |
| Initial Measurements (Note 1) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |
| Vibration |  |  |  |  |  |
| During Testing | No intermittent contact ≥ 1ms, or improper open or short circuit | - | - |  | - |
| Mechanical Shock |  |  |  |  |  |
| During Testing | No intermittent contact ≥ 1ms, or improper open or short circuit | - | - |  | - |
| Thermal and Electrical Measurements | Voltage Proof | VP | See Para. 2.5 |  | Vrms |
|  | Insulation Resistance | RI | See Para. 2.5 |  | MΩ |
|  | Contact Resistance | RC | See Para. 2.5 |  | mΩ |
|  | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Change of Functioning Temperature | ΔTF | -1.7 |  | +1.7 |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Change of Restoring Temperature | ΔTR | -1.7 |  | +1.7 |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |
|  | Change of Actual Temperature Differential | ΔAct Diff | -1.7 |  | +1.7 |
| Resistance to Soldering Heat |  |  |  |  |  |
| Initial Measurements (Note 1) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Restoring Temperature | Tr | See Para. 2.4 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |
| Response Time |  |  |  |  |  |
| Initial Measurements (Note 1) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
| During Testing | Response Time | trp | - | 20 | s |
| Operating Life (4A, 30Vdc, resistive) |  |  |  |  |  |
| Initial Measurements (Note 1) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |
| During Testing | Monitor for correct switching and voltage during 50% of closure period | - | - |  | - |
| Intermediate Measurements (25000 and 50000 cycles) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Change of Functioning Temperature | ΔTF | -1.7 | +1.7 | °C |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Change of Restoring Temperature | ΔTR | -1.7 | +1.7 | °C |
| Final Measurements | Voltage Proof | VP | See Para. 2.5 |  | Vrms |
|  | Insulation Resistance | RI | See Para. 2.5 |  | MΩ |
|  | Contact Resistance | RC | - | 100 | mΩ |
|  | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Change of Functioning Temperature | ΔTF | -1.7 | +1.7 | °C |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Change of Restoring Temperature | ΔTR | -1.7 | +1.7 | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |
|  | Change of Actual Temperature Differential | ΔAct Diff | -1.7 | +1.7 | °C |
| Overload (6A, 30Vdc, resistive) |  |  |  |  |  |
| Initial Measurements (Note 1) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |
| High and Low Temperature Storage |  |  |  |  |  |
| Initial Measurements (Note 1) | Functioning Temperature | TF | See Para. 2.4 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.4 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.4 |  | °C |

NOTES:

Final measurements from Thermal and/or Electrical Measurements in Chart F3, Screening Tests, of the Generic Specification may be used as initial measurements.

## APPENDIX A

AGREED DEVIATIONS FOR COMEPA (F)

| Items Affected | Description of Deviations |
| --- | --- |
| Para. 2.1.1.1, Deviations from the Generic Specification: Screening Tests - Chart F3 | Creepage: may be performed using creepage detection equipment in accordance with Comepa specification POT4711 with Creepage limit: 5ms maximum. |
| Para. 2.1.1.2, Deviations from the Generic Specification: Deviations from Qualification and Periodic Testing - Chart F4 | Para. 8.17, Permanence of Marking This test is not applicable as all marking uses a permanent engraving process. |
| Para. 2.1.1, Deviations from the Generic Specification: Data Documentation | Para. 9.3, List of Equipment Used The list of equipment used shall not be delivered to the Orderer but shall available for review at Comepa’s facilites. |