Page 1 of 14

DIODE, SILICON, SWITCHING

BASED ON TYPE BAY6642

ESCC Detail Specification No. 5101/029

| Issue 1 | November 2013 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2013, All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without prior permission of the European Space Agency and provided it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |

TABLE OF CONTENTS

1	General	5

1.1	Scope	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	5

1.6	HANDLING PRECAUTIONS	6

1.7	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	7

1.7.1	Leadless Chip Carrier Package (HSL2-1808) - 2 Terminal	7

1.8	FUNCTIONAL DIAGRAM	8

1.9	MATERIALS AND FINISHES	8

2	REQUIREMENTS	8

2.1	GENERAL	8

2.1.1	Deviations from the Generic Specification	8

2.1.1.1	Deviations from Screening Tests (Chart F3)	8

2.1.1.2	Deviations from Qualification and Periodic Tests (Chart F4)	9

2.2	MARKING	9

2.3	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	9

2.3.1	Room Temperature Electrical Measurements	9

2.3.2	High and Low Temperatures Electrical Measurements	10

2.3.3	Notes to Electrical Measurement Tables	11

2.4	PARAMETER DRIFT VALUES	11

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	12

2.6	HIGH TEMPERATURE REVERSE BIAS BUN-IN CONDITIONS	12

2.7	POWER BURN-IN CONDITIONS	13

2.8	OPERATING LIFE CONDITIONS	13

# General

## Scope

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 5000

MIL-STD-750, Test Methods and Procedures for Semiconductor Devices

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 510102901:

Detail Specification Reference: 5101029

Component Type Variant Number: 01

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case | Breakdown Voltage V(BR) (V) | Working Peak Reverse Voltage VRWM (V) | Terminal Material and Finish | Weight max g |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | BAY6642 | HSL2-1808 | 100 | 75 | D2 | 0.06 |

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Forward Surge Current | IFSM | 2.5 | A (pk) | Notes 1, 2 |
| Working Peak Reverse Voltage | VRWM | Note 3 | V |  |
| Average Output Rectified Current | IO | 300 | mA | Note 4 |
| Operating Temperature Range (Case Temperature) | Top | -65 to +175 | oC |  |
| Junction Temperature | Tj | +175 | oC |  |
| Storage Temperature Range | Tstg | -65 to +175 | oC |  |
| Soldering Temperature | Tsol | +245 | oC | Note 5 |
| Thermal Resistance, Junction to Soldering Point | Rth(j-s) | 100 | oC/W |  |

NOTES:

Sinusoidal pulse of 10ms duration.

At Tamb≤25oC.

See Component Type Variants for VRWM value.

At Ts>+110oC, derate linearly to 0A at +175oC. Ts is measured on the PCB at the soldering point to the cathode terminal.

Duration 10s maximum and the same package shall not be resoldered until 3 minutes have elapsed.

## HANDLING PRECAUTIONS

These devices are susceptible to damage by electrostatic discharge. Therefore, suitable precautions shall be employed for protection during all phases of manufacture, testing, packaging, shipment and any handling.

These components are categorised as Class 3 per ESCC Basic Specification No. 23800 with a Minimum Critical Path Failure Voltage of 10kV

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

### Leadless Chip Carrier Package (HSL2-1808) - 2 Terminal

| Symbols | Dimensions mm |  |
| --- | --- | --- |
|  | Min | Max |
| A | 4.4 | 4.8 |
| B | 1.8 | 2.2 |
| C | 1.1 | 1.5 |
| D | 0.85 | 1.15 |
| E | 2.8 | 3.2 |
| F | 0.5 | 0.9 |
| G | 1.6 | 2 |
| H | 3.6 | 3.9 |
| I | 1.4 | 1.7 |

NOTES:

Terminal identification: The cathode is identified by a chamfer on the bottom metallisation and an index mark on the lid seal ring.

## FUNCTIONAL DIAGRAM

NOTES:

For the HSL2-1808 package, the lid is connected to the cathode terminal.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows:

Case

- The case shall be herrmetically sealed and have a ceramic body with a Kovar lid.

Terminal Finish

- As specified in Component Type Variants.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests (Chart F3)

Particle Impact Noise Detection may be performed at any point after Temperature Cycling, prior to Seal.

Following Temperature Cycling, Parameter Drift Values (initial measurements) shall be performed as specified herein.

Immediately following Parameter Drift Values measurements, a Surge Current test shall be performed on a sample of 32 components with 0 failures allowed. The Surge Current test shall be as follows:

MIL-STD-750, Test Method 4066, condition A

IFS = 2.5A(pk) for duration 10ms or 1A(pk) for duration 250ms

IO = 300mA

10 surges at a rate of 10 per minute maximum.

After the Surge Current test, Parameter Drift Values measurements shall be performed as specified herein referred to the measurements prior to the Surge Current test.

Parameter Drift Values measurements need not be repeated prior to HTRB. The electrical measurements results from after the Surge Current test may be used as initial measurements prior to HTRB.

#### Deviations from Qualification and Periodic Tests (Chart F4)

Mechanical Shock: Not applicable

Vibration: Not applicable

Constant Acceleration: Not applicable

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

Terminal Identification.

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures. Consolidated notes are given after the tables.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb=+25 ±3oC.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Forward Voltage | VF1 | 4011 | IF=1mA, Note 1 | 0.54 | 0.62 | V |
|  | VF2 | 4011 | IF=10mA, Note 1 | 0.64 | 0.8 | V |
|  | VF3 | 4011 | IF=100mA, Note 1 | 0.78 | 0.92 | V |
|  | VF4 | 4011 | IF=500mA, Note 1 | 0.87 | 1.2 | V |
| Reverse Current 1 | IR1 | 4016 | DC Method VR= 20V | - | 25 | nA |
| Reverse Current 2 | IR2 | 4016 | DC Method VR=VRWM, Note 2 | - | 100 | nA |
| Breakdown Voltage | V(BR) | 4022 | DC Method IR=10A | Note 2 | - | V |
| Capacitance | C | 4001 | VR = 0V Vsig=50mV (p-p)max f = 1MHz Note 3 | - | 2.5 | pF |
| Reverse Recovery Time | trr | 4031 | Test Condition A IF= IR= 10mA Irr=2mA Note 3, 4 | - | 4 | ns |
| Forward Recovery Time | tfr | 4026 | IF=200mA Vfr=1.1VF Note 3, 4 | - | 10 | ns |
| Forward Recovery Voltage | Vfr | 4026 | IF=200mA Vfr=1.1VF Note 3, 4 | - | 5 | V |
| Thermal Impedance Junction to Case | Zth(j-c) | 3101 | IH=1A, tH=10ms tmd=70µs max | - | 35 | oC/W |

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions Note 5 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Forward Voltage | VF2 | 4011 | Tamb=+150(+0 -5)oC IF=10mA, Note 1 | - | 0.65 | V |
|  | VF3 | 4011 | Tamb=-55(+5 -0)oC IF=100mA, Note 1 | - | 1.2 | V |
|  | VF4 | 4011 | Tamb=-55(+5 -0)oC IF=500mA, Note 1 | - | 1.3 | V |
| Reverse Current 1 | IR1 | 4016 | Tamb=+150(+0 -5)oC DC Method VR=20V | - | 50 | µA |
| Reverse Current 2 | IR2 | 4016 | Tamb=+150(+0 -5)oC DC Method VR=VRWM, Note 2 | - | 100 | µA |

### Notes to Electrical Measurement Tables

Pulsed measurement, pulse width 20ms maximum.

See Component Type Variants for VRWM and V(BR) values.

Read and record measurements shall be performed on a sample of 32 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

The sample may be soldered to a suitable test substrate to facilitate the test but then shall not form part of the delivery lot.

Read and record measurements shall be performed on a sample of 5 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb=+25 ±3oC.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value | Absolute |  |  |
|  |  |  | Min | Max |  |
| Forward Voltage 2 | VF2 | ±0.03 | 0.64 | 0.8 | V |
| Reverse Current 1 | IR1 | ±10 or (1) ±100% | - | 25 | nA |
| Reverse Current 2 | IR2 | ±25 or (1) ±100% | - | 100 | nA |
| Breakdown Voltage | V(BR) | ±15 or (1) ±15% | Note 2 | - | V |

Notes

Whichever is the greater referred to the initial value.

See Component Type Variants for V (BR) value.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb=+25 ±3oC.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value | Absolute |  |  |
|  |  |  | Min | Max |  |
| Forward Voltage 2 | VF2 | ±0.03 | 0.64 | 0.8 | V |
| Reverse Current 1 | IR1 | ±10 or (1) ±100% | - | 25 | nA |
| Reverse Current 2 | IR2 | ±25 or (1) ±100% | - | 100 | nA |
| Breakdown Voltage | V(BR) | ±15 or (1) ±15% | Note 2 | - | V |

NOTES:

Whichever is the greater referred to the initial value.

See Component Type Variants for V(BR) value.

## HIGH TEMPERATURE REVERSE BIAS BUN-IN CONDITIONS

| Characteristics | Symbols | Limits | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | oC |
| Reverse Voltage | VR | 60 (+0 -3) | V |
| Duration | t | 72 ±12 | hours |

NOtes

Voltage may be switched off during cool down.

## POWER BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | < 150 (Note 1) | oC |
| Junction Temperature | TJ | +175 (+0 -5) | oC |
| Forward Current | IF | 300 (+0 -30) | mA |

NOTES:

Tamb shall be adjusted to attain the specified junction temperature.

## OPERATING LIFE CONDITIONS

APPENDIX ‘A’

# AGREED DEVIATIONS FOR INFINEON TECHNOLOGIES (D)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Deviations from Production Control (Chart F2) | The 3 component sample Dimension Check need only be performed once on each component package production lot. |
| Deviations from Screening Tests (Chart F3) | Temperature Cycling shall be performed in accordance with MIL-STD-883, Test Method 1010, Test Condition F, 20 cycles at maximum storage temperature rating specified in the Detail Specification. |
|  | High and Low Temperatures Electrical Measurements may be performed at any point after Power Burn-in, prior to Seal, but shall still count towards Check for Lot Failure. |
|  | Radiographic Inspection is not applicable. |
|  | Seal Fine Leak. The maximum leak rate shall be 2 x 10-8 atm.cm3/s. |
|  | Solderability is not applicable unless otherwise stipulated in the Purchase Order. |
| Deviations from Qualification and Periodic Tests (Chart F4) | Seal Fine Leak. The maximum leak rate shall be 2 x 10-8 atm.cm3/s. |
|  | Temperature Cycling shall be performed in accordance with MIL-STD-883, Test Method 1010, Test Condition F, 100 cycles at maximum storage temperature rating specified in the Detail Specification. |
|  | Permanence of Marking is not applicable. |
| Room Temperature Electrical Measurements | The read and record 32 component sample electrical measurements for characteristics trr, tfr and Vfr need only be performed once on each wafer used to supply components to this specification. Any failure shall result in rejection of the wafer. The sample measurement may be performed at any time during production. |
| High and Low Temperature Electrical Measurements | The read and record 5 component sample low temperature electrical measurements for characteristics VF3 and VF4 need only be performed once on each wafer used to supply components to this specification. Any failure shall result in rejection of the wafer. The sample measurement may be performed at any time during production. |