Page 1 of 14

TRANSISTORS, LOW POWER, PNP

BASED ON TYPE SN3962

ESCC Detail Specification No. 5202/015

| Issue 1 | November 2013 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2013, All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without prior permission of the European Space Agency and provided it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	6

1.6	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	7

1.6.1	Metal Can Package (TO-18) – lead	7

1.6.2	Chip Carrier Package (CCP) - 3 terminal	8

1.7	FUNCTIONAL DIAGRAM	9

1.8	MATERIALS AND FINISHES	9

2	REQUIREMENTS	9

2.1	GENERAL	9

2.1.1	Deviations from the Generic Specification	10

2.2	MARKING	10

2.3	TERMINAL STRENGTH	10

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	10

2.4.1	Room Temperature Electrical Measurements	10

2.4.2	High and Low Temperatures Electrical Measurements	12

2.5	PARAMETER DRIFT VALUES	12

2.6	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	13

2.7	HIGH TEMPERATURE REVERSE BIAS BUN-IN CONDITIONS	14

2.8	POWER BURN-IN CONDITIONS	14

2.9	OPERATING LIFE CONDITIONS	14

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 5000

MIL-STD-750, Test Methods and Procedures for Semiconductor Devices

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 520201501

Detail Specification Reference: 5202015

Component Type Variant Number: 01 (as required)

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case | Lead/Terminal Material and/or Finish | Weight max g |
| --- | --- | --- | --- | --- |
| 01 | 2N3962 | TO-18 | D2 | 0.4 |
| 02 | 2N3962 | TO-18 | D3 or D4 | 0.4 |
| 03 | 2N3962 | TO-18 | D7 | 0.4 |
| 04 | 2N3962 | CCP | 2 | 0.06 |
| 05 | 2N3962 | CCP | 4 | 0.06 |

The lead/terminal material and/or finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Collector-Base Voltage | VCBO | -60 | V | Over entire operating temperature range |
| Collector-Emitter Voltage | VCEO | -60 | V |  |
| Emitter-Base Voltage | VEBO | -6 | V |  |
| Collector Current | IC | -200 | mA | Continuous |
| Power Dissipation For TO-18 and CCP | Ptot1 | 0.36 | W | At Tamb ≤ +25oC Note 1 |
| For CCP | Ptot2 | 0.58 (Note 2) | W |  |
| For TO-18 | Ptot3 | 1.2 | W | At Tcase ≤ +25oC Note 1 |
| Operating Temperature Range | Top | -65 to +200 | oC | Note 3 |
| Storage Temperature Range | Tstg | -65 to +200 | oC | Note 3 |
| Soldering Temperature For TO-18 For CCP | Tsol | +260 +245 | oC | Note 4 Note 5 |

NOTES:

For Tamb or Tcase > +25oC, derate linearly to 0W at +200oC.

When mounted on an 8 x 10 x 0.6mm ceramic substrate.

For Variants with tin-lead plating or hot solder dip lead finish all testing, and any handling, performed at Tamb > +125oC shall be carried out in a 100% inert atmosphere.

Duration 10 seconds maximum at a distance of not less than 1.5mm from the device body and the same lead shall not be resoldered until 3 minutes have elapsed.

Duration 5 seconds maximum and the same terminal shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

### Metal Can Package (TO-18) – lead

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 4.32 | 5.33 |  |
| Øb | 0.406 | 0.533 | 2, 3 |
| Øb2 | 0.406 | 0.483 | 2, 3 |
| ØD | 5.31 | 5.84 |  |
| ØD1 | 4.52 | 4.95 |  |
| e | 2.54 BSC |  | 4 |
| e1 | 1.27 BSC |  | 4 |
| F | - | 0.762 |  |
| j | 0.914 | 1.17 |  |
| k | 0.711 | 1.22 | 5 |
| L | 12.7 | - | 2 |
| L1 | - | 1.27 | 3 |
| L2 | 6.35 | - | 3 |
| a | 45o BSC |  | 1, 4, 6 |

NOTES:

Terminal identification is specified by reference to the tab position where lead 1 = emitter, lead 2 = base, lead 3 = collector.

Applies to all leads.

Øb2 applies between L1 and L2. Øb applies between L2 and 12.7mm from the seating plane. Diameter is uncontrolled within L1 and beyond 12.7mm from the seating plane.

Leads having maximum diameter 0.483mm measured in the gauging plane 1.37(+0.025,-0)mm below the seating plane of the device shall be within 0.178mm of their true position relative to a maximum-width-tab.

Measured from the maximum diameter of the actual device.

Tab centreline.

### Chip Carrier Package (CCP) - 3 terminal

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 1.15 | 1.5 |  |
| C | 0.45 | 0.56 | 2 |
| D | 0.6 | 0.91 | 2 |
| E | 0.91 | 1.12 |  |
| F | 1.9 | 2.15 |  |
| G | 2.9 | 3.25 |  |
| I | 2.4 | 2.85 |  |
| L | 0.4 | 0.6 | 2 |
| M | 2.4 | 2.65 |  |
| N1 | 0.855 | 1.055 |  |
| N2 | 1.8 | 2 |  |
| r | 0.3 TYPICAL |  | 1 |

NOTES:

Terminal identification is specified by reference to the corner notch position where terminal 1 = emitter, terminal 2 = base, terminal 3 = collector.

Applies to all terminals

## FUNCTIONAL DIAGRAM

NOTES:

For TO-18, the collector is internally connected to the case.

For CCP, the lid is not connected to any terminal.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows:

Case

- For the metal can package the case shall be hermetically sealed and have a metal body with hard glass seals.

For the chip carrier package the case shall be hermetically sealed and have a ceramic body with a Kovar lid.

Leads/Terminals

- As specified in Component Type Variants.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

None.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## TERMINAL STRENGTH

The test conditions for terminal strength, tested as specified in the ESCC Generic Specification, shall be as follows:

For TO-18, Test Condition: E, lead fatigue.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb=+25 ±3oC.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Collector-Emitter Breakdown Voltage | V(BR)CEO | 3011 | IC = -10mA Note 1 Bias condition D | -60 | - | V |
| Collector-Base Breakdown Voltage | V(BR)CBO | 3001 | IC = -10µA Bias condition D | -60 | - | V |
| Emitter-Base Breakdown Voltage | V(BR)EBO | 3026 | IE = -10µA Bias condition D | -6 | - | V |
| Emitter-Base Cut-off Current | IEBO | 3061 | VEB = -4V Bias condition D | - | -10 | nA |
| Collector-Emitter Cut-off Current | ICES | 3041 | VCE = -50V Bias condition D | - | -10 | nA |
| Forward-Current Transfer Ratio | hFE1 | 3076 | VCE=-5V ; IC =-1µA | 60 | - | - |
|  | hFE2 | 3076 | VCE=-5V ; IC =-10µA | 100 | 300 | - |
|  | hFE3 | 3076 | VCE=-5V ; IC =-1mA Note 1 | 100 | 450 | - |
|  | hFE4 | 3076 | VCE=-5V ; IC =-50mA Note 1 | 90 | - | - |
| Base-Emitter Saturation Voltage | VBE(sat)1 | 3066 | IC=-10mA IB=-500nA Test condition A Note 1 | - | -900 | mV |
|  | VBE(sat)2 | 3066 | IC=-50mA IB=-5mA Test condition A Note 1 | - | -950 | mV |
| Small-Signal Current Gain | hfe1 | 3206 | VCE=-5V, IC=-1mA f=1kHz Note 2 | 100 | 550 | - |
|  | hfe2 | 3206 | VCE=-5V, IC=-500nA f=20MHz Note 2 | 2 | 8 | - |
| Output Capacitance | Cobo | 3236 | VCB=-5V, IE=0A f=1MHz Note 2 | - | 6 | pF |
| Input Capacitance | Cibo | 3240 | VEB=-500mV IC=0A f=1MHz Note 2 | - | 15 | pF |
| Noise Figure | NF | 3246 | VCE=-5V IC=-20µA RG=10kΩ f=10Hz to 10kHz Note 2 | - | 3 | dB |

NOTES:

Pulse measurement: Pulse Width ≤ 300µs, Duty Cycle ≤ 2%.

For AC characteristics read and record measurements shall be performed on a sample of 32 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions Note 1 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Collector-Emitter Cut-off Current | ICES | 3041 | Tamb=+150 (+0 -5)oC VCE=-50V Bias condition D | - | -10 | µA |
| Forward-Current Transfer Ratio 2 | hFE2 | 3076 | Tamb=-55 (+5 -0)oC VCE=-5V IC=-10µA | 40 | - | - |

NOTES:

Read and record measurements shall be performed on a sample of 5 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb=+25 ±3oC.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value ± | Absolute |  |  |
|  |  |  | Min | Max |  |
| Collector-Emitter Cut-off Current | ICES | ±2 or (1) ±100% | - | -10 | nA |
| Forward-Current Transfer Ratio 2 | hFE2 | ±15% | 100 | 300 | - |
| Collector-Emitter Saturation Voltage 2 | VCE(sat)2 | ±30 or (1) ±15% | - | -400 | mV |

NOTES:

Whichever is the greater referred to the initial value.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb=+25 ±3oC.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- |
|  |  | Min | Max |  |
| Collector-Emitter Cut-off Current | ICES | - | -10 | nA |
| Forward-Current Transfer Ratio 2 | hFE2 | 100 | 300 | - |
| Collector-Emitter Saturation Voltage 2 | VCE(sat)2 | - | -400 | mV |

## HIGH TEMPERATURE REVERSE BIAS BUN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | oC |
| Collector-Base Voltage | VCB | 48 | V |
| Duration | t | 48 minimum | Hours |

## POWER BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +20 to +50 | oC |
| Power Dissipation | Ptot | As per Maximum Ratings Ptot1 derated at the chosen Tamb | W |
| Collector-Base Voltage | VCB | -40 | V |

## OPERATING LIFE CONDITIONS

The conditions shall be as specified for Power Burn-in.