# Low ESR tantalum capacitor Evaluation and Qualification

Company : AVX/CZ

| Start TRL 3 |  |
| --- | --- |
| Achieved TRL 6 |  |
| Start date | November 2010 End date September 2017 |
| Introduction There were limited specification options on tantalum capacitors for aerospace manufacturers based on European standardization. The limited choice of ESCC EQPL part numbers (ESCC 3012) running in relatively low volumes that increase manufacturing price and reduce interest from users towards the near future. There is also currently a limited range of CV for tantalum capacitors based on the 3012/001 TAJ-ESCC qualified range. European aerospace manufactures are today dependent on MIL (CWR or COTS+) based standardization offering the requested CV values and low ESR tantalum capacitors. The latest low voltage, high power computing technologies require lower ESR tantalum capacitors that provide higher ripple current capability at smaller case sizes. Thus these low ESR capacitors may reduce component count and the total weight. Low ESR tantalum capacitor matrix will be created a based on the existing TAJ (ESCC-3012/001) capacitors, professional TRJ low ESR capacitors and TRM tantalum multianode very low ESR capacitors. The development have been lead by AVX Czech Republic located in Lanskroun, Czech Republic as already manufacturing plant of ESCC qualified tantalum solid capacitors, type TAJ-ESA ESCC 3012/001 for long time.. |
| Objectives The main technical objective is to develop a low ESR, High CV tantalum capacitor manufactured in European location for space application, mainly European space manufacturers.. |
| Achievement Design and Process Trade off PID and ESCC specification Evaluation Test report Qualification Test Report TES series have been qualified to meet requirements of ESCC 3012/004 detail specification. The matrix is significant improvement of maximum capacitance and lower ESR compare to the existing ESCC 3012/001 on solid tantalum chip capacitors |  |
| Application Any applications |  |
| Further Activities |  |
| Non Space Application |  |
| Publications Low ESR SMD Tantalum Capacitors for Aerospace Applications – SPCD 2013 |  |
| Source Reference Report |  |