ECSS Secretariat

ESA-ESTEC

Requirements & Standards Section

Noordwijk, The Netherlands

Space engineering

Electrical and electronic

Foreword

This Standard is one of the series of ECSS Standards intended to be applied together for the management, engineering, product assurance and sustainability in space projects and applications. ECSS is a cooperative effort of the European Space Agency, national space agencies and European industry associations for the purpose of developing and maintaining common standards. Requirements in this Standard are defined in terms of what shall be accomplished, rather than in terms of how to organize and perform the necessary work. This allows existing organizational structures and methods to be applied where they are effective, and for the structures and methods to evolve as necessary without rewriting the standards.

This Standard has been prepared by the ECSS-E-ST-20C Rev.2 Working Group, reviewed by the ECSS Executive Secretariat and approved by the ECSS Technical Authority.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Standard, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Requirements and Standards Section

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2022 © by the European Space Agency for the members of ECSS

Change log

| ECSS-E-20A 4 October 1999 | First issue |
| --- | --- |
| ECSS-E-20B | Never issued |
| ECSS-E-ST-20C 31 July 2008 | Second issue |
| ECSS-E-ST-20C Rev.1 15 October 2019 | Second issue Revision 1 |
| ECSS-E-ST-20C Rev.2 8 April 2022 | Second issue Revision 2 Changes with respect to ECSS-E-ST-20C (6 March 2009) are the following and identified in the document with revision tracking. Main changes are: - Clause 4.2.1.1 added due to addition of new clause 4.2.1.2 - Addition of requirements in new clause 4.2.1.2 “Reliable insulation” - The addition of the new clause 4.2.1.2 made it necessary to add the new header 4.2.1.1 “General requirements” to separate the requirement from the former clause 4.2.1 “Failure containment and redundancy” from the new requirements for “Reliable insulation”. - Update to cover the aspects of “reliable insulation” also known as “double insulation” - Addition of several terms in clause 3.2 related to the added subject of “Reliable insulation” Detailed changes Added requirements: - 4.2.1.1y; 4.2.1.2.2a; 4.2.1.2.3a-cc; Table 4-1; 5.5.2r-u. Modified requirements: - 4.2.2.1j; 4.2.2.2e; 5.5.2e-g; 5.5.2i and p Note; Table 5-2 Note; 5.6.2j and o; A.2.1d. Deleted requirements: - 5.8.1c. |

Table of contents

Change log	3

1 Scope	8

2 Normative references	9

3 Terms, definitions and abbreviated terms	10

3.1	Terms from other standards	10

3.2	Terms specific to the present standard	10

3.3	Abbreviated terms	17

3.4	Nomenclature	19

4 General requirements	20

4.1	Interface requirements	20

4.1.1	Overview	20

4.1.2	Signals interfaces	20

4.1.3	Commands	21

4.1.4	Telemetry	22

4.2	Design	23

4.2.1	Failure containment and redundancy	23

4.2.2	Data processing	34

4.2.3	Electrical connectors	36

4.2.4	Testing	38

4.2.5	Mechanical: Wired electrical connections	39

4.2.6	Miscellaneous	39

4.3	Verification	40

4.3.1	Provisions	40

4.3.2	Documentation	41

5 Electrical power	42

5.1	Functional description	42

5.2	Power subsystem and budgets	42

5.2.1	General	42

5.2.2	Provisions	42

5.3	Failure containment and redundancy	44

5.4	Electrical power interfaces	45

5.5	Power generation	46

5.5.1	Solar cell, coverglass, SCA and PVA qualification	46

5.5.2	Solar array specification and design	46

5.5.3	Solar array power computation	49

5.5.4	Solar array drive mechanisms	52

5.6	Electrochemical Energy Storage	52

5.6.1	Applicability	52

5.6.2	Batteries	53

5.6.3	Battery cell	55

5.6.4	Battery use and storage	56

5.6.5	Battery safety	57

5.7	Power conditioning and control	58

5.7.1	Applicability	58

5.7.2	Spacecraft bus	58

5.7.3	Battery Charge and Discharge Management	63

5.7.4	Bus under-voltage or over-voltage	64

5.7.5	Power converters and regulators	66

5.7.6	Payload interaction	67

5.8	Power distribution and protection	68

5.8.1	General	68

5.8.2	Harness	72

5.9	Safety	74

5.10	High voltage engineering	74

5.11	Verification	74

5.11.1	Provisions	74

5.11.2	<<deleted>>	75

6 Electromagnetic compatibility (EMC)	76

6.1	Overview	76

6.2	Policy	76

6.2.1	Overall EMC programme	76

6.2.2	EMC control plan	76

6.2.3	Electromagnetic compatibility advisory board (EMCAB)	77

6.3	System level	78

6.3.1	Electromagnetic interference safety margin (EMISM)	78

6.3.2	Inter-element EMC and EMC with environment	79

6.3.3	Hazards of electromagnetic radiation	79

6.3.4	Spacecraft charging protection program	80

6.3.5	Intrasystem EMC	81

6.3.6	Radio frequency compatibility	81

6.3.7	Spacecraft DC magnetic field emission	82

6.3.8	Design provisions for EMC control	82

6.3.9	Detailed design requirements	83

6.4	Verification	83

6.4.1	Verification plan and report	83

6.4.2	Safety margin demonstration for critical or EED circuit	83

6.4.3	Detailed verification requirements	84

7 Radio frequency systems	85

7.1	Functional description	85

7.2	Antennas	86

7.2.1	General	86

7.2.2	Antenna structure	87

7.2.3	Antenna interfaces	94

7.2.4	Antennas Verification	95

7.3	RF Power	96

7.3.1	Overview	96

7.3.2	RF Power handling (thermal)	96

7.3.3	Corona or Gas Discharge	97

7.3.4	Qualification for power handling and gas discharge	97

7.4	Passive intermodulation	98

7.4.1	Overview	98

7.4.2	General requirements	98

7.4.3	Identification of potentially critical intermodulation products	98

7.4.4	Verification	99

7.4.5	Qualification for passive intermodulation	99

7.5	Verification	99

8 Pre-tailoring matrix per space product and feature types	100

8.1	Introduction	100

8.2	Use of the inclusive and exclusive requirement categories	101

Annex A (normative) EMC control plan - DRD	166

Annex B (normative) Electromagnetic effects verification plan (EMEVP) - DRD	169

Annex C (normative) Electromagnetic effects verification report (EMEVR) - DRD	172

Annex D (normative) Battery user manual - DRD	174

Bibliography	175

Figures

Figure 5-1: Output impedance mask (Ohm)	62

Figure 5-2: Source and load impedance characterisation	70

Figure 5-3: Thevenin equivalent model	70

Figure 5-4: Norton equivalent model	71

Tables

Table 4-1: List of rigid and non-rigid materials	31

Table 5-1: Parameters for BOL worst and best case power calculations	51

Table 5-2: Additional power parameters for EOL worst and best case calculations.	52

Table 5-3: <<deleted, merged with new Table 8-3>>	75

Table 7-1: <<deleted, merged with new Table 8-3>>	95

Table 7-2: <<deleted, merged with new Table 8-3>>	99

Table 8-1: Definition of pre-tailoring matrix applicability statuses	103

Table 8-2: Definition of features for exclusive requirements	103

Table 8-3: Pre-tailoring matrix per “Space product and feature types"	104

# Scope

This Standard establishes the basic rules and general principles applicable to the electrical, electronic, electromagnetic, microwave and engineering processes. It specifies the tasks of these engineering processes and the basic performance and design requirements in each discipline.

It defines the terminology for the activities within these areas.

It defines the specific requirements for electrical subsystems and payloads, deriving from the system engineering requirements laid out in ECSS-E-ST-10 “Space engineering – System engineering general requirements”.

This standard may be tailored for the specific characteristic and constrains of a space project in conformance with ECSS-S-ST-00.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revision of any of these publications do not apply, However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the more recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

| ECSS-S-ST-00-01 | ECSS system – Glossary of terms |
| --- | --- |
| ECSS-E-ST-10 | Space engineering – System engineering general requirements |
| ECSS-E-ST-20-06 | Space engineering – Spacecraft charging |
| ECSS-E-ST-20-07 | Space engineering – Electromagnetic compatibility |
| ECSS-E-ST-20-08 | Space engineering - Photovoltaic assemblies and components |
| ECSS-E-ST-20-20 | Space engineering - Electrical design and interface requirements for power supply |
| ECSS-E-ST-33-11 | Space engineering – Explosive systems and devices |
| ECSS-E-ST-50-05 | Space engineering – Radio frequency and modulation |
| ECSS-E-ST-50-14 | Space engineering – Spacecraft discrete interfaces |
| ECSS-Q-ST-30-02 | Space product assurance – Failure modes, effects (and criticality) analysis (FMEA/FMECA) |
| ECSS-Q-ST-30-11 | Space product assurance – Derating – EEE components |
| ECSS-Q-ST-40 | Space product assurance – Safety |
| ECSS-Q-ST-70-12 | Space product assurance – Design rules for printed circuit boards |
| IEEE 145-1993 | Antenna terms |
| Impedance Specifications for Stable DC Distributed Power Systems, EEE transactions on power electronics, Vol. 17, no. 2, March 2002 | Impedance Specifications for Stable DC Distributed Power Systems, X. Feng, J. Liu, F.C. Lee, IEEE Transactions on power electronics, Vol. 17, no. 2, March 2002 |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this Standard, the terms and definitions from ECSS-S-ST-00-01 apply.

For the purpose of this Standard, the following terms and definitions from ECSS-E-ST-20-20 apply:

latching current limiter (LCL)

retriggerable latching current limiter (RLCL)

## Terms specific to the present standard

antenna farm

ensemble of all antennas accommodated on the spacecraft and provides for all the transmission and reception of RF signals

antenna port

abstraction of the physical connection among the antenna and its feeding lines, realised by means of connectors or waveguide flanges

antenna RF chain

sequence of microwave components inserted between an antenna input port or a BFN output port and a corresponding individual radiating element

Examples of microwave components are: ortho-mode transducers, polarisers, transformers as well as filters.

antenna support structure

part of an antenna having no electrical function, which can however impact its electrical performances, either directly due to scattering or indirectly

Example of indirect effect is induced thermo-elastic deformations.

array antenna

antenna composed by a number of, possibly different, elements that radiate RF signals directly into free space operating in combination, such that all or a part of them radiate the same signals

array-fed reflector antenna

antenna composed by a feed array, which can include or not a beam forming network, and one or more optical elements like reflectors and lenses

battery bus

primary power bus directly connected to the battery

Battery bus is sometimes called unregulated bus (although the battery charge is regulated).

beam forming network (BFN)

wave-guiding structure composed a chain of microwave components and devices aimed at distributing the RF power injected at the input ports to a number of output ports; in a transmitting antenna the RF power injected from the transmitter is routed to the radiating elements, in a receiving antenna the RF power coming from the radiating elements is routed to the antenna ports connected to the receiver

Examples of microwave components and devices are lines, phase shifters, couplers, loads.

conducted emission (CE)

desired or undesired electromagnetic energy that is propagated along a conductor

critical line

[CONTEXT: reliable insulation] line that is part of a critical net

As an example, limited to a solar array, typically a critical line is a line that carries the current of a section downstream from the electrical node collecting the current from the different strings that constitute the section. However, in case a short between strings within a section leads to a failure propagation, then correspondingly a string is considered as a critical line.

critical net

[CONTEXT: reliable insulation] electrical net that if short circuited with another electrical net or another conductor including satellite and launcher structure can cause critical effects

For “critical effects” see Table 4‐1 of ECSS‐Q‐ST‐30‐02.

critical pressure

pressure at which corona or partial discharge can occur in an equipment

diffusivity

ability of a body to generate incoherent diffuse scattering due to local roughness, inhomogeneity or anysotropy when illuminated by RF waves

depth of discharge (DOD)

ampere–hour removed from a battery expressed as a percentage of the nameplate capacity

double insulation

see “reliable insulation”

electrical bonding

process of connecting conductive parts to each other so that a low impedance path is established for grounding and shielding purposes

electromagnetic compatibility (EMC)

ability of equipment or an element to function satisfactorily in its electromagnetic environment without introducing intolerable electromagnetic disturbances to anything in that environment

electromagnetic compatibility control

set of techniques to effectively regulate the electromagnetic interference environment or susceptibility of individual space system components or both

They include, among others, the design, placement of components, shielding, and employment of rejection filters.

electromagnetic interference (EMI)

undesired electrical phenomenon that is created by, or adversely affects any device whose normal functioning is predicated upon the utilization of electrical phenomena

It is characterized by the manifestation of degradation of the performance of an equipment, transmission channel, or element caused by an electromagnetic disturbance.

electromagnetic interference safety margin (EMISM)

ratio between the susceptibility threshold and the interference present on a test point

emission

electromagnetic energy propagated by radiation or conduction

energy balance

situation in which the spacecraft energy budget is positive when elaborated over a considered period of time

1	Energy budget is generation minus consumption and losses.

2	The considered period of time can be one orbit, several orbits or any relevant mission period.

energy reserve

energy that remains available from the energy storage assembly at the worst-case, most depleted, point of nominal operations

It is important that the energy reserve is sufficient to permit reaching a safe operating mode upon occurrence of an anomaly

essential function

function without which the spacecraft cannot be recovered following any conceivable on-board or ground-based failure

Examples of unrecoverable spacecraft is when spacecraft cannot be commanded, or permanently losses attitude and control, or the energy balance is no longer ensured, or the spacecraft consumables (e.g. hydrazine or Xenon) are depleted to such an extent that more than 10% of its lifetime is affected, or the safety of the crew is threatened.

faulty signal

signal generated by a circuit, appearing at its interface to another circuit, going out of its nominal range because of a failure

foldback current limiter (FCL)

non latching current-limiting function where the current limit decreases with the output voltage

This function is used for power distribution and protection typically for essential loads.

fully regulated bus

bus providing power during sunlight and eclipse periods with a regulated voltage

grounding

process of establishing intentional electrical conductive paths between an electrical circuit reference or a conductive part and equipment chassis or space vehicle structure

grounding is typically performed for safety, functionality, signal integrity, EMI control or charge bleeding purpose.

high Priority telecommand (HPC)

command originated from ground and issued by the telecommand decoder for essential spacecraft functions without main on board software intervention

high voltage

AC or DC voltage at which partial discharges, corona, arcing or high electrical fields can occur

insulation

separation of elements either by material or by a distance

Etymologically, insulation is the act of protecting something with a material that prevents heat, sound, electricity, etc. from passing through. To insulate will then correspond to the action to protect by adding a material, an insulation (materials or device used for this protection).

invariable gap

physical distance among electrically conductive elements respecting the specified minimum limits independent from the stresses applied to the unit or part of the unit

1	Changes of the gap can result from effects of mechanical, thermomechanical or other nature, applied to the unit or part of the unit.

2	Stresses include the impacts of AIT operations, environmental tests, ageing and the use of insulation materials.

isolation

separation of elements put far from each other, with the notion of distance

To isolate is the action to separate by adding distance and to be isolated means protected by a distance.

lens antenna

antenna composed by a number of RF lenses and reflecting surfaces illuminated by a primary source, the feed

lightning indirect effects

electrical transients induced by lightning in electrical circuits due to coupling of electromagnetic fields

major reconfiguration function

function used to recover from system failures of criticality 1, 2 or 3

Criticality categories are defined in ECSS-Q-ST-30 and ECSS-Q-ST-40.

nameplate capacity

capacity stated by the manufacturer of an energy storage cell or battery

It is given in ampere-hours. It is not necessarily equal to any measurable capacity.

non essential loads

loads related to units which do not implement essential functions for the spacecraft

passive intermodulation products (PIM)

spurious signals generated by non-linear current-voltage characteristics in materials and junctions exposed to sufficiently RF high power carried by guided or radiated fields and currents, possibly triggered by microscopic mechanical movement

photovoltaic assembly (PVA)

power generating network comprising the interconnected solar cell assemblies, the shunt and blocking diodes, the busbars and wiring collection panels, the string, section and panel wiring, the wing transfer harness, connectors, bleed resistors and thermistors

primary cell or battery

battery or cell that is designed to be discharged once and never to be recharged

primary power bus

spacecraft electrical node closest to the power sources where power is controlled and made available to the user equipment

radiofrequency (RF)

frequency band used for electromagnetic waves transmission

radiated emission (RE)

radiation and induction field components in space

recharge ratio (k)

ampere–hours charged divided by the ampere–hours previously discharged, starting and finishing at the same state of charge

It is also known as the k factor.

reflector antenna

antenna composed by a number of reflecting surfaces, RF reflectors, illuminated by a primary source, the feed

reliable insulation

barrier between conductors or elements of an electrical circuit such that after any credible single failure, conductors or elements of an electrical circuit are still insulated from each other

The term “reliable insulation” is preferred to the term “double insulation” that was used in the previous version.

RF chain

sequence of microwave components inserted between the RF power amplifier and the antenna input port

RF lens

plastic, composite or metallic structure acting on transmitted RF waves to control the antenna pattern

Example of metallic structures are waveguide array lenses.

RF reflector

metallic or composite structure, possibly metallised or with printed or embedded metallic elements, acting on reflected RF waves to control the antenna pattern

Frequency and polarisation surfaces as well as other fully reflecting or partially reflecting and transmitting structures, also having non-uniform or anisotropic scattering behaviour, are considered reflectors

secondary cell or battery

battery or cell that is designed to be charged and discharged multiple times.

solar aspect angle (SAA)

angle between the normal to a solar panel and the sun vector

solar cell assembly (SCA)

solar cell together with interconnector, coverglass and if used, also a by-pass diode

susceptibility

malfunction, degradation of performance, or deviation from specified indications, beyond the tolerances indicated in the individual equipment or subsystem specification in response to other than intended stimuli

susceptibility threshold

interference level at a test point which just causes malfunction in the equipment, subsystem, or system

vacuum

environment with a pressure of 10 Pa or below

variable gap

physical distance among electrically conductive elements that can be subject to changes due to different effects

1	Changes of the gap can result from effects of mechanical, thermomechanical or other nature, applied to the unit or part of the unit.

2	Changes include the impacts of AIT operations, environmental tests, ageing and the use of insulation materials.

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| A | Analysis |
| AC | alternating current |
| AR | acceptance review |
| BOL | beginning–of–life |
| CDR | critical design review |
| DC | direct current |
| DDF | design definition file |
| DJF | design justification file |
| DOD | depth of discharge |
| DRB | Delivery review board |
| DRD | document requirement definition |
| DRL | document requirement list |
| EED | electro-explosive device |
| EGSE | electrical ground support equipment |
| EIDP | end item data-package |
| EMC | electromagnetic compatibility |
| EMCAB | EMC Advisory Board |
| EMEVP | electromagnetic effects verification plan |
| EMEVR | electromagnetic effects verification report |
| EMI | electromagnetic interference |
| EOL | end of life |
| EPS | electrical power subsystem |
| ESA | European space agency |
| ESD | electrostatic discharge |
| FCL | fold-back current limiter |
| FDIR | failure detection isolation and recovery |
| FMECA | failure mode effect and criticality analysis |
| INS | Inspection |
| ICD | interface control document |
| I-V | current-voltage |
| LCL | latching current limiter |
| MPPT | maximum power point tracker |
| MRB | manufacturing review board |
| OBC | on-board computer |
| PCB | printed circuit board |
| PDR | preliminary design review |
| PVA | photovoltaic assembly |
| QTR | qualification test report |
| RF | radio frequency |
| RLCL | retriggerable latching current limiter |
| ROD | review of design |
| PSA | Part stress analysis |
| SAA | solar aspect angle |
| SADM | solar array drive mechanism |
| SCA | solar cells assembly |
| SEE | single event effects |
| SEU | single event upsets |
| SPF | single point failure |
| SRR | system requirement review |
| T | test |
| TRB | test review board |
| TRR | test readiness review |
| TM&TC | telemetry/telecommand |
| UV | ultraviolet |
| VCD | verification control document |
| WCA | worst case analysis |

## Nomenclature

The following nomenclature applies throughout this document:

- The word “shall” is used in this Standard to express requirements. All the requirements are expressed with the word “shall”.

The word “should” is used in this Standard to express recommendations. All the recommendations are expressed with the word “should”.

- It is expected that, during tailoring, recommendations in this document are either converted into requirements or tailored out.

The words “may” and “need not” are used in this Standard to express positive and negative permissions, respectively. All the positive permissions are expressed with the word “may”. All the negative permissions are expressed with the words “need not”.

The word “can” is used in this Standard to express capabilities or possibilities, and therefore, if not accompanied by one of the previous words, it implies descriptive text.

- In ECSS “may” and “can” have completely different meanings: “may” is normative (permission), and “can” is descriptive.

The present and past tenses are used in this Standard to express statements of fact, and therefore they imply descriptive text.

# General requirements

## Interface requirements

### Overview

ECSS-E-ST-10 specifies that interfaces external or internal to a system are adequately specified and verified. The following requirements address this issue and are processed in phase B, C and D of a project (see ECSS-E-ST-10).

### Signals interfaces

ECSS-E-ST-20_0020001

Interface engineering shall ensure that the characteristics on both sides of each interface are compatible, including source and load impedances, the effects of the interconnecting harness and the grounding network between both sides comprising: common mode impedance conducted and radiated susceptibility and emission.

ECSS-E-ST-20_0020002

In order to minimize the number of interface types, standard interface circuitry shall be defined to be applied throughout a project.

ECSS-E-ST-20_0020003

Reconfiguration, high level or high priority command lines shall be immune to spurious activation.

ECSS-E-ST-20_0020004

The application of the nominal signals or a faulty signal to an un-powered interface shall not cause damage to that interface.

This requirement covers all types of interfaces. Standard interfaces are covered in clauses 4.2.4.3 and 4.2.4.4 of ECSS-E-ST-50-14.

ECSS-E-ST-20_0020005

An undetermined status at the interfaces of a powered unit shall not cause damage to an un-powered interface.

Undetermined status includes: non-nominal operating modes, permanent and non-permanent failure modes, powered and un-powered interfaces.

ECSS-E-ST-20_0020006

Signal interfaces shall withstand without damage positive or negative nominal voltages that are accessible on the same connector, coming from the unit itself, from the interfaced units or from EGSE.

### Commands

ECSS-E-ST-20_0020007

Every command (intended to be sent to the spacecraft) shall be assessed for criticality at equipment level, and confirmed at subsystem/system level.

The criticality of a command is measured as its impact on the mission in case of inadvertent function (erroneous transmission), incorrect function (aborted transmission) or loss of function. The definition of criticalities can be found in ECSS-Q-ST-30 and ECSS–Q-ST-40.

ECSS-E-ST-20_0020008

All executable commands shall be explicitly acknowledged by telemetry.

ECSS-E-ST-20_0020009

High Priority telecommand decoding and generation shall be independent from the main on-board processor and its software.

For failure case, refer to requirement 4.2.1.1a.

ECSS-E-ST-20_0020010

With the exception of pyrotechnic commands, the function of an executable command shall

not change throughout a mission, and

not depend on the history of previous commands.

ECSS-E-ST-20_0020011

For commands of category 1 and 2 criticality, at least two separate commands for execution: an arm/safe or enable/disable followed by an execute command shall be used.

For criticality categories, see ECSS-Q-ST-30 or ECSS-Q-ST-40.

ECSS-E-ST-20_0020012

The functionality shall be provided to repeat the transmission of all the executable commands without degradation of the function or a change of its status.

ECSS-E-ST-20_0020013

In case of critical commands of category 1 and 2, at least two physically independent electrical barriers, including associated control circuits, shall be implemented for arming and executing the command.

1	For criticality categories, see ECSS-Q-ST-30 or ECSS-Q-ST-40.

2	Mechanical barriers can be considered.

3	Physically independent electrical barriers and associated control circuits are the ones not sharing any hardware function and without risk of reciprocal failure propagation.

ECSS-E-ST-20_0020014

Processor and simple logic circuits shall not be able to issue category 1 and 2 critical commands without a ground commanded arm/safe or enable/ disable command.

To avoid inadvertent activation of processes enabled/disabled by category 1 or 2 critical commands during ground operations and in low earth orbit phases, it is necessary to foresee safety barriers (arm/safe commands) to inhibit the execution of such critical commands. Such safety barriers might be spacecraft skin connections (to be established or broken just before flight) or connections/disconnection plugs to be activated by launcher stages release (in flight). The activation/deactivation of such barriers has to be independent from on board processor.

ECSS-E-ST-20_0020015

Any on–board processing which issues commands to reconfigure subsystems or payloads shall be overridable and potentially inhibited by ground command.

For criticality categories, see ECSS-Q-ST-30 or ECSS-Q-ST-40.

ECSS-E-ST-20_0020016

No valid command shall be issued until the transmitter power supply is within operational voltage range and ready to transmit the command.

### Telemetry

ECSS-E-ST-20_0020017

Telemetry data devoted to the spacecraft subsystem and payloads monitoring shall allow

the retracing of the overall configuration at least up to all reconfigurable elements.

the location of any failure able to impact the mission performances and reliability at least up to all reconfigurable elements.

ECSS-E-ST-20_0020386

The operational status (On/Off, enabled/disabled, active/not-active) of each element of any telemetry acquisition chain should be provided to the on-board computer in order to determine without ambiguity the validity of the telemetry data at the end of the overall chain.

ECSS-E-ST-20_0020019

Primary bus load currents shall be monitored by telemetry, to enable, together with the bus voltage telemetry, a complete monitoring of a primary bus power load.

ECSS-E-ST-20_0020020

Telemetry shall be implemented to monitor the evolution of the power-energy resources and the source temperatures during the mission.

## Design

### Failure containment and redundancy

#### General requirements

ECSS-E-ST-20_0020021

Failure propagation shall meet the following conditions:

A single hardware failure does not propagate to neighbouring components circuits or interfaces in an undetermined way.

Failure propagation is verified by analysis.

Mechanical, thermal or electrical propagation of single hardware failures does not impair the corresponding protection or redundancy implemented at equipment or system level.

Single hardware failure does not propagate to equipment or functions under different contractual responsibility than the item where the failure takes place.

1	4.2.1.1a.4 is normally covered by specification of fault emission and tolerance conditions.

2	Component assembly (e.g. single cavity hybrid) and integrated circuits, especially if they contain redundancy or protection, require special attention.”

ECSS-E-ST-20_0020022

Redundant signal or power lines should be segregated via physically separated connectors and harnesses.

ECSS-E-ST-20_0020387

Routing of redundant power or signal lines within common harness or connector shall be justified by analysis showing that inside the electrical unit and at external connector interface level there is no potential single failure leading to affect both nominal and redundant lines or to generate electrical or electromagnetic interference between both.

1	Typical analysis for demonstrating compliance is provided within FMEA and EMC coupling analysis.

2	Potential single failure includes short-circuit cases.”

ECSS-E-ST-20_0020024

Redundant functions shall be physically separated with no risk of failure propagation by thermal or other coupling and as a minimum, contained within different integrated circuits to avoid failure propagation.

ECSS-E-ST-20_0020025

For redundant functions implemented on the same PCB, a physical separation shall be provided, with no risk of thermal or other failure propagation.

Example of physical separation are by a minimum distance, insulation, or cut-out.

ECSS-E-ST-20_0020026

For redundant functions implemented on the same PCB, any deviation of the physical separation specified in 4.2.1.1e shall be tracked in the Critical item List.

ECSS-E-ST-20_0020027

<<deleted>>

ECSS-E-ST-20_0020028

In case a cold redundant function is simultaneously activated together with the nominal one, by a deliberate or wrong command or due to a fault, this shall not induce permanent degradation of either of the two functions or loss of the mission before FDIR action.

E.g. thermal and EMC functions.

ECSS-E-ST-20_0020029

<<deleted>>

ECSS-E-ST-20_0020030

Any active equipment, excluding heaters, continuously dissipating more than 20 W in nominal or failure condition shall include a temperature monitoring function, which is implemented and provides continuous temperature monitoring.

ECSS-E-ST-20_0020031

In case of signal cross-strapping, no single failure of either interface circuit shall propagate to the other one.

ECSS-E-ST-20_0020032

In the case of hot redundant essential functions, latching protection shall not be used unless it has an autonomous periodic reset.

ECSS-E-ST-20_0020033

Disabling critical on-board autonomous functions shall be allowed only if an interlock mechanism is implemented, which prevents the disabling of both main and redundant functions at the same time.

Critical functions refers to functions which prevent the satellite from being recovered when both main and redundant are switched off, e.g. RF receiver or command decoder.

ECSS-E-ST-20_0020034

Any protection latch, which does not have autonomous reset capability, shall be at least re-settable from ground command.

ECSS-E-ST-20_0020035

Any protection of an essential function shall not share with the essential function itself the same component or integrated circuit nor utilize common references or auxiliary supply.

ECSS-E-ST-20_0020036

Essential functions shall not be lost upon a single failure of other functions which are centrally powered or controlled.

1	E.g. on synchronization and auxiliary supply.

2	That can imply the capability of an equipment performing an essential function of operating independently of any external synchronization and auxiliary power supply if these are not SPF free.

ECSS-E-ST-20_0020037

For essential functions supplied by an FCL, lock-up phenomenon requiring recovery via the removal of external power shall be prevented.

ECSS-E-ST-20_0020038

All units to be powered during launch shall be designed for operation with critical pressure.

ECSS-E-ST-20_0020039

A venting analysis shall be performed for all units not designed to operate under critical pressure and not powered during launch, to determine when they can safely be turned-on.

ECSS-E-ST-20_0020040

Any on-board autonomous function, the failure of which can result in malfunctions of category 1 and 2 criticality, shall have override capability.

Examples of override are:

a simple inhibition or isolation (e.g. cold or hot redundant chain[s] exists)

an H/W reset (e.g. in case of SEU)

an inhibition + by-pass (e.g. a stepper motor control loop by-passed by a direct step by step command as back-up)

ECSS-E-ST-20_0020041

Any on-board autonomous protection override, leading to hazardous situation for the mission (category 1 and 2 criticality), shall not be implemented.

E.g. an LCL function for instance protecting the main power Bus against a short circuit at Bus user level or Main Bus over-voltage protection.

ECSS-E-ST-20_0020042

SEE shall not activate protection circuits of essential functions.

Mitigation techniques can be implemented to avoid such phenomena: filtering, majority voting, etc

ECSS-E-ST-20_0020043

The spacecraft electrical system shall be single failure tolerant for unmanned mission and double failure tolerant for manned mission.

ECSS-E-ST-20_0020407

Occurrence of a non-destructive SEE after a failure shall not lead to the loss of the mission.

Accordingly, a non-destructive SEE cannot derail a reconfiguration process after first hardware failure.

ECSS-E-ST-20_0020426

For electrical connections of critical lines directly exposed to free space, the distance between power lines or between power and return lines shall not be considered an effective insulation from dielectric point of view, including inside connectors.

#### Reliable insulation

##### Reliable insulation assumptions

“Reliable insulation” was previously called “Double insulation”.

Reliable Insulation requirements present in this standard:

are intended to avoid short circuit of critical lines or nets due to foreign conductive particles;

are intended to reduce to negligible the risks of short circuit of critical nets or lines due to errors or marginality in design, manufacturing or integration;

do not cover short circuits that might be caused by meteorites hitting a spacecraft;

do not intend to prevent insulation losses inside EEE components. The effects of components internal short-circuit risk is managed through FMECA and mitigated by design. Nevertheless, when an internal short-circuit risk between the chip or the leads and the component case (if metallic) is possible, external insulation or other mitigation actions are required.

1	For EEE components different ECSS standards are applicable

2	Wires/cables are an exception to this assumption. For them all layers of insulation, including those between wire cores and the respective wire/cable shield, are subject to reliable insulation.

For EEE components (like opto, relay, hybrid, MOS-FET) their internal construction is key to demonstrate compliance to reliable insulation requirements.

Additionally for solar arrays, reliable insulation requirements present in this standard:

are intended to avoid short circuit of critical lines or nets due to foreign conductive particles, or to plasma or ESD effects.

##### Reliable insulation applicability

ECSS-E-ST-20_0020427

Reliable insulation requirements in this standard shall apply to all critical nets.

1	Examples of critical nets are:

non–protected sections of a main bus power distribution system up to and including the first overload or overcurrent protection device;

cross‐strapped power or signal critical lines or nets and associated common links, connecting main and redundant circuits and matrices;

lines or nets where short circuits would result into failure propagation of critical nature;

lines or nets where basic insulation provisions may be damaged by mission environment or mechanical interference.

2	Insulation loss due to mechanical interference is for example present when there are sharp edges and there is the need for harness protection.

3	For Reliable Insulation rules for PCBs, refer to ECSS-Q‐ST‐70‐12.

4	For a solar array, it is assumed that Reliable Insulation requirements apply to lines carrying the current of a section because they are critical; otherwise the Reliable Insulation requirements are tailored in an appropriate way, for example by increasing solar array capability allowing loss of at least one section.

##### Reliable Insulation requirements

ECSS-E-ST-20_0020428

For those space applications where electrical lines insulation can be damaged by meteorites or micro‐meteorites, dedicated Reliable Insulation requirements or agreed equivalent solution shall be established before satellites development according to the environment encountered and to the required reliability figure at system level.

ECSS-E-ST-20_0020429

The presence of foreign particles inside equipment, subsystem or system shall be treated as a credible event.

1	Example of foreign particles are the materials used for manufacturing, piece of wire, nuts, washers, components and pins.

2	Visual inspection is normally done to avoid the presence of foreign conductive particles that can create loss of insulation. Such particles might be there before equipment/subsystem /system closure or integration, or being the result of vibration exposure due to poor fixation. Potential particles and the activities that leads to emerging of these particles are identified and control plan is created if needed.

3	Visual inspection is done on equipment, subsystem or system level before final assembly to avoid the presence of such particles that might create loss of insulation.

4	The visual inspection method used is compatible with the minimum size of the particle to be detected.

ECSS-E-ST-20_0020430

The non-existence of foreign particles shall be verified by visual inspection or test.

ECSS-E-ST-20_0020431

The presence of foreign particles shall be assessed, recorded in the DJF, and resolved if not compatible with the application.

ECSS-E-ST-20_0020432

All conductive elements of critical nets shall be provided with insulation means.

Insulation means are intended as insulating coating, or confinement of bolted connections or conductor inside insulation box (for example in the case of bus bars). Distance alone is not considered as a valid insulation means.

ECSS-E-ST-20_0020433

Requirement 4.2.1.2.3e shall not apply to:

Solar Array solar cells, by-pass diodes, conductive coverglasses, solar cell interconnectors, blocking diode assembly and terminations, where no insulating material apart from vacuum is present among them,

Solar Array Drive Mechanism slip rings, and

Battery cell technologies for which loss of insulation is covered by other specific provisions.

For example NiH2.

ECSS-E-ST-20_0020434

To ensure an effective insulation from dielectric point of view, the gap among critical lines or between a critical line and any other conductor shall comply to the following two cases, for operational voltages up to 250 V:

Case 1: invariable gap greater than 1mm : use of 1 insulating material (non-gaseous / non-vacuum);

Case 2: variable gap or gap smaller than 1mm : use of 2 different materials (including at least 1 rigid).

1	For rigid material, see requirements 4.2.1.2.3o and 4.2.1.2.3p.

2	In case 2, two different materials or two layers of individually cured material are used.

3	In the few particular cases where no suitable insulation layer can be added, the distance between the critical potential and any other potential is increased and guaranteed. The absence of risk of a particle with a size that could jeopardize the insulation at that level is guaranteed by inspection and by design (e.g. quasi-hermetic area).

ECSS-E-ST-20_0020435

The worst case minimum thickness of the materials for reliable insulation shall be taken into account, and not their nominal or design values.

ECSS-E-ST-20_0020436

It shall be demonstrated that no critical air bubbles to cause discharges are present in adhesives.

1	The absence of critical air bubbles in adhesives is demonstrated by a quality control sample and DPA.

2	Adhesives are normally not considered as insulators for reliable insulation purposes.

ECSS-E-ST-20_0020437

The physical implementation of the reliable insulation shall be documented and recorded.

The implementation of reliable insulation is provided either in a project specific or in a standard document from the equipment provider and recorded in the respective manufacturing documentation (e.g. traveler or inspection record).

ECSS-E-ST-20_0020438

The rigid and non-rigid materials used to ensure reliable insulation shall be listed in the document mentioned in 4.2.1.2.3j with their thickness and characteristics or the reference to the respective ESCC standard be indicated.

As an information on rigid and non-rigid materials, consult Table 4-1.

Table 4-1: List of rigid and non-rigid materials

| Material | Characteristic |
| --- | --- |
| Epoxy fiberglass | Rigid (thickness ≥ 100 μm) |
| PEEK | Rigid (thickness ≥ 100 μm) |
| ULTEM Polyetherimide (PEI) | Rigid (thickness ≥ 100 μm) |
| DURATRON Polyetherimide (PEI) | Rigid (thickness ≥ 100 μm) |
| KYNAR (shrinkable sleeves) | Rigid |
| Epoxy FR4 laminate and pre-preg | Rigid |
| Polyimide laminate and pre-preg | Rigid |
| Ceramic (no crack risk under constraint) | Rigid |
| Scotchweld EC2216 (epoxy resin) | Rigid |
| Enamel grade 2 | Rigid |
| Araldite AW 139 (epoxy resin) | Rigid (thickness ≥ 100 μm) |
| Stycast 2850FT and 2651 (epoxy resin) | Rigid (thickness ≥ 100 μm) |
| Kapton | Rigid (thickness ≥ 50 μm) |
| Solithane 113 + cabosil (silice load) used as resin | non-rigid |
| Mapatox41B varnish (PCB conformal coating) | non-rigid |
| DC 93500 (varnish) | non-rigid |
| DC 6-1104 (thick varnish) | non-rigid |
| Cho-therm T500 and R1671 | non-rigid |
| TIM (thermal material interface) | non-rigid |
| RTV silicon | non-rigid |
| Mapsil 213 | non-rigid |
| Enamel grade 1 | non-rigid |

ECSS-E-ST-20_0020439

The electrical insulation materials shall have a dielectric strength in excess of 2 times the peak operative voltage in the actual application.

As a reference, the dielectric strength value of Air (3 kV/mm) can be used as selection criterion for materials.

For resistivity, typical value is 10E9 Ohm.m.

The selection of insulation material needs to take into account potential charging/ESD constraints (see ECSS-E-ST-20-06).

ECSS-E-ST-20_0020440

The electrical insulation materials shall be resistant to the different phases of the unit MAIT (Manufacturing, Assembly, Integration and Test) process.

ECSS-E-ST-20_0020441

Materials used in direct contact between 2 parts to be reliably insulated and between which an electrical field is present, shall not be contaminated, or contain other non-homogeneous pathway for leakage current to develop in clean room conditions, with max 65 % RH.

For instance, epoxy fiberglass or laminate/pre-preg can be procured against standards that allow some contamination to be present. Other types of non-homogenous pathway could be caused by agglomeration of fillers or flame retardant.

ECSS-E-ST-20_0020442

In relation to reliable insulation, a material shall be treated as rigid if its Vickers hardness is greater than 20, either in raw state or after treatment.

ECSS-E-ST-20_0020443

In relation to reliable insulation and in alternative to requirement 4.2.1.2.3o, a material shall be considered as rigid if the insulation layer produced with it can guarantee the required minimum thickness, for all required operative conditions including manufacturing and test process.

The required minimum thickness is guaranteed either by process or by verification.

ECSS-E-ST-20_0020444

For critical lines and nets involving voltages exceeding 250 V, a minimum of 2x the distance identified by a dedicated analysis shall be used.

The dedicated analysis is performed to demonstrate the ability to cope with HV effects for critical nets.

ECSS-E-ST-20_0020445

For critical lines and nets involving voltages exceeding 250 V, insulation shall be demonstrated by test with the application of 2x the operational voltage limit.

ECSS-E-ST-20_0020446

A foreign particle should not jeopardise one insulation layer of the reliable insulation.

ECSS-E-ST-20_0020447

Conductors with sharp edges, or geometries preventing the access to conformal coating shall be locally insulated with suitable material.

Example of geometries with difficult access are pins of PCB connectors or bus bars, components leads end (at PCB verso).

ECSS-E-ST-20_0020448

Where the use of shielded wires and/or over-shielded harness together with reliable insulation is required:

the issue shall be brought to customer attention, and

the absence of any short-circuit risk between wire core and shield shall be demonstrated all along the harness, as well as at shield grounding connection points.

It is best practice not to use shielded or over-shielded harness and wires for signals to be reliable insulated.

ECSS-E-ST-20_0020449

In case flexible connections are used, i.e. flex PCBs, the applicable requirements of clause 8 of ECSS-Q-ST-70-12 shall be applied.

ECSS-E-ST-20_0020450

In the connectors mating area, the risk of short-circuit between a pin to be reliably insulated and a conductor at any other potential, due to a particle or to a bent pin shall be avoided.

1	For example, conductor at any other potential can be a pin at a different potential or connector metallic case.

2	Such short-circuit risk can be mitigated either by design (interfacial seal implementation, provision of not mounted pins locations, no risk of pin bending by design) or by inspection (absence of particle).

ECSS-E-ST-20_0020451

Critical lines and nets shall be identified as part of a FMECA process or of a dedicated analysis at the beginning of the system, subsystem or equipment development, at relevant PDR at the latest.

ECSS-E-ST-20_0020452

A dedicated analysis shall confirm that reliable insulation of critical lines or nets is not jeopardized by failures, at system, subsystem or equipment level.

Failures can be of electrical, thermal or mechanical nature (for example issues of interference especially in integration and manufacturing phases). Dissipative failures are part of the analysis required.

ECSS-E-ST-20_0020453

Critical lines or nets shall be clearly identifiable in electrical diagrams and in layout and assembly drawings.

The critical lines are preferably identified for example with colour codes on assembly drawings or, if not possible, via a statement or note on the drawing.

ECSS-E-ST-20_0020454

The electrical diagrams, layout and assembly drawings providing the information of the critical lines or nets may be part of the relevant analysis report.

ECSS-E-ST-20_0020455

Compliance to Reliable insulation requirements of critical lines or nets shall be verified by review of design as well as by inspection and/or by test if possible.

The method of verification is matter of agreement between customer and supplier.

ECSS-E-ST-20_0020456

Verification of reliable insulation requirements should take place any time when it might be jeopardised by integration or rework activities, exposure to specific environment, electrical connection or disconnection.

### Data processing

#### Overview

All operational and mission specific data are processed for acquisition, algorithm application, transmission, storage. On board time is managed by data handling subsystem, in line with the mission requirements. Data processing includes the man machine, interface if any. The data processing system includes all hardware and software elements used for that purpose (e.g. microprocessor and its instruction set, interface means, data busses and remote terminals).

#### Provisions

ECSS-E-ST-20_0020044

For technical budgets and margin policy the requirements of ECSS-E-ST-10 clause 5.4.1.2 shall be applied.

ECSS-E-ST-20_0020388

The margin for available memory size and load factors of processors should be

for new developments, 50 % as a minimum at PDR for new on board software parts;

25 % at launch.

ECSS-E-ST-20_0020389

The margin on the throughput of on-board communication networks should be

for new developments, 50 % as a minimum at PDR on the average throughput;

such that real time overflow is avoided.

ECSS-E-ST-20_0020390

In the absence of specific mission requirements the following applies: After error correction, reset or data corruption of main functions at equipment level should be kept to a rate of occurrence less or equal to 10-4 per day for worst case conditions of environment.

ECSS-E-ST-20_0020391

For programmable logic devices (FPGA), the available margin of unused blocks and pin shall be agreed at FPGA SRR based on design complexity, requirement stability and experience return.

1	To consider experience return, a factor of heritage of each block/IP can be used to multiply the estimated occupancy to have at the end, an overall occupancy < 100%. Possible example of heritage factor are:

completely new component: 40 %

full heritage for code, but ported to another technology: 25 %

full heritage for both code and technology: 10 %

If the requirement are not stable or possible modification to accommodate future products/designs need are foreseen, a factor can also be applied to the overall design (i.e. 10 %).

2	Design is highly re-used (> 80 % of the overall design), technology is mature (i.e. RTAX2000) and requirements are considered stable.

A margin of 20 % is not considered a high risk.

A margin of 10 % is still acceptable, but a mitigation plan can be requested (i.e. identifying some functions that can be removed or optimised, check if a pin-compatible bigger device is available, etc.)

3	Most of the design is new and it is be implemented in relatively new technology (i.e. RTG4).

In this case the risk related to low margin is much higher and a margin lower than 40 % is considered high risk. The risk can be assessed, the cost/benefit relation be evaluated and eventually the device selection might be re-assessed.

### Electrical connectors

ECSS-E-ST-20_0020049

A connector carrying source power or external test connectors on units shall have no contact areas exposed to possible short circuit during mating and de-mating process.

They generally are female type connectors.

ECSS-E-ST-20_0020050

All external test connectors on a unit and spacecraft skin connectors shall be covered for flight.

Umbilical connectors (detached during launch) are not part of skin connectors (detached or connected before launch).

ECSS-E-ST-20_0020392

The test connector covers should be metallic or metallized and grounded to structure.

ECSS-E-ST-20_0020052

The use of a connector saver for ground testing shall not alter the performance of the equipment.

RF savers are known to introduce extra insertion losses, in the range of 0,1 dB.

ECSS-E-ST-20_0020053

It shall be ensured that erroneous mating is avoided by connector keying or marking.

The requirement is met either by harness routing, or by using keyed connectors, or adequate positioning of connectors, or connectors of different type or size, or connector marking.

ECSS-E-ST-20_0020054

If the equipment has several connectors, visibility and clearance around each of them should be such as to enable mating or de-mating without disturbing others already in place or necessitating custom–made tooling.

A usual practice is the insertion of a breakout box for trouble shooting.

ECSS-E-ST-20_0020393

For supplies and signals of pyrotechnics and non-explosive single shot device drivers, different connectors should be used for different classes of electrical functions.

ECSS-E-ST-20_0020056

When 4.2.3g is not met, power, signals, and telemetry shall be separated in the connector by a set of unused pin locations.

ECSS-E-ST-20_0020057

Except when pin allocation is imposed by a standard specification, spare contacts or sockets should be available on each connector.

ECSS-E-ST-20_0020058

For new developments, when the connection is not aligned to a defined standard, 10% spare contacts at unit PDR and at least 5 % at CDR shall be achieved with in any case a minimum of two spare contacts available at CDR.

ECSS-E-ST-20_0020059

In the absence of grounding provision at connector shell level, at least one contact per connector shall be connected to the unit structure as provision for potential additional grounding at subsystem or system level.

ECSS-E-ST-20_0020060

Provision shall be taken to avoid arcing or short circuits in connectors.

For example: unused pins, placed between positive and return lines; specific connector design.

ECSS-E-ST-20_0020061

The following shall be performed for any connector the loss of which can lead to the loss of the mission:

Document the connector in the single point failure list

Verify and document its integrity up to the highest spacecraft integration level, to avoid accidental demating.

ECSS-E-ST-20_0020062

<<deleted>>

ECSS-E-ST-20_0020063

Battery and solar array power shall be distributed by multiple contacts on both positive and return lines.

### Testing

ECSS-E-ST-20_0020064

Test-stimulus points shall be accessible without the need of modifying the electrical configuration of an item of equipment.

ECSS-E-ST-20_0020065

Test-stimulus points shall be protected for flight operation.

ECSS-E-ST-20_0020394

For the purpose of meeting requirement 4.2.4a and 4.2.4b, dedicated test connectors should be used.

ECSS-E-ST-20_0020067

<<deleted>>

ECSS-E-ST-20_0020068

Test points on equipment shall be protected against damage up to the maximum fault voltage present on the connector either coming from the equipment or the EGSE.

It is expected that the design of the EGSE incorporates protections limiting the fault voltage emission to a level acceptable by the unit.

ECSS-E-ST-20_0020069

Test points on equipment shall be such that unintentional connection of these points to ground does not damage the equipment.

ECSS-E-ST-20_0020070

The redundancy of parts and functions, which failure can lead to the loss of the mission or human injury, shall be verified by test simulating the failure event.

ECSS-E-ST-20_0020071

Circuits meant for on-ground testing or unused circuits shall not create or be sensitive to the noise expected during operation.

For example spacecraft stimuli, unit test points, unused operational amplifiers or comparators.

ECSS-E-ST-20_0020072

The protection of functions, which failure can lead to the loss of the mission or human injury, shall be verified by test simulating the failure event.

ECSS-E-ST-20_0020073

The test of a protection function or a redundant function shall present no risk of stress or failure propagation due to the injection of stimuli.

ECSS-E-ST-20_0020074

<<deleted>>

ECSS-E-ST-20_0020075

Hot redundant functions and protection functions that cannot be tested beyond unit level shall be identified in the critical item list.

ECSS-E-ST-20_0020076

All redundant functions and protection functions shall be tested, up to the highest possible level of integration of the unit.

ECSS-E-ST-20_0020077

Redundant units within a system shall be verified by test at system level.

ECSS-E-ST-20_0020078

Protection functions within a unit protecting other units shall be verified by test at system level or at unit level with representative interfaces.

1	This is the case e.g. of a battery discharge regulator switching OFF autonomously at low input voltage to protect a Li-ion battery against irreversible damage to overdischarge.

2	In the case of the LCL in power distribution, the limitation of current at turn-on is considered representative for validation of the protection at system level.

### Mechanical: Wired electrical connections

ECSS-E-ST-20_0020079

Wired electrical connections shall contain stress relief.

The objective is to avoid excessive mechanical loads on wires.

### Miscellaneous

ECSS-E-ST-20_0020080

Each item shall be directly interchangeable in form, fit, and function with other equipment of the same part number and of the same qualification status.

ECSS-E-ST-20_0020081

The uniformity of the electrical performance characteristics of the units shall enable equipment interchange provided a minimum set of adjustments and recalibration as described in the unit user’s manual.

ECSS-E-ST-20_0020082

When components operating in a single event are used, 4 times the quantity to be used for flight units shall be procured as one lot: 25 % for the lot acceptance test, 25 % for flight use, 25 % for spares and 25 % for a confirmation test near to the launch date.

Example of such components are fuses.

ECSS-E-ST-20_0020083

The number of components to be procured shall be defined to ensure, as a minimum, the quantity needed for flight and flight spares, plus the number of components to be tested at incoming reception and components to be tested just before launch in case of alert or failure.

## Verification

### Provisions

ECSS-E-ST-20_0020084

The requirements of this Clause 4 should be verified by the verification methods and at the verification points listed in Table 8-3.

1	Table 8-3 can be used as a starting point for the definition of the verification methods.

2	For more details on the verification strategy see also ECSS-E-ST-10-02 in particular the requirements 5.2.1c, d and e.

ECSS-E-ST-20_0020408

In case verification by analysis of an electrical part or circuit is not possible by lack of data, complementary verification by test shall be performed.

Electrical part or circuit encompasses in particular to EEE components, solar cell or solar array, battery cell or battery assembly.

### Documentation

ECSS-E-ST-20_0020085

The design report, PSA, WCA, FMECA, thermal analysis, radiation analysis, EMC analysis and the detailed circuit diagrams including component values, shall be part of the Design Definition and Justification Files.

ECSS-E-ST-20_0020086

Failure modes of all components used in a unit shall be defined.

ECSS-E-ST-20_0020087

FMECA shall be performed and based on the failure modes previously defined at component level.

ECSS-E-ST-20_0020409

The Design Definition and Justification Files shall be delivered by the supplier to the customer at the agreed verification points in compliance with Table 8-3.

ECSS-E-ST-20_0020380

Table 4-1: <<deleted, merged with new Table 8-3>>

# Electrical power

## Functional description

Electrical power is used by all active spacecraft systems and equipment for their operation. Electrical power engineering includes power generation, energy storage, conditioning, line protection and distribution as well as high voltage engineering.

## Power subsystem and budgets

### General

ECSS-E-ST-20_0020088

Budgets and margins shall be established during Project phase B, and reviewed in all subsequent phases of the project.

### Provisions

#### Power subsystem

ECSS-E-ST-20_0020089

The power subsystem of a spacecraft shall be able to generate, store, condition, distribute and monitor the electrical power used by the spacecraft throughout all mission phases in the presence of all environments actually encountered.

For passivation, refer to ECSS-U-AS-10.

#### Engineering process

ECSS-E-ST-20_0020090

An analysis of power demand versus power available shall be performed, including average peak power, for all phases of the mission.

ECSS-E-ST-20_0020091

An analysis of the energy demand versus energy available shall be performed in all phases of the missions, including inrush power demands, eclipses, solar aspect angle and depointing and also failure mode affecting the power system.

Failure modes can result in transient or permanent overconsumption.

ECSS-E-ST-20_0020092

A power budget shall be established based on the peak power values and an energy budget based on the average power values for all mission phases.

ECSS-E-ST-20_0020093

A plan shall be established for the maintenance and periodical review of the budget established in requirement 5.2.2.2c during all project phases.

These budgets take into account:

spacecraft–sun distance,

sun and eclipse durations,

solar aspect angle,

shadowing,

pointing accuracy,

environmental temperature and degradation effects,

reliability and safety aspects,

any one failure in the system (two failures for manned mission) not counting solar array string and battery cell failure, that are provisioned for achieving reliability of these item, but including transient or permanent overconsumption of a load,

Failure detection, isolation and recovery scenarios.

ECSS-E-ST-20_0020094

A system margin of not less than 5 % at AR on available power and energy shall be included in the budget, available (as a minimum) with the solar array string losses as defined by the customer with the minimum of one string lost and one battery cell failed during all the designed life of the power system including all spacecraft modes of operation.

ECSS-E-ST-20_0020095

When actually using a MPPT, it shall be ensured that the transferred power is at least the required power, independent of the solar array operating conditions or environment.

The factors affecting the transferred power include in particular the following ones:

Consideration of possible multiple local maximum power points,

Different illumination (e.g. resulting from different SAA) for different SA sections,

Temperature dispersion,

Shadowing,

Dispersion of the SA string I/V curves,

Tracking accuracy and response time of MPPT control,

Dynamic stability of MPPT control loop,

Power conversion efficiency of MPPT converters.

## Failure containment and redundancy

ECSS-E-ST-20_0020096

Any protection function of a power converter or regulator preventing failure propagation shall:

not be implemented in the same integrated circuit, and

not utilize common references.

ECSS-E-ST-20_0020097

It shall not be possible to inhibit a protection feature if such an inhibition can lead to the loss of the main primary power bus in case of a single failure at spacecraft level.

ECSS-E-ST-20_0020098

In flight operation, if primary power bus shutdown happens, the system, including the power subsystem, shall be able to restart.

1	Startup is possible as soon as sufficient power is available for a sufficient time from any source of the primary power bus, without considering failure for unmanned mission, or after single failure for manned mission.

2	This requirement can imply that the battery is sufficiently recharged before its energy starts to be provided, for example to recover spacecraft attitude.

ECSS-E-ST-20_0020099

<<deleted>>

## Electrical power interfaces

ECSS-E-ST-20_0020100

The electrical power interface internal or external to the power subsystem shall be specified, including source and load impedance.

Main examples of internal power interface are battery to conditioning electronics, SA to conditioning electronics. Main examples of external power interface are primary power conditioning electronics to loads, EGSE to and from primary power conditioning electronics, umbilical to and from primary power conditioning electronics. Docked modules on board of spacecraft, like orbiter or rover, are concerned as well.

ECSS-E-ST-20_0020101

<<deleted>>

ECSS-E-ST-20_0020102

The availability of the specified solar array power up to the power conditioning electronics shall be verified as follows:

on solar array level, availability of the specified solar array power up to and including the solar array connector by means of flasher tests, supported by correlated analysis,

on spacecraft level, full steady-state solar array power conditioning capability from solar array connector to power conditioning electronics, including solar array drive mechanism if any and harness, using solar array simulator,

and finally, on spacecraft level, correct electrical connection of the solar array to the power conditioning unit by means of a flood test, that is by illumination of the solar array with a portable continuous lamp, checking if the generated voltage and current are detected at power conditioning side.

ECSS-E-ST-20_0020103

The solar array interface voltage shall be defined at the solar array connector interface.

ECSS-E-ST-20_0020104

The solar array interface voltage shall include voltage losses within the electrical circuitry of the solar array, including at least blocking diodes, wiring resistance and losses associated with harness interconnections in operational conditions.

## Power generation

### Solar cell, coverglass, SCA and PVA qualification

For the qualification of solar cells, protection diodes, coverglass, SCA and PVA, see ECSS-E-ST-20-08.

### Solar array specification and design

ECSS-E-ST-20_0020105

The solar array shall be specified to provide the requested power and ensure the energy balance in each mission phase during operational life including any string loss tolerance defined by the customer, spacecraft charging effects and worst case conditions.

1	The solar array is designed to be single-failure tolerant at string level.

2	In order to meet the solar array reliability requirements, the impact of other loss factors can lead to the addition of other spare strings.

3	The computation of the energy balance is a typical task for the large system integrator based on defined power points expressed as a specification to the SA manufacturer.

ECSS-E-ST-20_0020106

<<deleted>>

ECSS-E-ST-20_0020107

<<deleted>>

ECSS-E-ST-20_0020108

Provision shall be made against potential failure propagation in case of short-circuit failure of a solar array section or short circuit of its connection to the power subsystem.

In particular short-circuit of solar array interface (for example during ground operations) can lead to short-circuit of the main bus or the battery if no protections (e.g. blocking diodes in conditioning electronics) are implemented.

ECSS-E-ST-20_0020109

The solar array design shall be such that charging phenomena do not degrade the performance of the solar array below the ones specified in 5.5.2a and meeting the requirements specified in clauses 5.5.1.5 of ECSS-E-ST-20-08.

ECSS-E-ST-20_0020110

The following requirements and omissions for ESD shall apply:

ESD testing is performed.

ESD testing is be performed in line with ECSS-E-ST-20-06 and ECSS-E-ST-20-08 demonstrating minimum safety margins as specified in clauses 5.5.1.5.of ECSS-E-ST-20-08.

ESD testing of solar arrays can be omitted for solar arrays with voltage less than or equal to 10 V, independent of current and gap distance.

For voltages below 30 V ESD testing can be omitted as well, provided the current remains below 0,6 A and gap width above 0,9 mm.

ECSS-E-ST-20_0020111

In all configurations, stowed and deployed, the solar array , deployable and body mounted, and spacecraft structure shall be insulated from each other, disregarding the bleed resistor.

1	Examples of such structural parts of the SA panel are hold-down mechanisms, yokes, hinges and CCL (Closed Cable Loop).

2	Typical value of insulation is in the order of 100 MΩ, and typical bleed resistor is in between 2 kΩ to 20 kΩ.

ECSS-E-ST-20_0020112

In the flight configuration, means to prevent differential voltage due to electrostatic charging between solar array structure and the spacecraft electrical ground reference shall be implemented.

ECSS-E-ST-20_0020113

In the flight configuration, bleeding resistors shall be implemented.

1	For typical solar array designs the panel substrate is conductive and mounted electrically insulated from the Spacecraft structure. In order to control the charging of the solar array substrate(s), the panel substrates are connected via bleeder resistors to spacecraft ground.

2	Bleeding resistors are used to control both electrostatic charging and power loss from the solar array section and dissipation in the resistor itself in case of a cell string to panel short (including de-rating).

3	Bleeding resistors can be considered as a separate protection together with a parallel insulating material in view of reliable insulation (reliable double protection).

ECSS-E-ST-20_0020114

At solar array level, one short between a solar cell string and a conductive panel structure shall not produce any solar array power loss.

ECSS-E-ST-20_0020115

At solar array level, in case of two shorts on the same panel, the power loss shall not be more than the power of two strings.

ECSS-E-ST-20_0020116

The PVA layout shall be designed to meet the solar array magnetic moment requirements.

ECSS-E-ST-20_0020117

<<deleted>>

ECSS-E-ST-20_0020118

<<deleted>>

ECSS-E-ST-20_0020119

Provision shall be made to prevent failure due to operation in shadow.

ECSS-E-ST-20_0020120

Solar array shall be designed in sections according to the redundancy principle specified at system level.

1	The number of sections can be equal to one or to the number of strings, depending on the redundancy principle.

2	Sectioning the SA can alleviate failure mitigation constraints.

ECSS-E-ST-20_0020121

Solar cells shall be protected against any deleterious reverse-bias conditions.

ECSS-E-ST-20_0020457

The conductive panels substrates shall be electrically insulated from each other and from the other structural parts of the solar array.

An examples of other structural parts is yoke.

ECSS-E-ST-20_0020458

For the purpose of insulation in solar arrays the following shall not be considered as a valid insulating layer:

Silicone based adhesives

Conformal coatings

1	Examples of silicon based adhesives are RTV-S 691, CV10-2568, RTV-S types, DC 93500. Examples of conformal coatings are Sulphur based compounds and silicone dioxide.

2	Issues of silicone based adhesives is the combination of their hygroscopic characteristics, the potential humidity absorbed on ground, the deposition layout and process, the severe thermal cycling and the radiation environment.

3	Silicone based adhesives can still be employed as insulation material for solar panel connectors. Good practice is to group the polarities and to separate them inside connectors as much as possible.

ECSS-E-ST-20_0020459

Insulation of solar array wires shall be provided with two different layers of materials or use of 2 individually cured layer of materials.

ECSS-E-ST-20_0020460

All individual electrical elements of the solar array shall be reliably insulated with respect to the ground of the spacecraft.

An example of an element of the solar array is the solar cell.

### Solar array power computation

ECSS-E-ST-20_0020122

Computation of solar array power shall be based on measurements at cell level performed in accordance with the requirements of clause 10 of ECSS-E-ST-20-08.

ECSS-E-ST-20_0020123

The model used for the computation of the I(V) curve of the solar cell shall be validated by test on the specific solar cell type for the mission in conditions representative of the expected domain of operation.

Representative conditions are, for example, illumination, temperature or SAA.

ECSS-E-ST-20_0020124

I(V) solar cells characteristics shall be computed in BOL and EOL conditions at maximum and minimum operating temperatures according to the mission profile.

ECSS-E-ST-20_0020125

The EOL solar cell I(V) curve shall be derived from measurements performed at the temperatures specified in 5.5.3c after irradiation with particles in conformance with the “Electron irradiation” and “Proton irradiation” tests for “Bare solar cells” specified in ECSS-E-ST-20-08 clause 7, and agreed with the customer.

The irradiated particles can either electrons or protons. The irradiation particles can be electrons or protons, chosen in order to facilitate the calculation of degradation, ideally using the particle type that dominates the degradation during the mission.

ECSS-E-ST-20_0020126

The forward voltage of the string blocking diode (if present) shall be computed:

using the worst-case voltage drop specified by the diode manufacturer,

at the diode operating temperature corresponding to the operational string current for each mission phase in worst case conditions.

ECSS-E-ST-20_0020127

The BOL worst and best case power calculations shall include the parameters indicated in Table 5-1.

ECSS-E-ST-20_0020128

For best case calculations, the string current shall account for the difference between the specified current and the average production value.

ECSS-E-ST-20_0020129

In addition with the parameters indicated in Table 5-1, the EOL worst and best case calculations shall include the parameters indicated in Table 5-2.

ECSS-E-ST-20_0020130

Shadowing and hot spot phenomena shall be analysed.

ECSS-E-ST-20_0020131

Leakage losses of bypass diodes shall be deducted from the power computation if they represent more than 0,1 % of the overall power to be provided.

ECSS-E-ST-20_0020132

Plume impingement effects shall be analysed.

ECSS-E-ST-20_0020381

Table 5-1: Parameters for BOL worst and best case power calculations

| Parameter | Applicable to string | Type of loss/gain |
| --- | --- | --- |
| Current Cell mismatch | Current | Random |
| Calibration error a | Current | Random |
| Cover glass gain / loss | Current | Direct |
| Blocking Diode Loss | Voltage | Direct |
| Harness Voltage Drop | Voltage | Direct |
| Pointing error due to disorientation and internal Solar Array error | Current | Direct |
| Orbital Losses & Sun Intensity b | Current & Voltage c | Direct |
| Shadow losses d | Current & Voltage | Direct |
| Temperature coefficient e | Current & Voltage | Direct |
| Temperature Gradient on String | Current & Voltage | Direct |
| a Typical value is ± 3%, including secondary working standard calibration and bare solar cell measurement accuracies, b Orbital losses as; EQX/SS, altitude, inclination, albedo, solar array angle including the cosine law deviation c E.g. High/Low Intensity interplanetary mission d E.g. Voltage losses due to cells and solar cell shunt diodes e For the average operational temperature on orbit ±5°C. |  |  |

ECSS-E-ST-20_0020382

Table 5-2: Additional power parameters for EOL worst and best case calculations.

| Parameter | Applicable to string | Type of loss/gain |
| --- | --- | --- |
| UV degradation a | Current | Direct |
| Micrometeorites b | Current | Direct |
| ”Loss of strings” tolerance | Current | Direct |
| Reliability of components and interconnection | Current & Voltage | Random |
| Degradation due to ESD Phenomena | Current & Voltage | Random |
| Solar array surface contamination | Current | Direct |
| Radiation c | Current & Voltage | Direct |
| a Maximum loss is solar cell assembly dependent and are determined by testing. The degradation follows an exponential decay with main part of the degradation in the first year. b Depending of in-orbit available data for each type of cell. c See ECSS-E-ST-10-04, clause 9.2. |  |  |

### Solar array drive mechanisms

ECSS-E-ST-20_0020133

The qualified de-rated current capability of wires, connector pins and slip ring contacts shall be greater than the best case BOL solar array section current in short circuit and include the effects of transient currents caused by the discharge of the solar array section capacitance.

ECSS-E-ST-20_0020134

The design of the insulation barriers between adjacent wires, connector pins and slip rings shall be such that no discharge phenomena can occur.

ECSS-E-ST-20_0020135

Where non-insulated conductors are used, arcing phenomena shall be prevented by design.

## Electrochemical Energy Storage

### Applicability

For the purpose of this clause, a battery is defined as a device that converts the chemical energy contained in its active materials into electric energy by means of electrochemical oxidation-reduction (redox) reaction.

It is made-up of one or more electrochemical cells, which can be grouped in modules permanently connected in series or parallel.

Clauses 5.6.2 to 5.6.5 apply to primary and secondary batteries where reference is not made to charge. Clause 5.6.5 defines additional safety requirements for all battery types.

Fuel cells and super capacitors are not addressed by the present standard.

### Batteries

ECSS-E-ST-20_0020136

The battery shall be specified to ensure the energy balance in each mission phase during operational life, including contingency modes resulting from a single failure for unmanned missions and two failures for manned missions.

ECSS-E-ST-20_0020137

<<deleted>>

ECSS-E-ST-20_0020138

Specific measures shall be taken in the battery design to keep under control the series inductance and the magnetic moment.

ECSS-E-ST-20_0020139

<<deleted>>

ECSS-E-ST-20_0020140

Batteries having to tolerate a single fault shall be designed such that they can operate with one cell either failed shorted or open circuit.

ECSS-E-ST-20_0020141

In batteries having to tolerate a single fault and where the effects of a single cell failure are mitigated by the use of a cell bypass device, the following shall be met:

The probability of the bypass circuit untimely operation is lower than the probability of a failure of the cell.

If the bypass operation is not instantaneous, the power subsystem design is able to operate without damage during the transient situation.

The maximum number of cells that can be bypassed after a failure or a wrong command is equal to the number of failures allowed by the specific mission design.

ECSS-E-ST-20_0020142

Transient currents, occurring when two or more separate strings of series-connected cells are connected together in parallel, or when a cell fails in short-circuit within a battery composed of parallel strings, shall not result in exceeding the peak cell current rating.

ECSS-E-ST-20_0020143

Battery supplier shall:

specify cell to cell performance variations so that mission requirements can be met,

provide methodology and validation of cell performance variations for any spare or flight models.

Cells making–up a battery are selected (matched) in accordance with the cell manufacturer’s requirements. Sufficient extra matched spare cells are procured to allow for replacement of any cells damaged during integration of batteries. If cells are not individually replaceable, then appropriately matched cell groups/modules are available. It is good practice to specify the number of spare cells in the battery procurement documentation.

ECSS-E-ST-20_0020144

When batteries are discharged in parallel, this discharge shall not result in current and temperature exceeding the cell qualification limits.

This requirement is essentially applicable to primary Lithium batteries that have a positive voltage vs temperature coefficient.

ECSS-E-ST-20_0020145

Conducting cases of battery cells in a battery package shall be insulated from each other with reliable insulation.

See definition of “reliable insulation” in clause 3.2.

ECSS-E-ST-20_0020146

Provisions for interfacing the battery with the ground support equipment during pre-launch operations shall be made.

Such provisions can include:

signal lines for monitoring battery voltage,

signal lines for monitoring battery temperature,

capability to charge the battery,

capability to discharge the battery.

ECSS-E-ST-20_0020147

A logbook shall be maintained by the supplier for each flight battery starting with the first activation after battery assembly up to launch, describing chronologically all test sequences, summary of observations, identification of related computer–based records, malfunctions, and references to test procedures and storage conditions.

The logbook is used for the following purposes:

to ensure compliance with storage, handling and operational requirements before launch (e.g. maximum time allowed at upper temperature limits, correct scheduling of maintenance activities);

to allow verification of flight worthiness.

special care has to be paid to external current discharge paths during integration phases.

ECSS-E-ST-20_0020148

Battery and spacecraft thermal design shall ensure together that:

maximum and minimum qualification temperature of cell operation under intended cycling conditions are not exceeded;

maximum qualification temperature gradients between different parts of the same cell and between two cells in a battery are not exceeded.

ECSS-E-ST-20_0020149

<<deleted>>

ECSS-E-ST-20_0020410

Conductive cases of battery cells shall be reliably insulated from spacecraft structure, with an insulation between any cell and the spacecraft structure greater than 10 M, measured at 500 V DC.

See definition of “reliable insulation” in clause 3.2.

### Battery cell

ECSS-E-ST-20_0020150

Absolute maximum ratings of the cell, in term of temperature, voltage, charge and discharge current in continuous and peak condition, shall be defined.

ECSS-E-ST-20_0020151

The ability of a cell to meet mission lifetime requirements, where not covered by qualification life testing or previous in flight experience, shall be justified by the ground test data or by dedicated tests under representative conditions.

ECSS-E-ST-20_0020395

The ability of a cell to meet mission life time requirements may be verified by similarity with qualification life testing or previous in flight experience only in case of identical design and identical manufacturing processes.

ECSS-E-ST-20_0020153

For any intended cell operation under acceleration greater than 1 g, the supplier shall ensure that no effect upon both short term (e.g. capacity) performance and lifetime can prevent battery nominal operation.

ECSS-E-ST-20_0020154

<<deleted>>

ECSS-E-ST-20_0020155

<<deleted>>

ECSS-E-ST-20_0020156

The battery supplier shall inform the customer of any change in design, materials or process from cells which have experienced life testing or flight.

### Battery use and storage

ECSS-E-ST-20_0020157

The design of the spacecraft shall be such that modules and batteries can be removed and replaced at any time prior to launch without affecting the acceptance status of the rest of the spacecraft.

ECSS-E-ST-20_0020158

For the procurement of cells and batteries the manufacturer shall supply a user manual in conformance with Annex D.

ECSS-E-ST-20_0020396

Flight batteries should not be used for ground operations to prevent any possible damage and subsequent degradation of life performance.

ECSS-E-ST-20_0020160

If 5.6.4c is not met, the flight worthiness of the batteries shall be re-verified after these ground operations are completed, in time for a possible replacement.

Re-verification can be done e.g. by capacity measurements.

ECSS-E-ST-20_0020161

Any test equipment interfacing with the battery shall include an associated undervoltage, overvoltage, overcurrent and over-temperature activated insulation switch.

ECSS-E-ST-20_0020162

<<deleted>>

ECSS-E-ST-20_0020397

<<deleted>>

### Battery safety

#### Overview

Almost all battery technologies used aboard spacecraft can be hazardous if not properly managed. Most are capable of delivering very high currents when shorted. When abused, cells can develop excessive internal pressure and eventually vent their contents, in extreme cases explosively. The electrolyte, cell reactants, and/or reaction products expelled can be corrosive (e.g. alkaline cells, lithium-SO2, Lithium SOCl2), flammable (e.g. lithium cell organic electrolytes) or toxic endangering any nearby personnel as well as neighbouring equipment. The principal cell failure modes, which can lead to these effects, are listed in 5.6.5.2b.

Detailed descriptions of the hazards associated with different battery chemistry are given in reference document: Crew vehicle battery safety requirements, JSC-20793 Rev B April 06.

The design rules in earlier clauses which aim at maximizing battery performance and cycle life also reduce the possibility that cells and batteries exhibit failure modes such as those listed above. However, in applying the safety rules of ECSS-Q-ST-40, some battery failure modes are critical or catastrophic. Further design or management provisions are implemented to achieve the required level of fault tolerance.

For safety requirements related to pressure vessels see ECSS-E-ST-32.

#### Provisions

ECSS-E-ST-20_0020164

The battery safety shall be managed in conformance with ECSS-Q-ST-40.

ECSS-E-ST-20_0020165

The design of the battery and associated monitoring and control electronics shall preclude the occurrence of any of the following:

Over-temperature (from battery thermal dissipation or environmental heating);

excessive currents (discharge or charge) including short–circuit (external or internal to the battery);

overcharging;

Attempt to charge in the case of primary cells;

over discharge (including cell reversal);

cell leakage (gases or electrolyte).

ECSS-E-ST-20_0020166

Where 5.6.5.2b is not met, the design shall mitigate the damaging effects of any such failure mode

E.g. by containment of cell leakage at battery level.

ECSS-E-ST-20_0020398

The failure of one or more cells within a battery due to imbalance in the state of charge, temperature or other parameter between cells should be prevented by the battery control electronics.

ECSS-E-ST-20_0020399

When the battery has non-insulated, exposed cell terminals, the battery should be delivered with a red insulation cover to be removed before spacecraft closure and for flight.

ECSS-E-ST-20_0020400

Provision should be made not to change the thermal balance of the battery during charge and discharge operations with the cover notified in 5.6.5.2e.

## Power conditioning and control

### Applicability

The requirements in 5.7.2 and 5.7.3 apply to power subsystems, those in 5.7.4 and 5.7.5 apply both to power subsystems and payloads, and those in 5.7.6 apply to payloads.

### Spacecraft bus

ECSS-E-ST-20_0020170

No single failure shall result in the loss of the power subsystem capability to the extent that the minimum mission requirements, in any of its phases, cannot be fulfilled.

ECSS-E-ST-20_0020171

For manned missions, no double failure shall result in the loss of the power subsystem capability to the extent that the minimum mission requirements, in any of its phases, cannot be fulfilled.

ECSS-E-ST-20_0020172

The primary power bus voltage regulation control for a fully regulated bus shall be independent from any control external to the electrical power subsystem.

1	Main control features do not include parameter settings by the OBC.

2	Loss of MPPT control can result in bus overvoltage hence the requirement concerns also MPPT control for regulated bus.

ECSS-E-ST-20_0020173

The ultimate switching between main and redundant MPPT circuitry, in case of MPPT malfunction, shall be implemented in a way to avoid infinite reconfiguration loops.

1	Autonomous MPPT can be implemented with redundancy and 2 out of 3 majority voter.”

2	Ultimate decision to switch over from nominal to redundant MPPT can depend on command from ground.

ECSS-E-ST-20_0020174

No single failure in the spacecraft shall open or short a main electrical power bus or violate the specified over voltage or under voltage limit requirements.

This includes for example failure of wiring, connectors and relays.

ECSS-E-ST-20_0020175

The design shall ensure that under all conditions during the required lifetime, including operation in eclipse with one battery cell failure and one solar array string failed, the primary bus voltage remains within specified performances.

ECSS-E-ST-20_0020401

For fully regulated buses, the nominal bus voltage value should be standardized according to the following:

28 V for power up to 1,5 kW;

50 V for power up to 8 kW;

100 V and 120 V for higher power.

1	Bus voltage types are standardized in order to maximize the reuse of equipment.

2	The rationale for this requirement is the following:

It is in practice difficult to design output impedance below 10 milliohm without an unwanted effect of the intrinsic connections and components resistance. For the design of a bus with 10 milliohm output impedance such that a 50 % load modulation induces a 1 % voltage change maximum as per 5.7.2i.1 requirement:

0,5 P/U  0,01 < 0,01U which means P < U2/0,5

Thus for	U = 28 V, P < 1,57 kW	
		U = 50 V, P < 5 kW	
		U = 100 V, P < 20 kW

In practice, at 50 V for example, higher power has been used on telecom spacecraft buses, because the 1 % voltage change referred to a lower load change of 20 % to 30 % instead of 50 %.

ECSS-E-ST-20_0020177

A fully regulated bus shall keep its nominal value in steady state within ± 0,5 % of the bus voltage at the main regulation point.

ECSS-E-ST-20_0020178

With a fully regulated bus in nominal operation the bus voltage transients shall:

for load transients of up to 50 % of the nominal load not exceed 1 % of its nominal value.

for any source and load transients remain within 5 % of its nominal value.

Load transient encompasses change of load current and ON/OFF load switching.

ECSS-E-ST-20_0020402

Fuses should be avoided to maintain the quality of the bus.

The rationale for requirement 5.7.2h to 5.7.2j is the following:

In order to be advantageous over an unregulated scheme, a regulated bus ensures a good regulation quality at the regulation point, including when the various loads on the bus are changing. The regulated bus is designed to ensure that normal transients including interdomain are within 5% all included. Abnormal transients are more than twice the normal transients; the load is then designed to operate nominally in normal transients and sustain without damage abnormal transients.

ECSS-E-ST-20_0020180

In case of fuse blowing, the recovery from the fuse clearance shall not produce an overshoot of more than 10 % above the nominal bus value.

ECSS-E-ST-20_0020181

The model of the fuse and of the electrical network to be protected by the fuse, shall be validated by test with a representative set-up

ECSS-E-ST-20_0020182

A fully regulated bus shall have a nominal ripple voltage below 0,5 % peak-to-peak of the nominal bus voltage, measured at the regulation point with at least 1 MHz bandwidth.

ECSS-E-ST-20_0020183

A fully regulated bus shall have commutation voltage spikes in the time domain of less than 2 % peak-to-peak of the nominal bus voltage, measured at the regulation point with a 50 MHz minimum bandwidth.

ECSS-E-ST-20_0020184

At the point of regulation, the impedance mask of a fully regulated bus, operating with one source shall be below the impedance mask shown in Figure 5-1.

1	E.g. battery, solar array.

2	Rationale for the impedance mask:

It translates requirement 5.7.2i.1 of 1 % voltage change for 50 % load change in a domain of regulation up to 10 kHz bandwidth. In DC the integrator in the control loop is designed to ensure no static error, in higher frequency, between 10 kHz and 100 kHz it is likely that the inductance effect of the components and connections are seen and the impedance rise not always making feasible to respect the ideal impedance mask.

ECSS-E-ST-20_0020383

Figure 5-1: Output impedance mask (Ohm)

ECSS-E-ST-20_0020185

For unregulated buses, the following parameters shall be specified, analysed and tested:

maximum and minimum bus voltage guaranteed at payload level in all steady state and transients conditions;

maximum ripple in time domain, measured with at least 1 MHz bandwidth.

maximum spikes in the time domain superimposed on the bus voltage, measured with a 50 MHz minimum bandwidth.

impedance mask.

Rationale for the requirement: Also for an unregulated bus, it is important to identify the bus impedance mask to verify the compatibility between the power bus and the loads, as for instance the guaranteed voltage range at bus level including the effects of load variations.

ECSS-E-ST-20_0020186

During integration phase the power subsystem shall be able to start up from any of its power sources irrespective of the connection of the other power source.

ECSS-E-ST-20_0020187

In the case of an unexpected battery or battery simulator disconnection, the main power bus voltage shall remain below its maximum specified overvoltage requirement.

1	This requirement applies only to ground operations, it can be profitably fulfilled by the EGSE.

2	This requirement can also apply in flight whenever a battery disconnection can occur as a result of ground command or single failure.

ECSS-E-ST-20_0020188

The design shall ensure that a short circuit to ground or to the return line of a solar array section does not result in a failure of category 1 and 2 criticality.

The definition of criticalities can be found in ECSS-Q-ST-30 or ECSS-Q-ST-40.

ECSS-E-ST-20_0020411

Control of the battery by OBC shall be robust to OBC failure and the time needed to reconfigure.

1	This requirement concerns in particular battery bus.

2	Issues include switch over from main to redundant OBC control, lack of protection during reconfiguration, battery overvoltage or overcurrent, battery undervoltage, loss of attitude, solar array MPPT operation on battery bus.

3	Safety issues during integration and operations on ground including launch is to be covered by overall system design and EGSE.

### Battery Charge and Discharge Management

ECSS-E-ST-20_0020189

On-board battery chargers shall be designed to ensure charging of a battery discharged down to zero volts.

The possibility of recovery applies mainly to the capability of recharging the battery exposed to extreme discharge conditions.

ECSS-E-ST-20_0020190

<<deleted>>

ECSS-E-ST-20_0020191

The minimum energy reserve in the battery shall be enough to guarantee the mission and a safe recovery of the spacecraft under all conditions.

Take into account that the charge rate plays a major role in the effectiveness of battery recharge.

ECSS-E-ST-20_0020192

The charging technique shall be designed to ensure that the batteries are managed in accordance with the manufacturer recommendations provided in the design description, justification file and user’s manual.

To avoid over (or under) charge when taper charging is employed, the voltage limit above which taper charging begins can be adjusted as a function of temperature, ageing or other parameters, depending on the battery technology. In some missions, required lifetime can only be obtained if the taper charge limit is lowered during periods of no or little battery use.

ECSS-E-ST-20_0020193

<<deleted>>

ECSS-E-ST-20_0020194

<<deleted>>

ECSS-E-ST-20_0020195

<<deleted>>

ECSS-E-ST-20_0020196

The ultimate over charging/discharging protection circuitry shall be implemented by hardware and independent from any on board software.

ECSS-E-ST-20_0020197

Battery charge and discharge management shall be such that a single failure for unmanned missions and two failures for manned missions does not impair the lifetime of the energy storage system with respect to minimum or maximum voltage as well as maximum charge or maximum discharge current.

Such failure tolerance can be implemented at cell, battery or subsystem level.

### Bus under-voltage or over-voltage

ECSS-E-ST-20_0020198

For fuse protected busses the electrical subsystem shall be robust against any fuse blowing event occurring on the primary bus, even after one failure anywhere in the power subsystem.

1	For voltage drop resulting from fuse blowing, robustness does not imply meeting performance parameters but means survival without overstressing of the system during this event and the recovery from it, so that proper operations can resume (autonomously or not) after the power bus transient, with nominal performance.

2	Functional spacecraft outage is prevented by having all software and configuration data (e.g. RAM, registers) integrity guaranteed with 0V power bus voltage, on a duration twice the one of the voltage drop resulting from fuse blowing.

3	See also requirement 5.8.1h.

ECSS-E-ST-20_0020199

All non-essential loads shall be switched-off autonomously in the event of reaching the battery energy level that is able to maintain all essential loads for a time guaranteeing safe recovery.

ECSS-E-ST-20_0020200

The ultimate non-essential load disconnection circuit shall be implemented as a full hard-wired chain from sensor to actuator.

Ultimate disconnection refers to the circuit being the last to disconnect the battery as a function of its state-of-charge.

ECSS-E-ST-20_0020201

The ultimate non-essential load disconnection circuit shall be one failure tolerant if centralised.

ECSS-E-ST-20_0020202

The spacecraft design shall be such that in the event of an under-voltage condition on the bus, no failure is induced in the power subsystem or the loads during and when recovering from this under-voltage.

ECSS-E-ST-20_0020203

After recovery as mentioned in 5.7.4e the loads shall be as follows:

all essential loads be supplied nominally;

all non-essential loads be in a known configuration that cannot create damage to any part of the spacecraft.

### Power converters and regulators

ECSS-E-ST-20_0020204

For converters and regulators in closed loop control, the phase margin shall be at least 50 and the gain margin 6 dB for worst case end–of–life conditions with representative loading.

ECSS-E-ST-20_0020205

For converters and regulators of the power subsystem, requirement 5.7.5a shall apply after any single failure.

Examples are solar array regulators, battery chargers and dischargers.

ECSS-E-ST-20_0020206

The electrical zero–volt reference of isolated converters and regulators shall be isolated from the unit case by more than 10 k per converter.

Rationale for this requirement:

The value of 10 k is a compromise: to be very large in DC and low frequency to minimize ground loop currents and to be small for high frequencies above 5 MHz in order to minimize the volt-drop between references due to common mode currents.

ECSS-E-ST-20_0020207

The capacitance between the zero–volt reference of isolated converters and regulators and the unit case shall be less than 150 nF per converter.

1	Rationale for this requirement:

The value of 150 nF is a compromise such that for a given piece of equipment this value is sufficiently high to dominate all parasitic capacitances to unit case, and low enough such that if many equipment are connected to a bus, the sum of bypassing capacitors to unit case and thus to ground reference is not significantly biasing the insulation of the bus or bus return to ground.

2	The measurement of the common mode capacitance is made with positive and return lines of the power switching converters shorted.

3	The common mode capacitance accounts for both filtering and damping capacitors.

ECSS-E-ST-20_0020208

If a switching converter is externally synchronized, it shall deliver output voltages within specification for any increase or decrease of synchronizing frequency, intermediate amplitude of synchronizing signal, phase jumps, or loss and recovery of the signal.

ECSS-E-ST-20_0020209

An analysis at unit level shall be performed to verify that no single failure generates an increase of conducted emission exceeding specified limit by more than 6 dB.

ECSS-E-ST-20_0020210

If an increase of conducted emission exceeding specified limit by more than 6 dB is identified from the unit level analysis of 5.7.5f, then a system level analysis shall be conducted to ensure that compatibility is maintained.

Rationale for this requirement:

6 dB is the margin usually taken between unit and subsystem when building up the EMC compatibility at system level. It means that failed equipment uses that EMC margin but does not perturb further the system.

ECSS-E-ST-20_0020211

A switching converter shall be able to reach nominal operation when the nominal input voltage is applied with any slope that can be provided by the power source and its associated impedance, connected to the switching converter.

### Payload interaction

ECSS-E-ST-20_0020212

Inrush, under-voltage and a representative set of failures agreed with the customer for the payload interaction with the primary bus, shall be verified by test.

ECSS-E-ST-20_0020213

No load shall generate a spurious response that can damage itself or any other equipment during bus voltage variation, up or down, at any ramp rate, and over the full range from zero to maximum bus voltage.

ECSS-E-ST-20_0020214

All current limiting devices and automatic switch-off circuits shall be monitored by telemetry.

ECSS-E-ST-20_0020215

The failure of the monitoring function of 5.7.6c shall not cause the protection elements to fail.

## Power distribution and protection

### General

ECSS-E-ST-20_0020216

The primary power source shall be grounded to the spacecraft structure at the star reference point with a connection capable of sustaining the worst case fault current.

ECSS-E-ST-20_0020217

<<deleted, replaced by requirements 5.8.1q to 5.8.1v>>

ECSS-E-ST-20_0020218

<<deleted>>

ECSS-E-ST-20_0020219

All load paths shall include protection circuitry on the source side.

The aim is to locate them as near as possible to the source.

ECSS-E-ST-20_0020220

No load shall be permanently disconnected from its power source as a consequence of an SEE.

ECSS-E-ST-20_0020221

If fuses are used to protect main bus distribution lines, provision shall be made allowing easy replacement of blown or defective fuse.

Provision can consist in easy accessibility to fuses or in replacement of concerned unit by available spare one.

ECSS-E-ST-20_0020222

<<deleted>>

ECSS-E-ST-20_0020223

If fuses are used to protect main bus distribution lines, the design shall ensure that the power generation system can fuse them within less than 45 ms in case of load short circuit.

ECSS-E-ST-20_0020224

<<deleted>>

ECSS-E-ST-20_0020225

Equipment connected to independent, redundant power buses not protected at the source shall ensure that:

for unmanned missions, no single failure causes the loss of more than one power bus;

for manned missions, two failures do not cause the loss of more than one power bus.

ECSS-E-ST-20_0020226

The stability of current limiters shall be ensured for the actual loads characteristics.

ECSS-E-ST-20_0020227

<<deleted>>

ECSS-E-ST-20_0020228

<<deleted>>

ECSS-E-ST-20_0020229

In case the distribution lines are protected by latching, foldback or periodically reset current limiters, it shall be verified by analysis or test that the transient current peaks at current limiter intervention are within the rated stress limits of the components used, for the worst case condition (minimum series impedance case).

ECSS-E-ST-20_0020230

When protection elements are in cascade, the closest one upstream from the anomaly should be the first to act.

ECSS-E-ST-20_0020231

When protections are used in cascade from a power source to a function to be supplied, the compatibility of these protections shall be ensured.

ECSS-E-ST-20_0020412

Whenever two or more blocks are connected in cascade, the stability of the cascade between each source block and load block shall be analysed with the source and load impedances characterised in compliance with Figure 5-2.

ECSS-E-ST-20_0020413

Figure 5-2: Source and load impedance characterisation

ECSS-E-ST-20_0020414

Whenever two or more blocks are connected in cascade, the power source being conveniently modelled with a Thevenin equivalent in compliance with Figure 5-3 and equation 1 for the sake of interface voltage stability analysis, the following two conditions shall be met:

the difference between the phases of the source impedance and the load impedance is comprised in between [-130°,+130°] ±n*360° at those frequencies in which the load and the source impedance are equal in magnitude,

the ratio of the magnitudes of the source and the load impedance is smaller than a factor 0,5 at those frequencies in which the difference between the phase of the source impedance and the load impedance is equal to -180°±n*360°.

ECSS-E-ST-20_0020415

Figure 5-3: Thevenin equivalent model

| VLVS= 11+ZSZL | equation [1] |
| --- | --- |

ECSS-E-ST-20_0020416

In alternative to requirements 5.8.1q and 5.8.1r, assuming that a power source is modelled with a Thevenin equivalent, stability criterion given in Impedance Specifications for Stable DC Distributed Power Systems, EEE transactions on power electronics, Vol. 17, no. 2, March 2002 shall be applied.

ECSS-E-ST-20_0020417

In alternative to, and under the same assumptions of requirement 5.8.1r, the magnitude of the source impedance shall be smaller than the magnitude of the load impedance by at least a factor 10.

ECSS-E-ST-20_0020418

Whenever two or more blocks are connected in cascade, the power source being conveniently modelled with a Norton equivalent in compliance with Figure 5-4 and equation 2 for the sake of interface current stability analysis, the following two conditions shall be met:

the difference between the phases of the load impedance phase and the source impedance is comprised in between [-130°,+130°] ±n*360° at those frequencies in which the load and the source impedance are equal in magnitude,

the ratio between the magnitudes of the load and the source impedance is smaller than a factor 0,5 at those frequencies in which the difference between the load impedance phase and the source impedance phase is equal to -180°±n*360°.

ECSS-E-ST-20_0020419

Figure 5-4: Norton equivalent model

| ILIS= 11+ZLZS | equation [2] |
| --- | --- |

ECSS-E-ST-20_0020420

In alternative to, and under the same assumptions of requirement 5.8.1u, the magnitude of the load impedance shall be smaller than the magnitude of the source impedance by at least a factor 10.

1	The requirements 5.8.1r to 5.8.1v can be used as alternative by the user for verification purposes (only one is used among 5.8.1r to 5.8.1t, or between 5.8.1u and 5.8.1v).

2	The diagrams for the verification of requirements 5.8.1r to 5.8.1v are provided in Figure 5-2.

3	The requirements 5.8.1r to 5.8.1v can be used for evaluating the small signal stability for systems that are linear or can be linearised around an operating point.

4	n is a positive integer including 0.

ECSS-E-ST-20_0020421

The stability of current limiters shall be verified by analysis under worst case conditions, and tested under a set of cases agreed with the customer.

ECSS-E-ST-20_0020422

The requirement 5.8.1k shall be verified by worst case analysis, in accordance with ECSS-Q-ST-30 Annex J, and test.

### Harness

ECSS-E-ST-20_0020232

No piece of harness shall be used to transfer mechanical loads.

ECSS-E-ST-20_0020233

With the exception of the solar array, routing of power lines shall be near ground.

ECSS-E-ST-20_0020234

With the exception of the solar array and electrical bus bars, harness power lines shall be such that each line is twisted with its return, when the structure is not used as a return.

The purpose of the requirements b and c is to minimize current loop area and harness inductance.

ECSS-E-ST-20_0020235

The power distribution shall be protected in such a way that no over-current in a distribution wire can propagate a thermal failure to another wire.

ECSS-E-ST-20_0020236

The harness inductance for a fully regulated bus, from the distribution node of the regulated bus to the load, shall be such that the break frequency is at least 5 000 Hz.

1	That means that:

L < R/2 f

where:

L	harness inductance in H

R	harness resistance in 

f	break frequency in Hz, i.e. f = 5 000.

2	Rationale for this requirement

This ties-up with the impedance mask requirement, because beyond the break frequency, the impedance is going to rise and one wants to keep the quality established on the regulation point with the impedance mask as best as possible and as far as possible to the loads.

ECSS-E-ST-20_0020237

Harness shall be tested up to connector brackets under 500 V DC between conductors, conductors and structure, conductors and shielding.

500 V DC is selected in order to detect insulation defects potentially induced by air voltage breakdown.

ECSS-E-ST-20_0020238

The harness restraining systems on the structure shall not bring about any stress at connector level.

ECSS-E-ST-20_0020239

There shall be umbilical and test connectors to provide external electrical interfaces.

1	E.g. with the launcher and with the EGSE.

2	Functions provided include all those necessary for supporting AIT and launch site activities (e.g. monitor spacecraft operation, maintain synchronization between spacecraft, EGSE and real time simulators, put the spacecraft in a defined operation scenario like a quick upload of SW).

ECSS-E-ST-20_0020240

Electrical and Safe and arm plugs shall be provided for disabling on ground hazard functions.

For harness design and manufacturing guidelines and handbook, see RNC-CNES-Q-70-511 and NASA-STD-8739.4.

ECSS-E-ST-20_0020241

If cross-strapping of redundant paths and circuits is carried out in the harness, then provisions of ECSS-E-ST-50-14 clause 4.2.5.2 shall apply.

## Safety

ECSS-E-ST-20_0020242

The design of electrical subsystems and payloads shall conform to ECSS-Q-ST-40.

## High voltage engineering

ECSS-E-ST-20_0020243

For non pressurised and non potted high voltage equipment, the applicable pressure range when this equipment is on shall be specified.

ECSS-E-ST-20_0020244

Non pressurised and non potted high voltage equipment shall be designed and manufactured to avoid discharge phenomena according to Paschen curves valid for its specified pressure range.

ECSS-E-HB-20-05 provides useful directions on this aspect and in general to high voltage engineering.

ECSS-E-ST-20_0020245

The field enhancement factors shall be ensured by the design.

This applies in particular to the routing of high voltage cables.

ECSS-E-ST-20_0020246

For potted circuits, the glass transition point of the potting material shall be outside the temperature range of qualification.

ECSS-E-ST-20_0020247

The design of high voltage equipment shall be such that worst case DC and AC field strengths are less than half of the values for which breakdown can occur.

## Verification

### Provisions

ECSS-E-ST-20_0020248

The requirements of this Clause 5 should be verified by the verification methods and at the verification points listed in Table 8-3.

1	Table 8-3 can be used as a starting point for the definition of the verification methods.

2	For more details on verification, see also ECSS-E-ST-10-02, in particular requirements 5.2.1c, 5.2.1d and 5.2.1e.

### <<deleted>>

ECSS-E-ST-20_0020249

<<deleted>>

ECSS-E-ST-20_0020250

<<deleted>>

ECSS-E-ST-20_0020251

<<deleted>>

ECSS-E-ST-20_0020384

Table 5-3: <<deleted, merged with new Table 8-3>>

# Electromagnetic compatibility (EMC)

## Overview

The objective of the following EMC requirements is to ensure that the space system is designed to achieve electromagnetic compatibility (EMC) between all equipment and subsystems within the space system and in the presence of its self–induced and external electromagnetic environment.

## Policy

### Overall EMC programme

ECSS-E-ST-20_0020252

The supplier shall establish an overall EMC programme.

1	The EMC programme is an activity the purpose of which is to provide for spacecraft-level compatibility with the minimum impact to programme cost, schedule and operational capabilities. The role of the customer in the EMC programme is that of top-level oversight.

2	The EMC programme is based on requirements of this standard, the statement of work, spacecraft specification, and other applicable contractual documents.

ECSS-E-ST-20_0020253

The EMC programme shall:

plan and verify that EMC technical criteria, mainly design and management controls are in place to achieve EMC;

plan and accomplish the verification of spacecraft–level EMC.

### EMC control plan

ECSS-E-ST-20_0020254

As part of the EMC programme, an EMC control plan shall be written by the supplier for the PDR in conformance with the DRD in Annex A.

The Control plan initial release documents the procedures of the EMC programme including basic design guidelines, while subsequent routine updates document the programme progress.

ECSS-E-ST-20_0020255

The EMC control plan shall apply to every item of equipment and subsystem in the project.

ECSS-E-ST-20_0020423

An EMC control plan shall be produced for every subsystem and equipment in answer to the requirements applicable at its level.

### Electromagnetic compatibility advisory board (EMCAB)

ECSS-E-ST-20_0020256

For such programmes where EMC has been identified during phase A as critical for mission performance, the EMC programme shall include an EMC Advisory Board (EMCAB).

ECSS-E-ST-20_0020257

The EMCAB shall:

Ensure the timely and effective execution of the EMC programme under the general project manager.

Respond to the problems related to EMC as they arise.

ECSS-E-ST-20_0020258

The supplier shall chair the EMCAB, with customer oversight.

1	The EMCAB members are representatives of the Spacecraft Supplier and payload suppliers and users.

2	EMCAB members can invite associate suppliers or independent experts.

3	The EMCAB accomplishes its duties and document its activities mainly through the use of the system-level EMC documentation.

## System level

### Electromagnetic interference safety margin (EMISM)

#### Circuits categories

ECSS-E-ST-20_0020259

Functional criticality of circuits for all equipment/subsystem circuits shall be identified in accordance with the following categories:

Safety critical circuit - EMI problems that can result in loss of life or loss of space platform. This category comprises electro-explosive devices and their circuits.

Mission critical circuit - EMI problems that can results in injury, damage to space platform, mission abort or delay, or performance degradation which unacceptably reduces mission effectiveness.

Non critical circuit – Any problems that do not belong to categories 6.3.1.1a.1 and 6.3.1.1a.2.

#### Critical points

ECSS-E-ST-20_0020260

The list of points where the margin is demonstrated (critical points) shall be submitted to the customer for approval.

#### Margins

ECSS-E-ST-20_0020261

Electromagnetic interference safety margins shall be determined at critical points under all operating conditions.

ECSS-E-ST-20_0020262

The minimum margins shall be 20 dB for safety critical circuits, and 6 dB for mission critical circuits.

### Inter-element EMC and EMC with environment

#### Overview

The objectives of the following requirements are to ensure that the space system operates without performance degradation in the electromagnetic environment due to external sources (natural sources and man-made sources, intentional or not).

#### EMC with the launch system

ECSS-E-ST-20_0020263

The electromagnetic environment seen by the spacecraft and the EMC requirements during the pre-launch and launch phases shall be according to those described in the applicable launchers user's manuals.

Specific EMC requirements during the pre-launch and launch phase are described in an Interface Control document established on a contractual basis between the launching company and the customer.

#### Protected frequency bands

ECSS-E-ST-20_0020264

For protection of radiometric and communication bands, requirements on “Emissions” of “Transmitted signals” in ECSS-E-ST-50-05 clause 5.5 shall apply.

#### Lightning

ECSS-E-ST-20_0020265

The space system shall be protected against both direct and indirect effects of lightning such that the mission is without degradation of performances after exposure to the lightning environment.

### Hazards of electromagnetic radiation

ECSS-E-ST-20_0020266

The space system shall be designed so that humans, fuels, explosive systems, and electronically actuated thrusters are not exposed to hazards of electromagnetic radiation present in the entire electromagnetic environment, including interference sources from possible external transmitters.

### Spacecraft charging protection program

#### Applicability

ECSS-E-ST-20_0020267

A spacecraft charging protection programme shall be produced by the supplier for the PDR, and submitted to the customer for approval, in conformance with ECSS-E-ST-20-06 clause 5 and Annex A.

#### General

ECSS-E-ST-20_0020268

The spacecraft charging protection programme shall include the preparation and maintenance of an analysis plan, and the preparation and maintenance of a test plan.

The objective of the programme is to ensure that the space vehicle is capable of operating in the specified space plasma charging environment and its energetic electron content without degradation of the specified space vehicle capability and reliability and without changes in operational modes, location, or orientation.

ECSS-E-ST-20_0020269

The performance shall be accomplished without the intervention of external control such as commands from a ground station.

ECSS-E-ST-20_0020270

The spacecraft charging protection programme shall include:

surface electrostatic charging,

threat from internal electrostatic charging of dielectric materials and isolated conducting items, due to the penetration of energetic electrons as defined in the environmental specification.

ECSS-E-ST-20-06 is intended to provide clear and consistent requirements to the application of measures to assess and mitigate hazardous effects arising from spacecraft charging and other environmental effects on a spacecraft’s electrical behaviour.

#### Performance

ECSS-E-ST-20_0020403

The space vehicle electrical subsystem and system may undergo an outage during an arc discharge if operation and performance returns to specified levels within

a telemetry main frame period after onset of the discharge, or

within some other period defined by the customer.

ECSS-E-ST-20_0020404

<<deleted>>

ECSS-E-ST-20_0020273

Occurrence of an arc discharge during transmission of a command to the space vehicle from an external source as a ground station shall not result in any unintended action, whether the command is executed or not.

An external source can be a ground station.

ECSS-E-ST-20_0020274

Provision shall be made such that the space vehicle is capable of receiving and executing subsequent commands.

ECSS-E-ST-20_0020275

Provision shall be made such that the space vehicle meets specified performances within the time period defined in clause 6.3.4.3a.

### Intrasystem EMC

ECSS-E-ST-20_0020276

The space system shall operate without performance degradation in the electromagnetic environment due to on-board sources, intentional or not.

### Radio frequency compatibility

ECSS-E-ST-20_0020277

The spacecraft shall be RF compatible with all antenna-connected equipments and subsystems, the compatibility criteria being based on the mission performance and operability requirements.

ECSS-E-ST-20_0020278

When an inter-system interface is required, each system shall be RF compatible with all antenna-connected equipments and subsystems, the compatibility criteria being based on the mission performance and operability requirements.

ECSS-E-ST-20_0020279

The RF compatibility analysis, if used instead of test, shall include the effects of inter-modulation products.

### Spacecraft DC magnetic field emission

#### Overview

DC magnetic emissions have impacts on two main areas, magnetic sensors of payloads and the attitude control system (ACS). Other specific components are susceptible (ultra-stable crystal oscillators, plasma monitors, high-permeability magnetic shields).

#### Spacecraft with susceptible payload

ECSS-E-ST-20_0020280

In case the payload involves equipments sensitive to DC H-Field, the maximum acceptable DC magnetic field at their location from the rest of the spacecraft shall be specified by the customer because of the mission performance requirements.

It is the role of the EMCAB to translate the customer’s DC magnetic field requirements, specified at the sensitive payload location, into subsystem and equipment magnetic requirements (magnetic field or magnetic moment limits, test methods).

#### Attitude control subsystem

ECSS-E-ST-20_0020281

On the basis of the attitude control requirements, the supplier shall derive magnetic requirements for the spacecraft so as to limit transient, diurnal and secular torques.

ECSS-E-ST-20_0020282

If magnetometers are used as part of the Spacecraft Attitude Control Subsystem, the maximum acceptable DC magnetic field at their location from the rest of the spacecraft shall be specified by the supplier because of the attitude control subsystem requirements and submitted to the customer approval.

### Design provisions for EMC control

#### Electrical bonding

ECSS-E-ST-20_0020283

The electrical bonding shall be in conformance with the requirements specified in clauses 4.2.11 and 5.3.10 of ECSS-E-ST-20-07.

#### Grounding

ECSS-E-ST-20_0020284

A controlled ground reference concept, including the definition of circuit and unit categories shall be specified and agreed with the customer for the spacecraft prior to initial release of the EMC control plan.

#### Wiring

ECSS-E-ST-20_0020285

Classification of cables, and cables shield shall be in conformance with the requirements specified in clauses 4.2.13 and 5.3.11 of ECSS-E-ST-20-07.

### Detailed design requirements

ECSS-E-ST-20_0020286

The EMC system design shall be performed in conformance with the requirements specified in clause 4.2 of ECSS-E-ST-20-07.

## Verification

### Verification plan and report

ECSS-E-ST-20_0020287

The verification plan shall be accomplished by the supplier in the frame of the EMC programme.

ECSS-E-ST-20_0020288

The verification plan shall be documented in the electromagnetic effects verification plan (EMEVP) in conformance with the DRDs in Annex B.

ECSS-E-ST-20_0020289

An electromagnetic effects verification report (EMEVR) in conformance with the DRD in Annex C shall be prepared by the supplier.

### Safety margin demonstration for critical or EED circuit

ECSS-E-ST-20_0020290

Safety margins for critical or EED circuit shall be demonstrated at system–level.

ECSS-E-ST-20_0020291

If the demonstration of safety margins is done by test, the spacecraft suite of equipment and subsystems shall be operated in a manner simulating actual operations, agreed with the customer.

### Detailed verification requirements

ECSS-E-ST-20_0020292

EMC verification shall be performed in conformance with the requirements on “Verification” in specified in ECSS-E-ST-20-07.

# Radio frequency systems

## Functional description

Radio frequency (RF) systems include transmitters, receivers, antennas and their associated transmission lines (waveguides) including connectors, operating typically in the range from 30 MHz to 300 GHz. The transmitted or received signals can be narrowband or wideband, often with complex modulation and sometimes with multiple carriers. Transmitters and receivers require high mutual insulation and antennas can interact strongly with the spacecraft.

For achieving the RF performance requirements, the following parameters are considered by the engineering process:

antenna field of view and polarization;

link or radiometric budget;

spatial and spectral resolution;

signal to noise ratio;

frequency plan.

For achieving the performances requirement, the following parameters are considered by the RF design and development:

transmitter power;

receiver sensitivity;

active and passive intermodulation products;

multipaction;

corona;

spectral purity;

VSWR;

frequency stability;

reflection and diffraction effects on antenna performance;

mutual coupling between antennas;

insulation between transmitter and receiver;

EIRP.

## Antennas

### General

#### Overview

As specified in ECSS-E-ST-10, budgets and margins are established and requested during Project phase B, and reviewed in all subsequent phases of the project.

#### Provisions

##### Definition of terms in the documentation

ECSS-E-ST-20_0020293

All antenna terms used in all documentation (DDF, DJF, Test Report, Test Procedures, ICD and EIDP) shall follow the definitions found in IEEE 145:1993 ”Antenna Terms”.

##### Engineering process

ECSS-E-ST-20_0020294

The following engineering process shall be applied:

Perform an analysis of the mission requirements for RF signal transmission and reception for all systems and payload for all phases of the mission.

Perform electrical, mechanical and thermal computer assessments to identify feasibility and performance margin for the whole antenna farm

Establish performance budgets, including losses, simulation/measurement error and technology maturity margins for the whole antenna farm.

Establish prediction, measurement and operational error/accuracy budgets for the whole antenna farm.

Establish a plan for the maintenance and periodical review of the budgets established in requirement 7.2.1.2.2a.3 and 7.2.1.2.2a.4 during all project phases.

to item 4: E.g. Pointing, excitation, phase centre.

#### Failure containment and redundancy

ECSS-E-ST-20_0020295

Antennas are in general single point failure elements; therefore their failure rates shall be agreed with the customer, specified and demonstrated.

To improve the failure rate, special precautions in the redundancy architecture are commonly taken to cover the failures of active elements.

### Antenna structure

#### General

ECSS-E-ST-20_0020296

The antenna category (7.2.2.2), composing elements (7.2.2.2.4), used technologies (7.2.2.4) and the performance parameters (7.2.2.5) shall be established at the beginning of the project phase B.

#### Categories

##### TT&C and data transmission

ECSS-E-ST-20_0020297

The antenna radiation pattern shall be characterised including the scattering effects of all surrounding structures.

TT&C and data transmission antennas are in general compact antennas (individual radiating elements - 7.2.2.3.1) with broad radiation patterns and a single beam. In some cases (e.g. deep space missions), more complex antennas falling into one of the other categories are used.

ECSS-E-ST-20_0020298

If a number of TT&C antennas operate simultaneously, the combined radiation pattern shall be used in the performance evaluation.

##### Reflector/Lens antennas

ECSS-E-ST-20_0020299

The reflection and transmission properties (losses, depolarisation and diffusivity) of the reflecting or transmitting elements shall be quantified and their impact on antenna performances assessed.

Reflector/Lens antennas are constituted by one or more radiating elements (7.2.2.3.1), possibly including an antenna RF chain (7.2.2.3.5), one or more (partially) reflecting or transmitting elements (reflectors - 7.2.2.3.2, lenses - 7.2.2.3.3) and an antenna support structure (in one or more portions- 7.2.2.3.6). If several radiating elements are present, also a Beam Forming Network can be present to distribute the RF signal (7.2.2.3.4).

ECSS-E-ST-20_0020300

The effects of antenna support structures shall be quantified and the impact on antenna performances assessed.

ECSS-E-ST-20_0020301

Deformations of reflector antennas, which parts are physically attached to different portions of the spacecraft platform, shall be quantified and their impact on antenna performance assessed.

For large reflector antennas that use hold-down and release, deployment mechanisms as well as pointing devices, ECSS-E-ST-33-11 can be applied.

##### Array antennas

ECSS-E-ST-20_0020302

The effect of the radiation of individual array element on the others shall be quantified and the impact on antenna performances assessed.

Array antennas are constituted by a number of radiating elements (7.2.2.3.1), possibly including an antenna RF chain (7.2.2.3.5) and arranged in a more or less regular layout. The RF signals are routed to/from each element through a wave-guiding network generally known as Beam Forming Network (7.2.2.3.4). An antenna support structure can also be present 7.2.2.3.6.

ECSS-E-ST-20_0020303

The effects of antenna support structures on the main RF wave propagation path shall be quantified and the impact on performance assessed.

ECSS-E-ST-20_0020304

Deformations of array antennas, which parts are physically attached to different portions of the spacecraft platform, shall be quantified ant their impact on antenna performance assessed.

For large array antennas that use hold-down and release, deployment mechanisms as well as pointing devices, ECSS-E-ST-33-11 can be applied.

##### Array-fed reflector antennas

ECSS-E-ST-20_0020305

For array-fed reflector antennas clauses 7.2.2.2.2 (Reflector/Lens antennas) and 7.2.2.2.3 (Array antennas) shall apply.

#### Elements

##### Radiating elements

ECSS-E-ST-20_0020306

The isolated performances of radiating elements shall be characterised as part of the performance prediction of the whole antenna, at least up to the end of Phase B.

Individual radiating elements are a key element to the overall antenna performances. They can be completed by a chain of RF components (see antenna RF chain 7.2.2.3.5), to ensure a suitable RF interface.

ECSS-E-ST-20_0020307

Whenever an antenna RF chain is attached to the radiating element its impact on the radiating element performances shall be assessed.

ECSS-E-ST-20_0020308

Deviations from the nominal geometry of the radiating element shall be quantified and their impact on antenna performances assessed.

Typical deviations are due to manufacturing errors, thermo-elastic effects and modification of the material characteristic in the orbit environment, moisture release in composites.

ECSS-E-ST-20_0020309

It shall be demonstrated that the scattering of the radiation pattern of individual radiating elements does not affect the accuracy of all radiated performance measurement.

ECSS-E-ST-20_0020310

Thermal dissipation of RF power shall be quantified and the impact on antenna performances assessed.

ECSS-E-ST-20_0020311

Whenever a radiating element is used to route high power levels,

The applicable pressure range and gas properties shall be specified.

The design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties.

See clause 7.3 for further details.

ECSS-E-ST-20_0020312

All metallic parts in a radiating element shall be connected to the equipment DC ground to avoid electrostatic discharge (ESD).

##### RF Reflectors

ECSS-E-ST-20_0020313

Reflective properties (losses, depolarisation, and diffusivity) of the materials and composites used shall be quantified and their impact on antenna performances assessed.

ECSS-E-ST-20_0020314

The reflective and transmissive properties (losses, depolarisation, diffusivity) of the materials and composites used for polarisation and frequency selective reflectors shall be quantified and their impact on antenna performances assessed.

ECSS-E-ST-20_0020315

Deviations from the nominal geometry of the reflector shall be quantified and their impact on antenna performances assessed.

1	Reflectors can require hold-down and release, deployment as well as pointing devices. ECSS-E-ST-33-11 and ECSS-Q-ST-70 are relevant and applicable in this case.

2	Typical deviations are due to manufacturing errors, thermo-elastic effects and modification of the material characteristic in the orbit environment, moisture release in composites.

##### RF Lenses

ECSS-E-ST-20_0020316

Reflective and transmissive properties of the materials and composites used for the lenses shall be quantified and their impact on antenna performances assessed.

Examples of reflective and transmissive properties are losses, depolarization and diffusivity.

ECSS-E-ST-20_0020317

Deviations from the nominal geometry of the lens shall be quantified and their impact on antenna performances assessed.

Typical deviations are due to manufacturing errors, thermo-elastic effects and modification of the material characteristic in the orbit environment, moisture release in composites.

ECSS-E-ST-20_0020318

Measures to drain accumulated electric charges from all non conductive parts shall be implemented to avoid electrostatic discharge (ESD).

ECSS-E-ST-20_0020319

Any metallic parts shall be connected to the equipment DC ground to avoid electrostatic discharge (ESD).

##### RF Beam Forming Network

ECSS-E-ST-20_0020320

The circuit characteristics of the RF BFN shall be independently quantified and their impact on antenna performances assessed at least up to CDR.

ECSS-E-ST-20_0020321

Deviations from the nominal geometry of the RF BFN shall be quantified and their impact on antenna performances assessed.

Typical deviations are due to manufacturing errors, thermo-elastic effects and modification of the material characteristic in the orbit environment, moisture release in composites.

ECSS-E-ST-20_0020322

In all RF BFN structures having a central conductor (ideally insulated), the thermal power generated by Joule effect on the conductor itself shall be quantified and its impact on antenna performances assessed.

ECSS-E-ST-20_0020323

For RF BFN, the applicable pressure range and gas properties shall be specified.

ECSS-E-ST-20_0020324

For RF BFN, the design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties.

See clause 7.3 for further details.

##### Antenna RF chain

ECSS-E-ST-20_0020325

The circuit characteristics of the antenna RF chain shall be independently quantified and their impact on antenna performances assessed at least up to CDR.

ECSS-E-ST-20_0020326

The cumulative effects of wave propagation discontinuities along the whole antenna RF chain, including the radiating elements attached to it, shall be quantified and the impact on antenna performances assessed.

ECSS-E-ST-20_0020327

For antenna RF chain the applicable pressure range and gas properties shall be specified.

ECSS-E-ST-20_0020328

For antenna RF chain the design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties.

See clause 7.3 for further details.

##### Antenna support structures

ECSS-E-ST-20_0020329

The possible scattering effects of the support structures shall be quantified and their impact on the antenna performances assessed.

ECSS-E-ST-20_0020330

Deviations from the nominal geometry of the supporting structure shall be quantified and their impact on antenna performances assessed.

Typical deviations are due to manufacturing errors, thermo-elastic effects and modification of the material characteristic in the orbit environment, moisture release in composites.

#### Technologies

##### Metal based

ECSS-E-ST-20_0020331

The level of passive inter-modulation products generated by the antenna shall be quantified and their impact on antenna performances assessed.

1	See clause 7.4 for further details.

2	Ferro-magnetic materials and metal-to-metal junctions are the most common non-linear elements in antennas.

ECSS-E-ST-20_0020332

The impact of thermally-induced effects on the generation of passive intermodulation products shall be quantified and the impact on antenna performances assessed.

A typical example of thermally induced effects triggering the generation of PIM is the sudden releases of stresses in metal-to-metal joints due to temperature variations.

ECSS-E-ST-20_0020333

Thermally induced changes of dimension and shape in all metallic antenna parts shall be quantified and their impact on antenna performances assessed.

##### Composite based

ECSS-E-ST-20_0020334

The impact of surface characteristics and finish on antenna performances shall be assessed.

1	In particular this is essential for the RF conductive surfaces of the component.

2	Electrical conductivity and depolarisation properties are the most typical parameters affected.

ECSS-E-ST-20_0020335

Thermally induced changes of dimension and shape in all composite and combined metal-composite antenna parts shall be quantified and their impact on antenna performances assessed.

ECSS-E-ST-20_0020336

Measures to drain accumulated electric charges from composite parts shall be implemented to avoid electrostatic discharge (ESD).

##### Plastic based

ECSS-E-ST-20_0020337

The dielectric losses of plastic component in the RF power path shall be quantified and their impact on antenna performances assessed.

Components made from homogeneous plastic are usually limited to small parts (e.g. spacers or washers).

ECSS-E-ST-20_0020338

Thermally induced changes of dimension and shape in all plastic and combined metal-plastic antenna parts shall be quantified and their impact on antenna performances assessed.

ECSS-E-ST-20_0020339

Measures to drain accumulated electric charges from all plastic parts shall be implemented to avoid electrostatic discharge (ESD).

#### Performance parameters

ECSS-E-ST-20_0020340

The characterisation of antenna performances shall cover the following parameters.

Coverage or Beam shape;

Directivity;

Electrical boresight or Beam pointing;

Gain or Beam efficiency;

Input impedance mismatch factor;

Radiation pattern;

Sense of polarization;

Side lobe level;

Polarisation purity or Axial ratio;

Group delay;

Noise temperature, for receive antennas;

Phase centre position;

Variations with frequency, angle (where applicable) and aging of all above parameters.

### Antenna interfaces

#### Guided-wave interfaces

ECSS-E-ST-20_0020341

Connectors or waveguide flanges at the antenna ports shall be demonstrated to have the specified power handling capabilities and impedance mismatch factors.

Antenna RF ports are realised using a wave-guiding structure (coaxial cable or waveguide, in most instances). Connectors or flanges are used to realise the physical interface.

ECSS-E-ST-20_0020342

It shall be demonstrated that the generation of passive inter-modulation products that can occur at the antenna ports is below the specified limits agreed with the customer.

ECSS-E-ST-20_0020343

For antenna ports the applicable pressure range and gas properties shall be specified.

ECSS-E-ST-20_0020344

For antenna ports the design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties.

See clauses 7.3 and 7.4 for further details.

#### Radiative interfaces

ECSS-E-ST-20_0020345

Electromagnetic interactions among the antenna and the surrounding spacecraft structure and appendages shall be quantified starting from Phase B, as a minimum, and their impact on antenna performances assessed.

The field radiated or received by the antenna interacts with the surrounding environment. Interactions with the spacecraft structure and appendages usually have a direct impact on the antenna performances.

ECSS-E-ST-20_0020346

For all high-power applications, the risk of generation of passive inter-modulation products by the surrounding spacecraft structure and appendages shall be assessed starting from Phase B, as a minimum, and the impact on antenna performances assessed.

### Antennas Verification

ECSS-E-ST-20_0020347

The requirements of this clause 7.2 shall be verified by the verification methods, at the reviews, and recorded in the documents as specified in Table 8-3.

For verification, see also ECSS-E-ST-10-02.

ECSS-E-ST-20_0020385

Table 7-1: <<deleted, merged with new Table 8-3>>

## RF Power

### Overview

The objective of the following RF breakdown requirements is to ensure that the space system operates at maximum power levels without any risk of Multipaction, RF power handling limitation and Corona (also called “gas discharge”).

Multipaction requirements are described in ECSS-E-ST-20-01.

RF power handling requirements are described in clause 7.3.2.

Corona (or Gas Discharge) requirements are described in clause 7.3.3 and apply for:

vented RF components during launch and pressurisation due to out-gassing of the spacecraft or re-entry, and

pressurized RF components.

### RF Power handling (thermal)

#### General requirements

ECSS-E-ST-20_0020348

All the components and equipments of the RF chain shall be able to stand the maximum specified operating RF power during its application in space with:

no degradation of the component,

no degradation of the RF signal including radiative losses, and

with their thermal levels not exceeding those corresponding to the maximum available RF power at the maximum qualification temperature.

#### Design and Verification

ECSS-E-ST-20_0020349

Each element of the RF chain shall be designed and verified to withstand the maximum specified operating RF power levels plus safety margins agreed with the customer in the development phase at the maximum qualification temperature.

### Corona or Gas Discharge

#### General requirements

ECSS-E-ST-20_0020350

All the components and equipments of the RF chain shall be free of any risk of Gas discharge (Corona) at the maximum specified operating RF power over the full pressure range during:

the depressurization of the RF components and equipments at launch environmental conditions,

the pressurization due to out-gassing of the spacecraft in orbit,

ground testing at ambient pressure, and

the pressurization of the spacecraft during planetary re-entry phases at the mission environmental conditions.

ECSS-E-ST-20_0020351

For those components and equipments which design does not allow operating them over the full pressure range the following action shall be taken:

specify the applicable pressure range and gas properties,

ensure that the design and manufacturing is such to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties.

#### Design and Verification

ECSS-E-ST-20_0020352

RF components and equipments of the RF chain shall be designed and verified to withstand the maximum specified operating RF power levels plus safety margins agreed with the customer in the development phase.

### Qualification for power handling and gas discharge

ECSS-E-ST-20_0020353

The following criteria shall be met for qualification for power handling and gas discharge:

the RF component and equipment has no physical degradation,

the RF component and equipment has no degradation of the RF performance during and after the test.

## Passive intermodulation

### Overview

Passive intermodulation products are generated when two or more RF transmit signals illuminate or passing through a non-linear passive RF component. The RF frequencies of the passive intermodulation products are derived as for any other generation of intermodulation products, when two or more RF signals are present simultaneously. However, the power levels of the passive intermodulation products depend on the materials used, manufacturing tolerances and processes, assemble techniques, and oxidation of surfaces. Thus, they are hardly predictable implying that verification by test is mandatory for those intermodulation products that can adversely impact the mission or cause interference in third party protected frequency bands.

### General requirements

ECSS-E-ST-20_0020354

The acceptance level of interference caused by passive intermodulation products shall be agreed with the customer in the development phase.

ECSS-E-ST-20_0020355

All the components of the RF chain shall be designed and manufactured to guarantee that the passive intermodulation products derived from the transmit carriers do not cause interference with any of the spacecraft receive bands or third party protected frequency bands during the operating temperature cycles.

### Identification of potentially critical intermodulation products

ECSS-E-ST-20_0020356

All operating conditions shall be identified in which two or more transmit RF signals simultaneously illuminate or passed through a passive RF component, equipment or both.

ECSS-E-ST-20_0020357

For each of the conditions identified in 7.4.3a, the frequencies, number of carriers and power levels of these carriers shall be determined.

ECSS-E-ST-20_0020358

An analysis shall be performed to establish all the passive intermodulation products falling within any of the spacecraft receive bands or third party protected frequency bands, for all combinations of frequency carriers up to the intermodulation order of 100.

### Verification

ECSS-E-ST-20_0020359

Testing at the lowest intermodulation order as identified in 7.4.3c shall be performed to ensure that the amplitudes of the passive intermodulation products are below the specified interference level.

ECSS-E-ST-20_0020360

Passive Intermodulation tests shall be carried out on the flight hardware in the same configuration as it is during operational use.

ECSS-E-ST-20_0020361

The test frequencies, number of carriers and power levels of these carriers shall be those as identified in 7.4.3b.

ECSS-E-ST-20_0020362

Qualification testing shall be carried out

on RF non radiative passive components, or equipments, or systems, over the full qualification temperature range,

on RF radiative components, equipments or systems over a temperature range to be agreed with the customer, range which can be limited to ambient temperature.

ECSS-E-ST-20_0020363

Acceptance testing shall be carried out on flight components, equipments or systems over an acceptance temperature range to be agreed with the customer, range which can be limited to ambient temperature.

### Qualification for passive intermodulation

ECSS-E-ST-20_0020364

The amplitude of each passive intermodulation product falling within any of the spacecraft receive bands or third party protected frequency bands shall be lower than the level specified in 7.4.2a.

## Verification

ECSS-E-ST-20_0020405

The requirements of the clauses 7.3 and 7.4 should be verified by the verification methods, at the reviews, and recorded in the documentation as specified in Table 8-3.

ECSS-E-ST-20_0020406

Table 7-2: <<deleted, merged with new Table 8-3>>

# Pre-tailoring matrix per space product and feature types

## Introduction

The pre‐tailoring matrix of this ECSS Standard is defined in Table 8-3 and is given as a guidance to support tailoring.

It identifies “Inclusive requirements” per space product type and “Exclusive requirements” per features.

Inclusive requirements are applicable to one or several of the following product types:

- SSE: Space Segment Element (physical view of Space Segment System)

SSS: Space Segment Sub-system

SSEq: Space Segment Equipment

LSE: Launch Segment Elements (physical view of Launch Segment System)

LSS: Launch Segment Subsystem

LSEq: Launch Segment Equipment

GSE: Ground Segment Elements (physical view of Ground Segment System)

GSS: Ground Segment Subsystems

GSEq: Ground Segment Equipment

Electrical Ground Support Equipment

For the definition of the space product types, refer to ECSS‐S‐ST‐00‐01.

“Ground Support Equipment” is a separate product type not to be confused with “Ground segment equipment”.

“Exclusive requirements” are valid to the specific following subsystem, equipment, function, characteristic or component, referred to as “features” in the present context:

- Electrical Power Subsystem

Power Conditioning Unit

Power distribution and protection

Solar Array

Solar Array Drive Mechanism/Electronics

Battery

Battery management

Redundant

Switching converter

Generates/Receives telecommand

High Voltage

Single shot device

Circuit for single shot device

Antenna and RF chain

Exclusive requirements refer to requirements that are exclusively applicable for the identified feature(s).

## Use of the inclusive and exclusive requirement categories

The way to use the inclusive and exclusive categories is the following:

- Identify which space product type requirements are applicable to a given case by using the relevant column of the inclusive requirements set. For example, SSEq (third column of space product types).

If SSEq is for example not a PCU, neither a PDU nor a Solar Array, it is possible to derive the requirements valid for the given case by removing from the column SSEq (inclusive requirements) those exclusively applicable to PCU, PDU and Solar Array, since these are valid for those equipment only.

Another example is given below:

- If the unit is a PCU, but it does not contain single shot devices, then use the column SSEq (inclusive requirements) and remove from that the requirements valid for relay, Hybrid or SSDs in the relevant columns.

In summary, a given requirement can be ignored on the condition that the features identified as exclusive for that requirement are altogether not relevant to the scope of the concerned space product(s).

The different statuses of requirement applicability used in the pre‐tailoring matrix are defined in Table 8-1.

A requirement is considered as applicable to a given space product type if it is verified on this product type.

In Table 8-3 verification points, methods and records are also provided.

Finally, it important to understand that the present pre‐tailoring is made assuming typically applicable architectures.

For example, the following hypotheses are made:

- Launchers have no solar arrays

Spacecraft encompasses satellites, landers, rovers and probes

Space vehicles encompass spacecraft and launchers

Solar Arrays are considered as equipment part of the Electrical Power Subsystem

New architectures and new technologies can request an update of the ECSS‐E‐ST‐20 and its pre‐tailoring matrix.

For the requirements of clause 7, the following remarks are for consideration when using the re‐tailoring matrix:

- The accommodation of the different antennas on the spacecraft being the duty of the spacecraft provider, some requirements, for example the one related to scattering effect, are managed at spacecraft level even if few characterization test are required at equipment supplier level (e.g. antenna supplier level). The final performance of antenna at satellite level being also the duty of the spacecraft supplier.

The spacecraft often accommodate several antennas and it is then also the duty of the spacecraft supplier to manage their compatibility even if some characterisation tests are performed by the antenna providers.

On board of spacecraft and on ground, an antenna is generally directive and composed of a reflector and one or several feeds. The antenna provider can procure the reflector and feeds from different suppliers.

In most of the case, feeds and reflectors are acceptance tested separately. Base on all those considerations, it’s considered here that

the level of “equipment” is devoted to feeds and reflector and more generally to the components of an antenna. They are acceptance tested at stand‐alone including RF characterization (feeds), shape and surface characterization (reflectors). Stand‐alone acceptance include generally complete or partial environmental testing.

the level of subsystem is devoted to the provision of one whole antenna (e.g. active antenna array, gregorian antenna assembly, antenna composed of reflector and feeds assembled at satellite level). An antenna is therefore considered as a subsystem that is different to the assumption of the ECSS Glossary (ECSS‐S‐ST‐00‐01) Annex B.1.

Table 8-1: Definition of pre-tailoring matrix applicability statuses

| Applicability status | Description |
| --- | --- |
| X | Requirement applicable |
| - | Requirement not applicable |
| // | Requirement applicability pre-tailoring not definable (applicability to be determined during tailoring) |
| >> | Requirement applicability pending tailoring to be achieved at lower product type |
| E | Requirement exclusively concerned by the identified feature(s) |

Table 8-2: Definition of features for exclusive requirements

| # | Feature | Comment |
| --- | --- | --- |
| 1 | Electrical Power Subsystem | Applicable to any segment |
| 2 | Power Conditioning Unit | Relevant to primary power conditioning |
| 3 | Power distribution and protection | Typically relevant (but not limited) to primary Power Distribution Unit |
| 4 | Solar Array |  |
| 5 | Solar Array Drive Mechanism/Electronics | Relevant to Solar Array Drive Mechanism or Electronics |
| 6 | Battery |  |
| 7 | Battery management |  |
| 8 | Redundant | Relevant to redundancy implemented at equipment level or at higher level |
| 9 | Switching converter | Relevant to power conversion by switching (including S3R) |
| 10 | Generates/Receives telecommand | Generates or Receives telecommand |
| 11 | High Voltage | High Voltage assessed with respect to Corona effect (250 V threshold for Earth atmosphere) |
| 12 | Single shot device | E.g. pyro devices or Non Explosive Actuators |
| 13 | Circuit for single shot device | E.g. pyro driver |
| 14 | Antenna and RF chain |  |

ECSS-E-ST-20_0020424

Table 8-3: Pre-tailoring matrix per “Space product and feature types"

| ECSS Source Id | Reqt. Statement | ECSS Object Type | Verification |  |  | Inclusive requirements |  |  |  |  |  |  |  |  |  | Exclusive requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Verification Points (SRR, PDR, etc, see complete list at the end of this table) | Verification Methods (RoD, A, etc see complete list at the end of this table) | Recorded in [1] Electrical ICD (incl. SAR, Battery Antenna) [2] Budget document (e.g. Power, Energy, Processor, and memory budgets) [3] DDF or DJF [4] GDIR [5] Tests Report [6] Specification [7] User Manual [8] EIDP | SSE: Space Segment Element (physical view of Space Segment System) | SSS: Space segment sub-system | SSEq: Space Segment Equipment | LSE: Launch Segment Elements (physical view of Launch Segment System) | LSS: Launch Segment Subsystem | LSEq: Launch Segment Equipment | GSE: Ground Segment Elements (physical view of Ground Segment System) | GSS: Ground Segment Subsystems | GSEq: Ground Segment Equipment | Electrical Ground Support Equipment | Electrical Power Subsystem | Power Conditioning Unit | Power distribution and protection | Solar Array | Solar Array Drive Mechanism/Electronics | Battery | Battery management | Redundant | Switching converter | Generates/Receives telecommand | High Voltage | Single shot device | Circuit for single shot device | Antenna an RF chain |
| 4.1.2a | Interface engineering shall ensure that the characteristics on both sides of each interface are compatible, including source and load impedances, the effects of the interconnecting harness and the grounding network between both sides comprising: common mode impedance conducted and radiated susceptibility and emission. | Requirement | SRR | RoD, A, T | - | X | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.2b | In order to minimize the number of interface types, standard interface circuitry shall be defined to be applied throughout a project. | Requirement | SRR | RoD | - | X | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.2c | Reconfiguration, high level or high priority command lines shall be immune to spurious activation. | Requirement | SRR | RoD, A | - | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - |
| 4.1.2d | The application of the nominal signals or a faulty signal to an un-powered interface shall not cause damage to that interface. | Requirement | PDR | A, T | - | X | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.2e | An undetermined status at the interfaces of a powered unit shall not cause damage to an un-powered interface. | Requirement | PDR | RoD, A, T | - | X | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.2f | Signal interfaces shall withstand without damage positive or negative nominal voltages that are accessible on the same connector, coming from the unit itself, from the interfaced units or from EGSE. | Requirement | PDR | RoD, A, T | - | X | X | X | X | X | X | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3a | Every command (intended to be sent to the spacecraft) shall be assessed for criticality at equipment level, and confirmed at subsystem/system level. | Requirement | SRR | RoD, A | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3b | All executable commands shall be explicitly acknowledged by telemetry. | Requirement | SRR | RoD, T | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3c | High Priority telecommand decoding and generation shall be independent from the main on-board processor and its software. | Requirement | PDR, CDR | RoD | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - |
| 4.1.3d | With the exception of pyrotechnic commands, the function of an executable command shall 1 not change throughout a mission, and 2 not depend on the history of previous commands. | Requirement | PDR, CDR | RoD | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3e | For commands of category 1 and 2 criticality, at least two separate commands for execution: an arm/safe or enable/disable followed by an execute command shall be used. | Requirement | PDR, CDR | RoD | - | X | // | // | X | // | // | X | // | // | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3f | The functionality shall be provided to repeat the transmission of all the executable commands without degradation of the function or a change of its status. | Requirement | PDR, CDR | RoD, A, T | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3g | In case of critical commands of category 1 and 2, at least two physically independent electrical barriers, including associated control circuits, shall be implemented for arming and executing the command. | Requirement | PDR, CDR | RoD | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.3h | Processor and simple logic circuits shall not be able to issue category 1 and 2 critical commands without a ground commanded arm/safe or enable/ disable command. | Requirement | PDR, CDR | RoD | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - |
| 4.1.3i | Any on–board processing which issues commands to reconfigure subsystems or payloads shall be overridable and potentially inhibited by ground command. | Requirement | PDR, CDR | RoD, T | - | X | // | // | - | - | - | X | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - |
| 4.1.3j | No valid command shall be issued until the transmitter power supply is within operational voltage range and ready to transmit the command. | Requirement | PDR, CDR | RoD, A, T | - | X | // | // | X | // | // | X | // | // | X | - | - | - | - | - | - | - | - | - | E | - | - | - | - |
| 4.1.4a | Telemetry data devoted to the spacecraft subsystem and payloads monitoring shall allow 1 the retracing of the overall configuration at least up to all reconfigurable elements. 2 the location of any failure able to impact the mission performances and reliability at least up to all reconfigurable elements. | Requirement | PDR, CDR | RoD, A | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.4b | The operational status (On/Off, enabled/disabled, active/not-active) of each element of any telemetry acquisition chain should be provided to the on-board computer in order to determine without ambiguity the validity of the telemetry data at the end of the overall chain. | Recommendation | PDR | RoD, A, T | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.4c | Primary bus load currents shall be monitored by telemetry, to enable, together with the bus voltage telemetry, a complete monitoring of a primary bus power load. | Requirement | PDR | RoD, A, T | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.1.4d | Telemetry shall be implemented to monitor the evolution of the power-energy resources and the source temperatures during the mission. | Requirement | PDR, CDR | RoD, A | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1a | Failure propagation shall meet the following conditions: 1 A single hardware failure does not propagate to neighbouring components circuits or interfaces in an undetermined way. 2 Failure propagation is verified by analysis. 3 Mechanical, thermal or electrical propagation of single hardware failures does not impair the corresponding protection or redundancy implemented at equipment or system level. 4Single hardware failure does not propagate to equipment or functions under different contractual responsibility than the item where the failure takes place. | Requirement | PDR | A | - | X | X | X | X | X | X | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1b | Redundant signal or power lines should be segregated via physically separated connectors and harnesses. | Requirement | PDR, CDR | RoD | - | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.1.1c | Routing of redundant power or signal lines within common harness or connector shall be justified by analysis showing that inside the electrical unit and at external connector interface level there is no potential single failure leading to affect both nominal and redundant lines or to generate electrical or electromagnetic interference between both. | Requirement | PDR, CDR | A | - | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.1.1d | Redundant functions shall be physically separated with no risk of failure propagation by thermal or other coupling and as a minimum, contained within different integrated circuits to avoid failure propagation. | Requirement | PDR | RoD. A | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.1.1e | For redundant functions implemented on the same PCB, a physical separation shall be provided, with no risk of thermal or other failure propagation. | Requirement | PDR | RoD, A | - | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.1.1f | For redundant functions implemented on the same PCB, any deviation of the physical separation specified in 4.2.1.1e shall be tracked in the Critical item List. | Requirement | PDR | RoD, A | - | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.1.1g | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1h | In case a cold redundant function is simultaneously activated together with the nominal one, by a deliberate or wrong command or due to a fault, this shall not induce permanent degradation of either of the two functions or loss of the mission before FDIR action. | Requirement | PDR | RoD, A | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.1.1i | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1j | Any active equipment, excluding heaters, continuously dissipating more than 20 W in nominal or failure condition shall include a temperature monitoring function, which is implemented and provides continuous temperature monitoring. | Requirement | PDR | RoD, A | - | - | - | X | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1k | In case of signal cross-strapping, no single failure of either interface circuit shall propagate to the other one. | Requirement | PDR | RoD, A | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1l | In the case of hot redundant essential functions, latching protection shall not be used unless it has an autonomous periodic reset. | Requirement | PDR | RoD | - | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1m | Disabling critical on-board autonomous functions shall be allowed only if an interlock mechanism is implemented, which prevents the disabling of both main and redundant functions at the same time. | Requirement | PDR | RoD, A, T | - | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1n | Any protection latch, which does not have autonomous reset capability, shall be at least re-settable from ground command. | Requirement | PDR | RoD, A, T | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1o | Any protection of an essential function shall not share with the essential function itself the same component or integrated circuit nor utilize common references or auxiliary supply. | Requirement | PDR | RoD | - | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1p | Essential functions shall not be lost upon a single failure of other functions which are centrally powered or controlled. | Requirement | PDR | RoD | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1q | For essential functions supplied by an FCL, lock-up phenomenon requiring recovery via the removal of external power shall be prevented. | Requirement | PDR | A, T | - | X | X | // | X | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1r | All units to be powered during launch shall be designed for operation with critical pressure. | Requirement | PDR | A, T | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1s | A venting analysis shall be performed for all units not designed to operate under critical pressure and not powered during launch, to determine when they can safely be turned-on. | Requirement | PDR | A | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1t | Any on-board autonomous function, the failure of which can result in malfunctions of category 1 and 2 criticality, shall have override capability. | Requirement | PDR | RoD, A, T | - | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1u | Any on-board autonomous protection override, leading to hazardous situation for the mission (category 1 and 2 criticality), shall not be implemented. | Requirement | PDR | RoD, A | - | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1v | SEE shall not activate protection circuits of essential functions. | Requirement | PDR, CDR | RoD, A | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1w | The spacecraft electrical system shall be single failure tolerant for unmanned mission and double failure tolerant for manned mission. | Requirement | PDR | RoD, A | - | X | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1x | Occurrence of a non-destructive SEE after a failure shall not lead to the loss of the mission | Requirement | CDR | RoD, A | - | X | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.1y | For electrical connections of critical lines directly exposed to free space, the distance between power lines or between power and return lines shall not be considered an effective insulation from dielectric point of view, including inside connectors. | Requirement | CDR | RoD | [3] | X | X | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.2a | Reliable insulation requirements in this standard shall apply to all critical nets. | Requirement | PDR, CDR | A or T Hardware may not exist at PDR and hence Test not possible | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3a | For those space applications where electrical lines insulation can be damaged by meteorites or micro‐meteorites, dedicated Reliable Insulation requirements or agreed equivalent solution shall be established before satellites development according to the environment encountered and to the required reliability figure at system level. | Requirement | SRR | RoD | [6] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3b | The presence of foreign particles inside equipment, subsystem or system shall be treated as a credible event. | Requirement | SRR, PDR, CDR | RoD | - | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3c | The non-existence of foreign particles shall be verified by visual inspection or test. | Requirement | DRB | INS, T | [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3d | The presence of foreign particles shall be assessed, recorded in the DJF, and resolved if not compatible with the application. | Requirement | PDR, CDR | A | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3e | All conductive elements of critical nets shall be provided with insulation means. | Requirement | PDR, CDR, TRB, AR | RoD, INS | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3f | Requirement 4.2.1.2.3e shall not apply to: 1. Solar Array solar cells, by-pass diodes, conductive coverglasses, solar cell interconnectors, blocking diode assembly and terminations, where no insulating material apart from vacuum is present among them, 2. Solar Array Drive Mechanism slip rings, and 3. Battery cell technologies for which loss of insulation is covered by other specific provisions. | Requirement | CDR | RoD | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3g | To ensure an effective insulation from dielectric point of view, the gap among critical lines or between a critical line and any other conductor shall comply to the following two cases, for operational voltages up to 250 V: 1. Case 1: invariable gap greater than 1mm : use of 1 insulating material (non-gaseous / non-vacuum); 2. Case 2: variable gap or gap smaller than 1mm : use of 2 different materials (including at least 1 rigid). | Requirement | PDR, CDR, TRB, DRB, AR | A, T, INS | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3h | The worst case minimum thickness of the materials for reliable insulation shall be taken into account, and not their nominal or design values. | Requirement | PDR, CDR | A | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3i | It shall be demonstrated that no critical air bubbles to cause discharges are present in adhesives. | Requirement | DRB | INS | [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3j | The physical implementation of the reliable insulation shall be documented and recorded. | Requirement | PDR, CDR, AR | ROD,A | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3k | The rigid and non-rigid materials used to ensure reliable insulation shall be listed in the document mentioned in 4.2.1.2.3j with their thickness and characteristics or the reference to the respective ESCC standard be indicated. | Requirement | PDR, CDR | A | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3l | The electrical insulation materials shall have a dielectric strength in excess of 2 times the peak operative voltage in the actual application. | Requirement | PDR, CDR, DRB, AR | A, T | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3m | The electrical insulation materials shall be resistant to the different phases of the unit MAIT (Manufacturing, Assembly, Integration and Test) process. | Requirement | PDR, CDR, DRB, AR | A, T | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3n | Materials used in direct contact between 2 parts to be reliably insulated and between which an electrical field is present, shall not be contaminated, or contain other non-homogeneous pathway for leakage current to develop in clean room conditions, with max 65 % RH. | Requirement | DRB, AR | INS | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3o | In relation to reliable insulation, a material shall be treated as rigid if its Vickers hardness is greater than 20, either in raw state or after treatment. | Requirement | PDR, CDR | A | A | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3p | In relation to reliable insulation and in alternative to requirement 4.2.1.2.3o, a material shall be considered as rigid if the insulation layer produced with it can guarantee the required minimum thickness, for all required operative conditions including manufacturing and test process. | Requirement | PDR, CDR, DRB, AR | A, T | A, T | [3], [8] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3q | For critical lines and nets involving voltages exceeding 250 V, a minimum of 2x the distance identified by a dedicated analysis shall be used. | Requirement | PDR, CDR, TRB, DRB, AR | A, T | [3], [5], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3r | For critical lines and nets involving voltages exceeding 250 V, insulation shall be demonstrated by test with the application of 2x the operational voltage limit. | Requirement | TRB, DRB, AR | T | [5], [8] | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3s | A foreign particle should not jeopardise one insulation layer of the reliable insulation. | Recommendation | PDR, CDR | RoD, A | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3t | Conductors with sharp edges, or geometries preventing the access to conformal coating shall be locally insulated with suitable material. | Requirement | PDR, CDR, TRB, DRB, AR | RoD, INS | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3u | Where the use of shielded wires and/or over-shielded harness together with reliable insulation is required: 1.the issue shall be brought to customer attention, and 2.the absence of any short-circuit risk between wire core and shield shall be demonstrated all along the harness, as well as at shield grounding connection points. | Requirement | PDR, CDR, TRB, DRB, AR | RoD, INS | [1], [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3v | In case flexible connections are used, i.e. flex PCBs, the applicable requirements of clause 8 of ECSS-Q-ST-70-12 shall be applied. | Requirement | PDR | RoD | see ECSS-Q-ST-70-12 | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3w | In the connectors mating area, the risk of short-circuit between a pin to be reliably insulated and a conductor at any other potential, due to a particle or to a bent pin shall be avoided. | Requirement | PDR, CDR | ROD, I | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3x | Critical lines and nets shall be identified as part of a FMECA process or of a dedicated analysis at the beginning of the system, subsystem or equipment development, at relevant PDR at the latest. | Requirement | PDR | ROD | [1], [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3y | A dedicated analysis shall confirm that reliable insulation of critical lines or nets is not jeopardized by failures, at system, subsystem or equipment level. | Requirement | PDR, CDR | A | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3z | Critical lines or nets shall be clearly identifiable in electrical diagrams and in layout and assembly drawings. | Requirement | PDR, CDR | RoD | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3aa | The electrical diagrams, layout and assembly drawings providing the information of the critical lines or nets may be part of the relevant analysis report. | Permission | PDR, CDR | ROD | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3bb | Compliance to Reliable insulation requirements of critical lines or nets shall be verified by review of design as well as by inspection and/or by test if possible. | Requirement | PDR, CDR, DRB, AR | ROD, INS, T | [3], [5], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.1.2.3cc | Verification of reliable insulation requirements should take place any time when it might be jeopardised by integration or rework activities, exposure to specific environment, electrical connection or disconnection. | Recommendation | TRB, DRB, AR | INS, T | [3], [8] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.2.2a | For technical budgets and margin policy the requirements of ECSS-E-ST-10 clause 5.4.1.2 shall be applied. | Requirement | SRR | RoD | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.2.2b | The margin for available memory size and load factors of processors should be 1 for new developments, 50 % as a minimum at PDR for new on board software parts; 2 25 % at launch. | Recommendation | PDR | RoD, A | - | - | - | X | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.2.2c | The margin on the throughput of on-board communication networks should be 1 for new developments, 50 % as a minimum at PDR on the average throughput; 2 such that real time overflow is avoided. | Recommendation | PDR | RoD, A | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.2.2d | In the absence of specific mission requirements the following applies: After error correction, reset or data corruption of main functions at equipment level should be kept to a rate of occurrence less or equal to 10-4 per day for worst case conditions of environment. | Recommendation | PDR | RoD, A | - | - | - | X | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.2.2e | For programmable logic devices (FPGA), the available margin of unused blocks and pin shall be agreed at FPGA SRR based on design complexity, requirement stability and experience return. | Requirement | SRR | RoD, A | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3a | A connector carrying source power or external test connectors on units shall have no contact areas exposed to possible short circuit during mating and de-mating process. | Requirement | PDR | RoD, INS | - | X | X | X | X | X | X | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3b | All external test connectors on a unit and spacecraft skin connectors shall be covered for flight. | Requirement | AR | INS | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3c | The test connector covers should be metallic or metallized and grounded to structure. | Recommendation | AR | RoD | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3d | The use of a connector saver for ground testing shall not alter the performance of the equipment. | Requirement | CDR | RoD, A | - | - | - | X | - | - | X | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3e | It shall be ensured that erroneous mating is avoided by connector keying or marking. | Requirement | CDR | RoD, INS | - | X | X | X | X | X | X | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3f | If the equipment has several connectors, visibility and clearance around each of them should be such as to enable mating or de-mating without disturbing others already in place or necessitating custom–made tooling. | Recommendation | PDR, CDR | RoD | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3g | For supplies and signals of pyrotechnics and non-explosive single shot device drivers, different connectors should be used for different classes of electrical functions. | Recommendation | PDR | RoD | - | X | X | X | X | X | X | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | E | - |
| 4.2.3h | When 4.2.3g is not met, power, signals, and telemetry shall be separated in the connector by a set of unused pin locations. | Requirement | PDR | RoD | - | X | X | X | X | X | X | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | E | - |
| 4.2.3i | Except when pin allocation is imposed by a standard specification, spare contacts or sockets should be available on each connector. | Requirement | PDR | RoD | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3j | For new developments, when the connection is not aligned to a defined standard, 10% spare contacts at unit PDR and at least 5 % at CDR shall be achieved with in any case a minimum of two spare contacts available at CDR. | Requirement | PDR | RoD | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3k | In the absence of grounding provision at connector shell level, at least one contact per connector shall be connected to the unit structure as provision for potential additional grounding at subsystem or system level. | Requirement | PDR | RoD | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3l | Provision shall be taken to avoid arcing or short circuits in connectors. | Requirement | PDR | RoD, A | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3m | The following shall be performed for any connector the loss of which can lead to the loss of the mission: 1 Document the connector in the single point failure list 2Verify and document its integrity up to the highest spacecraft integration level, to avoid accidental demating | Requirement | PDR | RoD, INS | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3n | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.3o | Battery and solar array power shall be distributed by multiple contacts on both positive and return lines. | Requirement | PDR | RoD | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4a | Test-stimulus points shall be accessible without the need of modifying the electrical configuration of an item of equipment. | Requirement | PDR | RoD, INS | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4b | Test-stimulus points shall be protected for flight operation. | Requirement | PDR | RoD, INS | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4c | For the purpose of meeting requirement 4.2.4a and 4.2.4b, dedicated test connectors should be used. | Recommendation | PDR | RoD | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4d | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4e | Test points on equipment shall be protected against damage up to the maximum fault voltage present on the connector either coming from the equipment or the EGSE. | Requirement | PDR | RoD, A | - | - | - | X | - | - | // | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4f | Test points on equipment shall be such that unintentional connection of these points to ground does not damage the equipment. | Requirement | PDR | RoD, A | - | - | - | X | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4g | The redundancy of parts and functions, which failure can lead to the loss of the mission or human injury, shall be verified by test simulating the failure event. | Requirement | CDR | A, T | - | X | X | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4h | Circuits meant for on-ground testing or unused circuits shall not create or be sensitive to the noise expected during operation. | Requirement | PDR | RoD, A or T | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4i | The protection of functions, which failure can lead to the loss of the mission or human injury, shall be verified by test simulating the failure event. | Requirement | CDR | A, T | - | X | X | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4j | The test of a protection function or a redundant function shall present no risk of stress or failure propagation due to the injection of stimuli. | Requirement | PDR | RoD, A | - | X | X | // | // | // | // | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4k | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4l | Hot redundant functions and protection functions that cannot be tested beyond unit level shall be identified in the critical item list. | Requirement | PDR | RoD, T | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.4m | All redundant functions and protection functions shall be tested, up to the highest possible level of integration of the unit. | Requirement | PDR | RoD, T | - | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - |
| 4.2.4n | Redundant units within a system shall be verified by test at system level. | Requirement | PDR, AR | RoD, T | - | X | X | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.4o | Protection functions within a unit protecting other units shall be verified by test at system level or at unit level with representative interfaces. | Requirement | PDR | RoD, T | - | X | // | // | X | // | // | - | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.5a | Wired electrical connections shall contain stress relief. | Requirement | PDR | RoD, INS | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.6a | Each item shall be directly interchangeable in form, fit, and function with other equipment of the same part number and of the same qualification status. | Requirement | PDR | RoD | - | - | - | X | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.6b | The uniformity of the electrical performance characteristics of the units shall enable equipment interchange provided a minimum set of adjustments and recalibration as described in the unit user’s manual. | Requirement | PDR | RoD | - | - | - | X | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.2.6c | When components operating in a single event are used, 4 times the quantity to be used for flight units shall be procured as one lot: 25 % for the lot acceptance test, 25 % for flight use, 25 % for spares and 25 % for a confirmation test near to the launch date. | Requirement | PDR | INS | - | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 4.2.6d | The number of components to be procured shall be defined to ensure, as a minimum, the quantity needed for flight and flight spares, plus the number of components to be tested at incoming reception and components to be tested just before launch in case of alert or failure. | Requirement | PDR | INS | - | X | X | X | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.3.1a | The requirements of this Clause 4 should be verified by the verification methods and at the verification points listed in Table 8-3. | Recommendation | - | - | - | X | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.3.1b | In case verification by analysis of an electrical part or circuit is not possible by lack of data, complementary verification by test shall be performed. | Requirement | PDR,CDR | A, T | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.3.2a | The design report, PSA, WCA, FMECA, thermal analysis, radiation analysis, EMC analysis and the detailed circuit diagrams including component values, shall be part of the Design Definition and Justification Files. | Requirement | PDR,CDR | RoD | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.3.2b | Failure modes of all components used in a unit shall be defined. | Requirement | SRR | RoD | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.3.2c | FMECA shall be performed and based on the failure modes previously defined at component level. | Requirement | PDR | A | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4.3.2d | The Design Definition and Justification Files shall be delivered by the supplier to the customer at the agreed verification points in compliance with Table 8-3. ECSS-E-ST-20_0020380 | Requirement | N/A | RoD | - | X | X | X | X | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.1a | Budgets and margins shall be established during Project phase B, and reviewed in all subsequent phases of the project. | Requirement | PDR | RoD | [2] | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.1a | The power subsystem of a spacecraft shall be able to generate, store, condition, distribute and monitor the electrical power used by the spacecraft throughout all mission phases in the presence of all environments actually encountered. | Requirement | SRR | RoD, A, T | [3][5] | - | // | - | - | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.2a | An analysis of power demand versus power available shall be performed, including average peak power, for all phases of the mission. | Requirement | PDR | RoD, A | [2][3] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.2b | An analysis of the energy demand versus energy available shall be performed in all phases of the missions, including inrush power demands, eclipses, solar aspect angle and depointing and also failure mode affecting the power system. | Requirement | PDR | RoD, A | [2][3] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.2c | A power budget shall be established based on the peak power values and an energy budget based on the average power values for all mission phases. | Requirement | PDR | RoD | [2] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.2d | A plan shall be established for the maintenance and periodical review of the budget established in requirement 5.2.2.2c during all project phases. | Requirement | PDR | RoD | [3] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.2e | A system margin of not less than 5 % at AR on available power and energy shall be included in the budget, available (as a minimum) with the solar array string losses as defined by the customer with the minimum of one string lost and one battery cell failed during all the designed life of the power system including all spacecraft modes of operation. | Requirement | PDR, AR | RoD, A | [2] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.2.2.2f | When actually using a MPPT, it shall be ensured that the transferred power is at least the required power, independent of the solar array operating conditions or environment. | Requirement | PDR | RoD, A | [2][3] | - | // | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.3a | Any protection function of a power converter or regulator preventing failure propagation shall: 1 not be implemented in the same integrated circuit, and 2 not utilize common references. | Requirement | PDR | RoD | [3] | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.3b | It shall not be possible to inhibit a protection feature if such an inhibition can lead to the loss of the main primary power bus in case of a single failure at spacecraft level. | Requirement | PDR | RoD, A | [3] | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.3c | In flight operation, if primary power bus shutdown happens, the system, including the power subsystem, shall be able to restart. | Requirement | PDR | RoD, A | [3] | X | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.3d | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.4a | The electrical power interface internal or external to the power subsystem shall be specified, including source and load impedance. | Requirement | SRR | RoD | [1][4][6] | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.4b | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.4c | The availability of the specified solar array power up to the power conditioning electronics shall be verified as follows: 1on solar array level, availability of the specified solar array power up to and including the solar array connector by means of flasher tests, supported by correlated analysis, 2on spacecraft level, full steady-state solar array power conditioning capability from solar array connector to power conditioning electronics, including solar array drive mechanism if any and harness, using solar array simulator, 3and finally, on spacecraft level, correct electrical connection of the solar array to the power conditioning unit by means of a flood test, that is by illumination of the solar array with a portable continuous lamp, checking if the generated voltage and current are detected at power conditioning side. | Requirement | AR | A, T | [3][5] | X | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.4d | The solar array interface voltage shall be defined at the solar array connector interface. | Requirement | PDR | RoD | [1][4][6] | - | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.4e | The solar array interface voltage shall include voltage losses within the electrical circuitry of the solar array, including at least blocking diodes, wiring resistance and losses associated with harness interconnections in operational conditions. | Requirement | PDR | A | [3] | X | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2a | The solar array shall be specified to provide the requested power and ensure the energy balance in each mission phase during operational life including any string loss tolerance defined by the customer, spacecraft charging effects and worst case conditions. | Requirement | PDR | A | [2] | - | // | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.5.2b | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.5.2c | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.5.2d | Provision shall be made against potential failure propagation in case of short-circuit failure of a solar array section or short circuit of its connection to the power subsystem. | Requirement | PDR | RoD, A | [3] | >> | // | // | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.5.2e | The solar array design shall be such that charging phenomena do not degrade the performance of the solar array below the ones specified in 5.5.2a and meeting the requirements specified in clauses 5.5.1.5 of ECSS-E-ST-20-08. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2f | The following requirements and omissions for ESD shall apply: 1. ESD testing is performed. 2. ESD testing is be performed in line with ECSS-E-ST-20-06 and ECSS-E-ST-20-08 demonstrating minimum safety margins as specified in clauses 5.5.1.5.of ECSS-E-ST-20-08. 3. ESD testing of solar arrays can be omitted for solar arrays with voltage less than or equal to 10 V, independent of current and gap distance. 4. For voltages below 30 V ESD testing can be omitted as well, provided the current remains below 0,6 A and gap width above 0,9 mm. | Requirement | Qualification review | T | [5], [8] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2g | In all configurations, stowed and deployed, the solar array , deployable and body mounted, and spacecraft structure shall be insulated from each other, disregarding the bleed resistor. | Requirement | PDR, CDR, TRB, DRB, AR | RoD, T | [3][5] | >> | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2h | In the flight configuration, means to prevent differential voltage due to electrostatic charging between solar array structure and the spacecraft electrical ground reference shall be implemented. | Requirement | PDR | RoD, A, T | [3][5] | >> | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2i | In the flight configuration, bleeding resistors shall be implemented. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2j | At solar array level, one short between a solar cell string and a conductive panel structure shall not produce any solar array power loss. | Requirement | PDR | RoD, A | [3] | >> | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2k | At solar array level, in case of two shorts on the same panel, the power loss shall not be more than the power of two strings. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2l | The PVA layout shall be designed to meet the solar array magnetic moment requirements. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2m | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.5.2n | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.5.2o | Provision shall be made to prevent failure due to operation in shadow. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2p | Solar array shall be designed in sections according to the redundancy principle specified at system level. | Requirement | PDR | RoD | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2q | Solar cells shall be protected against any deleterious reverse-bias conditions. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.2r | The conductive panels substrates shall be electrically insulated from each other and from the other structural parts of the solar array. | Requirement | PDR, CDR, TRB, DRB, AR | RoD, T | [6], [3], [8] | >> | >> | // | - | - | - | - | - | - | - |  |  |  | E |  |  |  |  |  |  |  |  |  |  |
| 5.5.2s | For the purpose of insulation in solar arrays the following shall not be considered as a valid insulating layer: 1. Silicone based adhesives 2. Conformal coatings | Requirement | PDR, CDR | RoD | [3], [8] | >> | >> | // | >> | >> | // | - | - | - | - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.5.2t | Insulation of solar array wires shall be provided with two different layers of materials or use of 2 individually cured layer of materials. | Requirement | PDR, CDR | RoD | [1], [3] | >> | >> | // | - | - | - | - | - | - | - |  |  |  | E |  |  |  |  |  |  |  |  |  |  |
| 5.5.2u | All individual electrical elements of the solar array shall be reliably insulated with respect to the ground of the spacecraft. | Requirement | PDR, CDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - |  |  |  | E |  |  |  |  |  |  |  |  |  |  |
| 5.5.3a | Computation of solar array power shall be based on measurements at cell level performed in accordance with the requirements of clause 10 of ECSS-E-ST-20-08. | Requirement | PDR | INS | [6] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3b | The model used for the computation of the I(V) curve of the solar cell shall be validated by test on the specific solar cell type for the mission in conditions representative of the expected domain of operation. | Requirement | PDR | A, T | [3][5] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3c | I(V) solar cells characteristics shall be computed in BOL and EOL conditions at maximum and minimum operating temperatures according to the mission profile. | Requirement | PDR | A | [2][3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3d | The EOL solar cell I(V) curve shall be derived from measurements performed at the temperatures specified in 5.5.3c after irradiation with particles in conformance with the “Electron irradiation” and “Proton irradiation” tests for “Bare solar cells” specified in ECSS-E-ST-20-08 clause 7, and agreed with the customer. | Requirement | CDR | T | [5] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3e | The forward voltage of the string blocking diode (if present) shall be computed: 1using the worst-case voltage drop specified by the diode manufacturer, 2at the diode operating temperature corresponding to the operational string current for each mission phase in worst case conditions. | Requirement | PDR | A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3f | The BOL worst and best case power calculations shall include the parameters indicated in Table 5-1. | Requirement | PDR | RoD, A | [2][3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3g | For best case calculations, the string current shall account for the difference between the specified current and the average production value. | Requirement | PDR | RoD, A | [2][3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3h | In addition with the parameters indicated in Table 5-1, the EOL worst and best case calculations shall include the parameters indicated in Table 5-2. | Requirement | PDR | A | [2][3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3i | Shadowing and hot spot phenomena shall be analysed. | Requirement | PDR | A | [2][3] | // | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3j | Leakage losses of bypass diodes shall be deducted from the power computation if they represent more than 0,1 % of the overall power to be provided. | Requirement | PDR | A | [2][3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.3k | Plume impingement effects shall be analysed. | Requirement | PDR | A | [2][3] | // | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - |
| 5.5.4a | The qualified de-rated current capability of wires, connector pins and slip ring contacts shall be greater than the best case BOL solar array section current in short circuit and include the effects of transient currents caused by the discharge of the solar array section capacitance. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - |
| 5.5.4b | The design of the insulation barriers between adjacent wires, connector pins and slip rings shall be such that no discharge phenomena can occur. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - |
| 5.5.4c | Where non-insulated conductors are used, arcing phenomena shall be prevented by design. | Requirement | PDR | RoD, A | [3] | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - |
| 5.6.2a | The battery shall be specified to ensure the energy balance in each mission phase during operational life, including contingency modes resulting from a single failure for unmanned missions and two failures for manned missions. | Requirement | PDR | RoD, A | [1][2][3][6] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2b | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.2c | Specific measures shall be taken in the battery design to keep under control the series inductance and the magnetic moment. | Requirement | PDR | RoD, A | [3][6] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2d | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.2e | Batteries having to tolerate a single fault shall be designed such that they can operate with one cell either failed shorted or open circuit. | Requirement | PDR | RoD, A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2f | In batteries having to tolerate a single fault and where the effects of a single cell failure are mitigated by the use of a cell bypass device, the following shall be met: 1The probability of the bypass circuit untimely operation is lower than the probability of a failure of the cell. 2If the bypass operation is not instantaneous, the power subsystem design is able to operate without damage during the transient situation. 3The maximum number of cells that can be bypassed after a failure or a wrong command is equal to the number of failures allowed by the specific mission design. | Requirement | PDR | RoD, A | [1][2][3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2g | Transient currents, occurring when two or more separate strings of series-connected cells are connected together in parallel, or when a cell fails in short-circuit within a battery composed of parallel strings, shall not result in exceeding the peak cell current rating. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2h | Battery supplier shall: 1specify cell to cell performance variations so that mission requirements can be met, 2provide methodology and validation of cell performance variations for any spare or flight modelsprovide methodology and validation of cell performance variations for any spare or flight models | Requirement | PDR | RoD, INS | [3][6] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2i | When batteries are discharged in parallel, this discharge shall not result in current and temperature exceeding the cell qualification limits. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2j | Conducting cases of battery cells in a battery package shall be insulated from each other with reliable insulation. | Requirement | PDR | RoD | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2k | Provisions for interfacing the battery with the ground support equipment during pre-launch operations shall be made. | Requirement | PDR | RoD | [1][3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2l | A logbook shall be maintained by the supplier for each flight battery starting with the first activation after battery assembly up to launch, describing chronologically all test sequences, summary of observations, identification of related computer–based records, malfunctions, and references to test procedures and storage conditions. | Requirement | CDR | INS | [7] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2m | Battery and spacecraft thermal design shall ensure together that: 1maximum and minimum qualification temperature of cell operation under intended cycling conditions are not exceeded; 2maximum qualification temperature gradients between different parts of the same cell and between two cells in a battery are not exceeded. | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.2n | <<deleted>> ECSS-E-ST-20_0020410 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.2o | Conductive cases of battery cells shall be reliably insulated from spacecraft structure, with an insulation between any cell and the spacecraft structure greater than 10 M, measured at 500 V DC. | Requirement | PDR | RoD, T | - | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.3a | Absolute maximum ratings of the cell, in term of temperature, voltage, charge and discharge current in continuous and peak condition, shall be defined. | Requirement | PDR | RoD, A | [3][7] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.3b | The ability of a cell to meet mission lifetime requirements, where not covered by qualification life testing or previous in flight experience, shall be justified by the ground test data or by dedicated tests under representative conditions. | Requirement | PDR | RoD, A, T | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.3c | The ability of a cell to meet mission life time requirements may be verified by similarity with qualification life testing or previous in flight experience only in case of identical design and identical manufacturing processes. | Permission | PDR | RoD, A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.3d | For any intended cell operation under acceleration greater than 1 g, the supplier shall ensure that no effect upon both short term (e.g. capacity) performance and lifetime can prevent battery nominal operation. | Requirement | PDR | A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.3e | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.3f | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.3g | The battery supplier shall inform the customer of any change in design, materials or process from cells which have experienced life testing or flight. | Requirement | PDR | RoD, INS | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.4a | The design of the spacecraft shall be such that modules and batteries can be removed and replaced at any time prior to launch without affecting the acceptance status of the rest of the spacecraft. | Requirement | PDR | RoD, A | [3] | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.4b | For the procurement of cells and batteries the manufacturer shall supply a user manual in conformance with Annex D. | Requirement | PDR | RoD | [7] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.4c | Flight batteries should not be used for ground operations to prevent any possible damage and subsequent degradation of life performance. | Recommendation | TRR | RoD | [3][6] | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.4d | If 5.6.4c is not met, the flight worthiness of the batteries shall be re-verified after these ground operations are completed, in time for a possible replacement. | Requirement | TRR | RoD, T | [5] | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.4e | Any test equipment interfacing with the battery shall include an associated undervoltage, overvoltage, overcurrent and over-temperature activated insulation switch. | Requirement | PDR | RoD, A, T | [3][5] | - | - | - | - | - | - | - | - | - | X | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.4f | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.4g | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.6.5.2a | The battery safety shall be managed in conformance with ECSS-Q-ST-40. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.5.2b | The design of the battery and associated monitoring and control electronics shall preclude the occurrence of any of the following: 1Over-temperature (from battery thermal dissipation or environmental heating); 2excessive currents (discharge or charge) including short–circuit (external or internal to the battery); 3overcharging; 4Attempt to charge in the case of primary cells; 5over discharge (including cell reversal); 6cell leakage (gases or electrolyte). | Requirement | PDR | RoD, A | [3] | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | E | E | - | - | - | - | - | - | - |
| 5.6.5.2c | Where 5.6.5.2b is not met, the design shall mitigate the damaging effects of any such failure mode | Requirement | PDR | RoD, A | [3] | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | E | E | - | - | - | - | - | - | - |
| 5.6.5.2d | The failure of one or more cells within a battery due to imbalance in the state of charge, temperature or other parameter between cells should be prevented by the battery control electronics. | Recommendation | PDR | RoD, A | [3] | // | // | // | // | // | // | - | - | - | // | - | - | - | - | - | E | E | - | - | - | - | - | - | - |
| 5.6.5.2e | When the battery has non-insulated, exposed cell terminals, the battery should be delivered with a red insulation cover to be removed before spacecraft closure and for flight. | Recommendation | PDR | RoD, INS | [1][3][7] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.6.5.2f | Provision should be made not to change the thermal balance of the battery during charge and discharge operations with the cover notified in 5.6.5.2e. | Recommendation | PDR | RoD, A | [3] | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| 5.7.2a | No single failure shall result in the loss of the power subsystem capability to the extent that the minimum mission requirements, in any of its phases, cannot be fulfilled. | Requirement | PDR | RoD, A | [2][3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2b | For manned missions, no double failure shall result in the loss of the power subsystem capability to the extent that the minimum mission requirements, in any of its phases, cannot be fulfilled. | Requirement | PDR | RoD, A | [2][3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2c | The primary power bus voltage regulation control for a fully regulated bus shall be independent from any control external to the electrical power subsystem. | Requirement | PDR | RoD, A | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2d | The ultimate switching between main and redundant MPPT circuitry, in case of MPPT malfunction, shall be implemented in a way to avoid infinite reconfiguration loops. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2e | No single failure in the spacecraft shall open or short a main electrical power bus or violate the specified over voltage or under voltage limit requirements. | Requirement | PDR | RoD, A | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2f | The design shall ensure that under all conditions during the required lifetime, including operation in eclipse with one battery cell failure and one solar array string failed, the primary bus voltage remains within specified performances. | Requirement | PDR | RoD, A | [2][3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2g | For fully regulated buses, the nominal bus voltage value should be standardized according to the following: 128 V for power up to 1,5 kW; 250 V for power up to 8 kW; 3100 V and 120 V for higher power. | Recommendation | PDR | RoD | [2][3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2h | A fully regulated bus shall keep its nominal value in steady state within ± 0,5 % of the bus voltage at the main regulation point. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2i | With a fully regulated bus in nominal operation the bus voltage transients shall: 1for load transients of up to 50 % of the nominal load not exceed 1 % of its nominal value. 2for any source and load transients remain within 5 % of its nominal value.Fuses should be avoided to maintain the quality of the bus. | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2j | Fuses should be avoided to maintain the quality of the bus. | Recommendation | PDR | RoD | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 5.7.2k | In case of fuse blowing, the recovery from the fuse clearance shall not produce an overshoot of more than 10 % above the nominal bus value. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 5.7.2l | The model of the fuse and of the electrical network to be protected by the fuse, shall be validated by test with a representative set-up | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 5.7.2m | A fully regulated bus shall have a nominal ripple voltage below 0,5 % peak-to-peak of the nominal bus voltage, measured at the regulation point with at least 1 MHz bandwidth. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2n | A fully regulated bus shall have commutation voltage spikes in the time domain of less than 2 % peak-to-peak of the nominal bus voltage, measured at the regulation point with a 50 MHz minimum bandwidth. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2o | At the point of regulation, the impedance mask of a fully regulated bus, operating with one source shall be below the impedance mask shown in Figure 5-1. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2p | For unregulated buses, the following parameters shall be specified, analysed and tested: 1 maximum and minimum bus voltage guaranteed at payload level in all steady state and transients conditions; 2 maximum ripple in time domain, measured with at least 1 MHz bandwidth. 3 maximum spikes in the time domain superimposed on the bus voltage, measured with a 50 MHz minimum bandwidth. 4 impedance mask. | Requirement | PDR | RoD, A, T | [3][5][6] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2q | During integration phase the power subsystem shall be able to start up from any of its power sources irrespective of the connection of the other power source. | Requirement | PDR | RoD, A, T | [3][5] | >> | // | // | >> | // | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2r | In the case of an unexpected battery or battery simulator disconnection, the main power bus voltage shall remain below its maximum specified overvoltage requirement. | Requirement | PDR | RoD, A, T | [3][5] | >> | // | // | >> | // | // | - | - | - | // | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2s | The design shall ensure that a short circuit to ground or to the return line of a solar array section does not result in a failure of category 1 and 2 criticality. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.2t | Control of the battery by OBC shall be robust to OBC failure and the time needed to reconfigure. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3a | On-board battery chargers shall be designed to ensure charging of a battery discharged down to zero volts. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3b | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3c | The minimum energy reserve in the battery shall be enough to guarantee the mission and a safe recovery of the spacecraft under all conditions. | Requirement | PDR | RoD, A | [3][2] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3d | The charging technique shall be designed to ensure that the batteries are managed in accordance with the manufacturer recommendations provided in the design description, justification file and user’s manual. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - |
| 5.7.3e | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3f | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3g | <<deleted>>The ultimate over charging/discharging protection circuitry shall be implemented by hardware and independent from any on board software. | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.3h | The ultimate over charging/discharging protection circuitry shall be implemented by hardware and independent from any on board software. | Requirement | PDR | RoD, A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - |
| 5.7.3i | Battery charge and discharge management shall be such that a single failure for unmanned missions and two failures for manned missions does not impair the lifetime of the energy storage system with respect to minimum or maximum voltage as well as maximum charge or maximum discharge current. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.4a | For fuse protected busses the electrical subsystem shall be robust against any fuse blowing event occurring on the primary bus, even after one failure anywhere in the power subsystem. | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 5.7.4b | All non-essential loads shall be switched-off autonomously in the event of reaching the battery energy level that is able to maintain all essential loads for a time guaranteeing safe recovery. | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.4c | The ultimate non-essential load disconnection circuit shall be implemented as a full hard-wired chain from sensor to actuator. | Requirement | PDR | RoD, A | [3][6] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.4d | The ultimate non-essential load disconnection circuit shall be one failure tolerant if centralised. | Requirement | PDR | RoD, A | [3][6] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.4e | The spacecraft design shall be such that in the event of an under-voltage condition on the bus, no failure is induced in the power subsystem or the loads during and when recovering from this under-voltage. | Requirement | PDR | RoD, A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.4f | After recovery as mentioned in 5.7.4e the loads shall be as follows: 1 all essential loads be supplied nominally; 2 all non-essential loads be in a known configuration that cannot create damage to any part of the spacecraft. | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.5a | For converters and regulators in closed loop control, the phase margin shall be at least 50 and the gain margin 6 dB for worst case end–of–life conditions with representative loading. | Requirement | PDR | RoD, A, T | [3][5] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.5b | For converters and regulators of the power subsystem, requirement 5.7.5a shall apply after any single failure. | Requirement | PDR | RoD, A, T | [3][5] | >> | // | // | >> | // | // | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.5c | The electrical zero–volt reference of isolated converters and regulators shall be isolated from the unit case by more than 10 k per converter. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - |
| 5.7.5d | The capacitance between the zero–volt reference of isolated converters and regulators and the unit case shall be less than 150 nF per converter. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - |
| 5.7.5e | If a switching converter is externally synchronized, it shall deliver output voltages within specification for any increase or decrease of synchronizing frequency, intermediate amplitude of synchronizing signal, phase jumps, or loss and recovery of the signal. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - |
| 5.7.5f | An analysis at unit level shall be performed to verify that no single failure generates an increase of conducted emission exceeding specified limit by more than 6 dB. | Requirement | PDR | A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.5g | If an increase of conducted emission exceeding specified limit by more than 6 dB is identified from the unit level analysis of 5.7.5f, then a system level analysis shall be conducted to ensure that compatibility is maintained. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.5h | A switching converter shall be able to reach nominal operation when the nominal input voltage is applied with any slope that can be provided by the power source and its associated impedance, connected to the switching converter. | Requirement | PDR | RoD, A, T | [3][4][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - |
| 5.7.6a | Inrush, under-voltage and a representative set of failures agreed with the customer for the payload interaction with the primary bus, shall be verified by test. | Requirement | PDR | RoD, A, T | [3][4][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.6b | No load shall generate a spurious response that can damage itself or any other equipment during bus voltage variation, up or down, at any ramp rate, and over the full range from zero to maximum bus voltage. | Requirement | PDR | RoD, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.6c | All current limiting devices and automatic switch-off circuits shall be monitored by telemetry. | Requirement | PDR | RoD, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.7.6d | The failure of the monitoring function of 5.7.6c shall not cause the protection elements to fail. | Requirement | PDR | RoD, A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1a | The primary power source shall be grounded to the spacecraft structure at the star reference point with a connection capable of sustaining the worst case fault current. | Requirement | PDR | RoD, A, INS | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1b | <<deleted, replaced by requirements 5.8.1q to 5.8.1v>> | - | PDR | RoD, A | [3] | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1c | <<deleted>> | Requirement | PDR | RoD, A, INS | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1d | All load paths shall include protection circuitry on the source side. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1e | No load shall be permanently disconnected from its power source as a consequence of an SEE. | Requirement | PDR | RoD, A | [3] | >> | >> | X | >> | >> | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1f | If fuses are used to protect main bus distribution lines, provision shall be made allowing easy replacement of blown or defective fuse. | Requirement | PDR | RoD, INS | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 5.8.1g | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1h | If fuses are used to protect main bus distribution lines, the design shall ensure that the power generation system can fuse them within less than 45 ms in case of load short circuit. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - |
| 5.8.1i | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1j | Equipment connected to independent, redundant power buses not protected at the source shall ensure that: 1 for unmanned missions, no single failure causes the loss of more than one power bus; 2for manned missions, two failures do not cause the loss of more than one power bus. | Requirement | PDR | RoD, A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1k | The stability of current limiters shall be ensured for the actual loads characteristics. | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1l | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1m | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1n | In case the distribution lines are protected by latching, foldback or periodically reset current limiters, it shall be verified by analysis or test that the transient current peaks at current limiter intervention are within the rated stress limits of the components used, for the worst case condition (minimum series impedance case). | Requirement | PDR | RoD, A, T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1o | When protection elements are in cascade, the closest one upstream from the anomaly should be the first to act. | Requirement | PDR | RoD, A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1p | When protections are used in cascade from a power source to a function to be supplied, the compatibility of these protections shall be ensured. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1q | Whenever two or more blocks are connected in cascade, the stability of the cascade between each source block and load block shall be analysed with the source and load impedances characterised in compliance with Figure 5-2. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1r | Whenever two or more blocks are connected in cascade, the power source being conveniently modelled with a Thevenin equivalent in compliance with Figure 5-3 and equation 1 for the sake of interface voltage stability analysis, the following two conditions shall be met: 1the difference between the phases of the source impedance and the load impedance is comprised in between [-130°,+130°] ±n*360° at those frequencies in which the load and the source impedance are equal in magnitude, 2the ratio of the magnitudes of the source and the load impedance is smaller than a factor 0,5 at those frequencies in which the difference between the phase of the source impedance and the load impedance is equal to -180°±n*360°. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1s | In alternative to requirements 5.8.1q and 5.8.1r, assuming that a power source is modelled with a Thevenin equivalent, stability criterion given in Impedance Specifications for Stable DC Distributed Power Systems, EEE transactions on power electronics, Vol. 17, no. 2, March 2002 shall be applied. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1t | In alternative to, and under the same assumptions of requirement 5.8.1r, the magnitude of the source impedance shall be smaller than the magnitude of the load impedance by at least a factor 10. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1u | Whenever two or more blocks are connected in cascade, the power source being conveniently modelled with a Norton equivalent in compliance with Figure 5-4 and equation 2 for the sake of interface current stability analysis, the following two conditions shall be met: 1 the difference between the phases of the load impedance phase and the source impedance is comprised in between [-130°,+130°] ±n*360° at those frequencies in which the load and the source impedance are equal in magnitude, 2the ratio between the magnitudes of the load and the source impedance is smaller than a factor 0,5 at those frequencies in which the difference between the load impedance phase and the source impedance phase is equal to -180°±n*360°. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1v | In alternative to, and under the same assumptions of requirement 5.8.1u, the magnitude of the load impedance shall be smaller than the magnitude of the source impedance by at least a factor 10. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1w | The stability of current limiters shall be verified by analysis under worst case conditions, and tested under a set of cases agreed with the customer. | Requirement | PDR | A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.1x | The requirement 5.8.1k shall be verified by worst case analysis, in accordance with ECSS-Q-ST-30 Annex J, and test. | Requirement | PDR | A,T | [3][5] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2a | No piece of harness shall be used to transfer mechanical loads. | Requirement | PDR | RoD, INS | [3] | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2b | With the exception of the solar array, routing of power lines shall be near ground. | Requirement | PDR | RoD, INS | [3] | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2c | With the exception of the solar array and electrical bus bars, harness power lines shall be such that each line is twisted with its return, when the structure is not used as a return. | Requirement | PDR | RoD, INS | [3] | X | X | // | X | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2d | The power distribution shall be protected in such a way that no over-current in a distribution wire can propagate a thermal failure to another wire. | Requirement | PDR | RoD, A | [3] | X | X | // | X | X | // | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2e | The harness inductance for a fully regulated bus, from the distribution node of the regulated bus to the load, shall be such that the break frequency is at least 5 000 Hz. | Requirement | PDR | RoD, A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2f | Harness shall be tested up to connector brackets under 500 V DC between conductors, conductors and structure, conductors and shielding. | Requirement | TRR | T | [5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2g | The harness restraining systems on the structure shall not bring about any stress at connector level. | Requirement | PDR | RoD, INS | [3] | X | X | // | X | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2h | There shall be umbilical and test connectors to provide external electrical interfaces. | Requirement | PDR | RoD, INS | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2i | Electrical and Safe and arm plugs shall be provided for disabling on ground hazard functions. | Requirement | PDR | RoD, INS | [3] | X | // | // | X | // | // | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.8.2j | If cross-strapping of redundant paths and circuits is carried out in the harness, then provisions of ECSS-E-ST-50-14 clause 4.2.5.2 shall apply. | Requirement | PDR | RoD, INS | [3] | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.9a | The design of electrical subsystems and payloads shall conform to ECSS-Q-ST-40. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.10a | For non pressurised and non potted high voltage equipment, the applicable pressure range when this equipment is on shall be specified. | Requirement | SRR | RoD | [6] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - |
| 5.10b | Non pressurised and non potted high voltage equipment shall be designed and manufactured to avoid discharge phenomena according to Paschen curves valid for its specified pressure range. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - |
| 5.10c | The field enhancement factors shall be ensured by the design. | Requirement | PDR | RoD, A | [3] | >> | // | // | >> | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - |
| 5.10d | For potted circuits, the glass transition point of the potting material shall be outside the temperature range of qualification. | Requirement | PDR | RoD, A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - |
| 5.10e | The design of high voltage equipment shall be such that worst case DC and AC field strengths are less than half of the values for which breakdown can occur. | Requirement | PDR | RoD, A | [3] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E | - | - | - |
| 5.11.1a | The requirements of this Clause 5 should be verified by the verification methods and at the verification points listed in Table 8-3. | Recommendation | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.11.2a | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.11.2b | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5.11.2c | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.1a | The supplier shall establish an overall EMC programme. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.1b | The EMC programme shall: 1 plan and verify that EMC technical criteria, mainly design and management controls are in place to achieve EMC; 2plan and accomplish the verification of spacecraft–level EMC. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.2a | As part of the EMC programme, an EMC control plan shall be written by the supplier for the PDR in conformance with the DRD in Annex A. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.2b | The EMC control plan shall apply to every item of equipment and subsystem in the project. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.2c | An EMC control plan shall be produced for every subsystem and equipment in answer to the requirements applicable at its level. | - | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.3a | For such programmes where EMC has been identified during phase A as critical for mission performance, the EMC programme shall include an EMC Advisory Board (EMCAB). | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.3b | The EMCAB shall: 1Ensure the timely and effective execution of the EMC programme under the general project manager. 2Respond to the problems related to EMC as they arise. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.2.3c | The supplier shall chair the EMCAB, with customer oversight. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.1.1a | Functional criticality of circuits for all equipment/subsystem circuits shall be identified in accordance with the following categories: 1Safety critical circuit - EMI problems that can result in loss of life or loss of space platform. This category comprises electro-explosive devices and their circuits. 2Mission critical circuit - EMI problems that can results in injury, damage to space platform, mission abort or delay, or performance degradation which unacceptably reduces mission effectiveness. 3Non critical circuit – Any problems that do not belong to categories 6.3.1.1a.1 and 6.3.1.1a.2. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.1.2a | The list of points where the margin is demonstrated (critical points) shall be submitted to the customer for approval.6.4.1c | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.1.3a | Electromagnetic interference safety margins shall be determined at critical points under all operating conditions. | Requirement | - | - | - | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.1.3b | The minimum margins shall be 20 dB for safety critical circuits, and 6 dB for mission critical circuits. | Requirement | - | - | - | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.2.2a | The electromagnetic environment seen by the spacecraft and the EMC requirements during the pre-launch and launch phases shall be according to those described in the applicable launchers user's manuals. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.2.3a | For protection of radiometric and communication bands, requirements on “Emissions” of “Transmitted signals” in ECSS-E-ST-50-05 clause 5.5 shall apply. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.2.4a | The space system shall be protected against both direct and indirect effects of lightning such that the mission is without degradation of performances after exposure to the lightning environment. | Requirement | - | - | - | // | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.3a | The space system shall be designed so that humans, fuels, explosive systems, and electronically actuated thrusters are not exposed to hazards of electromagnetic radiation present in the entire electromagnetic environment, including interference sources from possible external transmitters. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.1a | A spacecraft charging protection programme shall be produced by the supplier for the PDR, and submitted to the customer for approval, in conformance with ECSS-E-ST-20-06 clause 5 and Annex A. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.2a | The spacecraft charging protection programme shall include the preparation and maintenance of an analysis plan, and the preparation and maintenance of a test plan. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.2b | The performance shall be accomplished without the intervention of external control such as commands from a ground station. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.2c | The spacecraft charging protection programme shall include: 1surface electrostatic charging, 2threat from internal electrostatic charging of dielectric materials and isolated conducting items, due to the penetration of energetic electrons as defined in the environmental specification. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.3a | The space vehicle electrical subsystem and system may undergo an outage during an arc discharge if operation and performance returns to specified levels within 1a telemetry main frame period after onset of the discharge, or 2 within some other period defined by the customer. | Permission | - | - | - | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.3b | <<deleted>> | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.3c | Occurrence of an arc discharge during transmission of a command to the space vehicle from an external source as a ground station shall not result in any unintended action, whether the command is executed or not. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.3d | Provision shall be made such that the space vehicle is capable of receiving and executing subsequent commands. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.4.3e | Provision shall be made such that the space vehicle meets specified performances within the time period defined in clause 6.3.4.3a. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.5a | The space system shall operate without performance degradation in the electromagnetic environment due to on-board sources, intentional or not. | Requirement | - | - | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.6a | The spacecraft shall be RF compatible with all antenna-connected equipments and subsystems, the compatibility criteria being based on the mission performance and operability requirements. | Requirement | - | - | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.6b | When an inter-system interface is required, each system shall be RF compatible with all antenna-connected equipments and subsystems, the compatibility criteria being based on the mission performance and operability requirements. | Requirement | - | - | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.6c | The RF compatibility analysis, if used instead of test, shall include the effects of inter-modulation products. | Requirement | - | - | - | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.7.2a | In case the payload involves equipments sensitive to DC H-Field, the maximum acceptable DC magnetic field at their location from the rest of the spacecraft shall be specified by the customer because of the mission performance requirements. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.7.3a | On the basis of the attitude control requirements, the supplier shall derive magnetic requirements for the spacecraft so as to limit transient, diurnal and secular torques. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.7.3b | If magnetometers are used as part of the Spacecraft Attitude Control Subsystem, the maximum acceptable DC magnetic field at their location from the rest of the spacecraft shall be specified by the supplier because of the attitude control subsystem requirements and submitted to the customer approval. | Requirement | - | - | - | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.8.1a | A controlled ground reference concept, including the definition of circuit and unit categories shall be specified and agreed with the customer for the spacecraft prior to initial release of the EMC control plan. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.8.2a | A controlled ground reference concept, including the definition of circuit and unit categories shall be specified and agreed with the customer for the spacecraft prior to initial release of the EMC control plan. | Requirement | - | - | - | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.8.3a | Classification of cables, and cables shield shall be in conformance with the requirements specified in clauses 4.2.13 and 5.3.11 of ECSS-E-ST-20-07. | Requirement | - | - | - | X | X | - | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.3.9a | The EMC system design shall be performed in conformance with the requirements specified in clause 4.2 of ECSS-E-ST-20-07. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.4.1a | The verification plan shall be accomplished by the supplier in the frame of the EMC programme. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.4.1b | The verification plan shall be documented in the electromagnetic effects verification plan (EMEVP) in conformance with the DRDs in Annex B. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.4.1c | An electromagnetic effects verification report (EMEVR) in conformance with the DRD in Annex C shall be prepared by the supplier. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.4.2a | Safety margins for critical or EED circuit shall be demonstrated at system–level. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.4.2b | If the demonstration of safety margins is done by test, the spacecraft suite of equipment and subsystems shall be operated in a manner simulating actual operations, agreed with the customer. | Requirement | - | - | - | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6.4.3a | EMC verification shall be performed in conformance with the requirements on “Verification” in specified in ECSS-E-ST-20-07. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7.2.1.2.1a | All antenna terms used in all documentation (DDF, DJF, Test Report, Test Procedures, ICD and EIDP) shall follow the definitions found in IEEE 145:1993 ”Antenna Terms”. | Requirement | All reviews | RoD | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.1.2.2a | The following engineering process shall be applied: 1Perform an analysis of the mission requirements for RF signal transmission and reception for all systems and payload for all phases of the mission. 2Perform electrical, mechanical and thermal computer assessments to identify feasibility and performance margin for the whole antenna farm 3Establish performance budgets, including losses, simulation/measurement error and technology maturity margins for the whole antenna farm. 4Establish prediction, measurement and operational error/accuracy budgets for the whole antenna farm. 5Establish a plan for the maintenance and periodical review of the budgets established in requirement 7.2.1.2.2a.3 and 7.2.1.2.2a.4 during all project phases. | Requirement | 1. All reviews 2. PDR 3. PDR, CDR 4. PDR, CDR 5. PDR, CDR | 1. RoD 2. RoD 3. RoD, A 4. RoD, A 5. RoD, A | [3] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.1.3a | Antennas are in general single point failure elements; therefore their failure rates shall be agreed with the customer, specified and demonstrated. | Requirement | PDR CDR | A T | [3] [5] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.1a | The antenna category (7.2.2.2), composing elements (7.2.2.2.4), used technologies (7.2.2.4) and the performance parameters (7.2.2.5) shall be established at the beginning of the project phase B. | Requirement | PDR, CDR | RoD | [3] | X | - | - | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.1a | The antenna radiation pattern shall be characterised including the scattering effects of all surrounding structures. | Requirement | PDR, CDR | A, T | [3][5] | X | // | - | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.1b | If a number of TT&C antennas operate simultaneously, the combined radiation pattern shall be used in the performance evaluation. | Requirement | PDR, CDR | A, T | [3][5] | X | // | - | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.2a | The reflection and transmission properties (losses, depolarisation and diffusivity) of the reflecting or transmitting elements shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | X | // | - | X | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.2b | The effects of antenna support structures shall be quantified and the impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.2c | Deformations of reflector antennas, which parts are physically attached to different portions of the spacecraft platform, shall be quantified and their impact on antenna performance assessed. | Requirement | PDR, CDR | A, T | [3][5] | X | // | // | X | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.3a | The effect of the radiation of individual array element on the others shall be quantified and the impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.3b | The effects of antenna support structures on the main RF wave propagation path shall be quantified and the impact on performance assessed. | Requirement | PDR, CDR | A, T | [3][5] | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.3c | Deformations of array antennas, which parts are physically attached to different portions of the spacecraft platform, shall be quantified ant their impact on antenna performance assessed. | Requirement | PDR, CDR | A, T | [3][5] | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.2.4a | For array-fed reflector antennas clauses 7.2.2.2.2 (Reflector/Lens antennas) and 7.2.2.2.3 (Array antennas) shall apply. | Requirement | PDR | RoD | [3] | X | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1a | The isolated performances of radiating elements shall be characterised as part of the performance prediction of the whole antenna, at least up to the end of Phase B. | Requirement | PDR | A, T | [3] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1b | Whenever an antenna RF chain is attached to the radiating element its impact on the radiating element performances shall be assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1c | Deviations from the nominal geometry of the radiating element shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1d | It shall be demonstrated that the scattering of the radiation pattern of individual radiating elements does not affect the accuracy of all radiated performance measurement. | Requirement | PDR, CDR | A | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1e | Thermal dissipation of RF power shall be quantified and the impact on antenna performances assessed. | Requirement | PDR, CDR, QTR, AR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1f | Whenever a radiating element is used to route high power levels, | Requirement | PDR, CDR, QTR, AR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.1g | All metallic parts in a radiating element shall be connected to the equipment DC ground to avoid electrostatic discharge (ESD). | Requirement | PDR, CDR, QTR, AR | RoD, T | [1][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.2a | Reflective properties (losses, depolarisation, and diffusivity) of the materials and composites used shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.2b | The reflective and transmissive properties (losses, depolarisation, diffusivity) of the materials and composites used for polarisation and frequency selective reflectors shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.2c | Deviations from the nominal geometry of the reflector shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.3a | Reflective and transmissive properties of the materials and composites used for the lenses shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.3b | Deviations from the nominal geometry of the lens shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.3c | Measures to drain accumulated electric charges from all non conductive parts shall be implemented to avoid electrostatic discharge (ESD). | Requirement | PDR, CDR, QTR, AR | RoD, T | [3][4][1] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.3d | Any metallic parts shall be connected to the equipment DC ground to avoid electrostatic discharge (ESD). | Requirement | PDR, CDR, QTR, AR | RoD, T | [3][5][1] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.4a | The circuit characteristics of the RF BFN shall be independently quantified and their impact on antenna performances assessed at least up to CDR. | Requirement | PDR, CDR | A, T | [3][5] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.4b | Deviations from the nominal geometry of the RF BFN shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.4c | In all RF BFN structures having a central conductor (ideally insulated), the thermal power generated by Joule effect on the conductor itself shall be quantified and its impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.4d | For RF BFN, the applicable pressure range and gas properties shall be specified. | Requirement | PDR, CDR, QTR, AR | A, T, RoD | [3][5][8] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.4e | For RF BFN, the design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties. | Requirement | PDR, CDR, QTR, AR | A, T, RoD | [3][5][8] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.5a | The circuit characteristics of the antenna RF chain shall be independently quantified and their impact on antenna performances assessed at least up to CDR. | Requirement | PDR, CDR | A, T | [3][5] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.5b | The cumulative effects of wave propagation discontinuities along the whole antenna RF chain, including the radiating elements attached to it, shall be quantified and the impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.5c | For antenna RF chain the applicable pressure range and gas properties shall be specified. | Requirement | PDR, CDR, QTR, AR | A, T, RoD | [3][5][8] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.5d | For antenna RF chain the design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties. | Requirement | PDR, CDR, QTR, AR | A, T, RoD | [3][5][8] | - | // | // | - | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.6a | The possible scattering effects of the support structures shall be quantified and their impact on the antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.3.6b | Deviations from the nominal geometry of the supporting structure shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.1a | The level of passive inter-modulation products generated by the antenna shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR, QTR, AR | A, T, RoD | [3][5][8] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.1b | The impact of thermally-induced effects on the generation of passive intermodulation products shall be quantified and the impact on antenna performances assessed. | Requirement | PDR, CDR, QTR, AR | A, T, RoD | [3][5][8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.1c | Thermally induced changes of dimension and shape in all metallic antenna parts shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.2a | The impact of surface characteristics and finish on antenna performances shall be assessed. | Requirement | PDR, CDR | A | [3] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.2b | Thermally induced changes of dimension and shape in all composite and combined metal-composite antenna parts shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.2c | Measures to drain accumulated electric charges from composite parts shall be implemented to avoid electrostatic discharge (ESD). | Requirement | PDR, CDR, QTR,AR | T, RoD | [3][5][8] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.3a | The dielectric losses of plastic component in the RF power path shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.3b | Thermally induced changes of dimension and shape in all plastic and combined metal-plastic antenna parts shall be quantified and their impact on antenna performances assessed. | Requirement | PDR, CDR | A | [3] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.4.3c | Measures to drain accumulated electric charges from all plastic parts shall be implemented to avoid electrostatic discharge (ESD). | Requirement | PDR, CDR, QTR, AR | T, RoD | [3][5][8] | - | - | // | - | - | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.2.5a | The characterisation of antenna performances shall cover the following parameters. 1Coverage or Beam shape; 2Directivity; 3Electrical boresight or Beam pointing; 4Gain or Beam efficiency; 5Input impedance mismatch factor; 6Radiation pattern; 7Sense of polarization; 8Side lobe level; 9Polarisation purity or Axial ratio; 10Group delay; 11Noise temperature, for receive antennas; 12Phase centre position; 13Variations with frequency, angle (where applicable) and aging of all above parameters. | Requirement | PDR, CDR, QTR, AR | RoD | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.3.1a | Connectors or waveguide flanges at the antenna ports shall be demonstrated to have the specified power handling capabilities and impedance mismatch factors. | Requirement | PDR, CDR | A, T, RoD | [3][5][8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.3.1b | It shall be demonstrated that the generation of passive inter-modulation products that can occur at the antenna ports is below the specified limits agreed with the customer. | Requirement | PDR, CDR, QTR,AR | A, T, RoD | [3], [5], [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.3.1c | For antenna ports the applicable pressure range and gas properties shall be specified. | Requirement | - | - | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.3.1d | For antenna ports the design and manufacturing shall be performed to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties. | Requirement | - | - | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.3.2a | Electromagnetic interactions among the antenna and the surrounding spacecraft structure and appendages shall be quantified starting from Phase B, as a minimum, and their impact on antenna performances assessed. | Requirement | PDR, CDR | A, T | [3][5] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.3.2b | For all high-power applications, the risk of generation of passive inter-modulation products by the surrounding spacecraft structure and appendages shall be assessed starting from Phase B, as a minimum, and the impact on antenna performances assessed. | Requirement | PDR | A | [3] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.2.4a | The requirements of this clause 7.2 shall be verified by the verification methods, at the reviews, and recorded in the documents as specified in Table 8-3. | Recommendation | - | - | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.3.2.1a | All the components and equipments of the RF chain shall be able to stand the maximum specified operating RF power during its application in space with: 1no degradation of the component, 2no degradation of the RF signal including radiative losses, and 3with their thermal levels not exceeding those corresponding to the maximum available RF power at the maximum qualification temperature. | Requirement | TRB | 1. INS 2. T 3. T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.3.2.2a | Each element of the RF chain shall be designed and verified to withstand the maximum specified operating RF power levels plus safety margins agreed with the customer in the development phase at the maximum qualification temperature. | Requirement | PDR, CDR, TRB | A, T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.3.3.1a | All the components and equipments of the RF chain shall be free of any risk of Gas discharge (Corona) at the maximum specified operating RF power over the full pressure range during: 1the depressurization of the RF components and equipments at launch environmental conditions, 2the pressurization due to out-gassing of the spacecraft in orbit, 3ground testing at ambient pressure, and 4the pressurization of the spacecraft during planetary re-entry phases at the mission environmental conditions. | Requirement | PDR, CDR, TRB | 1. A, T 2. A, T 3. T 4. A, T | 1. [1][8] 2. [1][8] 3. [8] 4. [1][8] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.3.3.1b | For those components and equipments which design does not allow operating them over the full pressure range the following action shall be taken: 1specify the applicable pressure range and gas properties, 2ensure that the design and manufacturing is such to avoid discharge phenomena according to Paschen curves valid for its specified pressure range and gas properties. | Requirement | 1. SRR 2. PDR, CDR | RoD | [1] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.3.3.2a | RF components and equipments of the RF chain shall be designed and verified to withstand the maximum specified operating RF power levels plus safety margins agreed with the customer in the development phase. | Requirement | PDR, CDR, TRR, TRB | A, RoD, T | [1][8] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.3.4a | The following criteria shall be met for qualification for power handling and gas discharge: 1the RF component and equipment has no physical degradation, 2the RF component and equipment has no degradation of the RF performance during and after the test. | Requirement | TRB | 1. INS 2. T | [8] | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.2a | The acceptance level of interference caused by passive intermodulation products shall be agreed with the customer in the development phase. | Requirement | SRR, PDR, CDR | RoD | [1] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.2b | All the components of the RF chain shall be designed and manufactured to guarantee that the passive intermodulation products derived from the transmit carriers do not cause interference with any of the spacecraft receive bands or third party protected frequency bands during the operating temperature cycles. | Requirement | SRR, PDR, CDR | RoD, A, INS | [1] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.3a | All operating conditions shall be identified in which two or more transmit RF signals simultaneously illuminate or passed through a passive RF component, equipment or both. | Requirement | PDR, CDR, TRR | A, RoD | [1][8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.3b | For each of the conditions identified in 7.4.3a, the frequencies, number of carriers and power levels of these carriers shall be determined. | Requirement | PDR, CDR, TRR | A, RoD, | [1][8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.3c | An analysis shall be performed to establish all the passive intermodulation products falling within any of the spacecraft receive bands or third party protected frequency bands, for all combinations of frequency carriers up to the intermodulation order of 100. | Requirement | PDR, CDR, TRR | A, RoD, | [1][8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.4a | Testing at the lowest intermodulation order as identified in 7.4.3c shall be performed to ensure that the amplitudes of the passive intermodulation products are below the specified interference level. | Requirement | TRR, TRB | T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.4b | Passive Intermodulation tests shall be carried out on the flight hardware in the same configuration as it is during operational use. | Requirement | TRR, TRB | T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.4c | The test frequencies, number of carriers and power levels of these carriers shall be those as identified in 7.4.3b. | Requirement | TRR, TRB | T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.4d | Qualification testing shall be carried out 1on RF non radiative passive components, or equipments, or systems, over the full qualification temperature range, 2on RF radiative components, equipments or systems over a temperature range to be agreed with the customer, range which can be limited to ambient temperature. | Requirement | TRR, TRB | T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.4e | Acceptance testing shall be carried out on flight components, equipments or systems over an acceptance temperature range to be agreed with the customer, range which can be limited to ambient temperature. | Requirement | - | - | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.4.5a | The amplitude of each passive intermodulation product falling within any of the spacecraft receive bands or third party protected frequency bands shall be lower than the level specified in 7.4.2a. | Requirement | TRR, TRB | T | [8] | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| 7.5a | The requirements of the clauses 7.3 and 7.4 should be verified by the verification methods, at the reviews, and recorded in the documentation as specified in Table 8-3. | Recommendation | - | - | - | // | // | // | // | // | // | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | E |
| A.2.1a | The EMC control plan shall contain a description of the purpose, objective, content and the reason of prompting its preparation. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| A.2.1b | The EMC control plan shall list the applicable and reference documents to support the generation of the document. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| A.2.1c | The EMC control plan shall include any additional definition, abbreviation or symbol used. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| A.2.1d | The EMC control plan shall list the EMC requirements to be verified, covering at least the following areas:…. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| B.2.1a | The EMEVP shall contain a description of the purpose, objective, content and the reason of prompting its preparation. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| B.2.1b | The EMEVP shall list the applicable and reference documents to support the generation of the document. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| B.2.1c | The EMEVP shall include any additional definition, abbreviation or symbol used. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| B.2.1d | The EMEVP shall list the requirements of the plan, including: 1. methods to be used to select critical circuits, used to monitor conformance to degradation criteria and safety margins, including the definition of the method of selection; 2. procedures used for developing failure criteria and limits; 3. test conditions and procedures for all electronic and electrical equipment installed in or associated with spacecraft and sequence for operations during tests, including switching; 4.specific tolerance for particular measurement; 5. implementation and application of test procedures, including modes of operation and monitoring points for each subsystem or equipment; 6. use of approved results from laboratory interference tests on subsystems and equipment; 7. methods and procedures for data readout and analysis; 8. means of verifying design adequacy of spacecraft electrification; 9.means of simulating and testing electro–explosive subsystems and devices (EEDs); 10. verifying electrical power quality, and methods for monitoring DC and AC power busses; 11. test locations and descriptions of arrangements for simulating operational performance in cases where actual operation is impractical; 12. configuration of equipment and subsystems modes of operation to ensure victim equipment and subsystems are tested in most sensitive modes, while culprit equipment and subsystems are tested in noisiest mode(s); 13. details concerning frequency ranges, channels, and combinations to be specifically tested such as image frequencies, intermediate frequencies, local oscillator, transmitter fundamental and harmonically related frequencies, and including subsystem susceptibility frequencies identified during laboratory testing; 14.to precise parallel or series injection for conducted susceptibility test; 15.personnel to perform the test, including customer and supplier personnel at all levels, and quality representatives; 16.list of all test equipment to use, including a description of unique EMC instrumentation for stimulating and measuring electrical, electronic, and mechanical outputs of equipment and subsystems to be monitored during the test programme; 17.description of cables attached to the equipment under test; 18.definition of the line impedance stabilization network (values of internal components); 19. need for calibration and check of the measurement setup; 20. antennas to use for RF emission and susceptibility tests; 21. Method of switching ON for inrush current testing. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| B.2.1e | An intra–system compatibility culprit/victim test matrix shall be included in the EMEVP, showing all combinations of individual equipment/subsystems to be tested in order to verify overall intra–system compatibility; | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| B.2.1f | The description of the Step–by–step test procedures for operation of all matrix equipment shall be included in the EMEVP to support test execution. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| C.2.1a | The EMEVR shall contain a description of the purpose, objective, content and the reason of prompting its preparation. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| C.2.1b | The EMEVR shall list the applicable and reference documents to support the generation of the document. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| C.2.1c | The EMEVR shall include any additional definition, abbreviation or symbol used. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| C.2.1d | The EMEVR shall include: 1. identification of specific objectives, including applicable requirements and EMEVP references; 2. description of test article (e.g. configuration and drawings and photographs); 3. description of any fixes or configuration changes to article resulting from verification failures; 4. description of changes to cables attached to the equipment under test with respect to the EMEVP 5. summary of results including an executive summary stating degree of conformance to requirements; 6. description of any deviations from test facilities, analysis techniques or tools, and inspection aids in EMEVP; 7. description of any deviations from step–by–step procedures in EMEVP; 8. test set–up diagrams/photographs as appropriate; 9. list of test equipment, including calibration information; 10. recorded data or logs, including instrument readings, correction factors, and reduced results; methods of data reduction. 11. If value of data has been compromised due to test conditions, the reason and impact on results; 12. description of ambient and other test conditions. | Requirement | - | - | - | X | X | X | X | X | X | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| D.2.1a | The battery user manual shall contain the following information: 1. maximum ground storage life (where applicable before and after activation); 2. maximum period of non–use without special “wake–up” cycling; 3. range of battery temperatures and maximum durations during pre–launch and operational phases; 4. battery maintenance procedures during integration and pre–launch phases including case of launch delay; 5. storage procedure, range of storage temperature, cell discharge requirements before storage; 6. humidity and packaging constraints for storage; 7. maximum and minimum state of charge to be maintained during storage, requirements on individual shorting of cells, details of any trickle charge or periodic maintenance (e.g. minimum voltage checks and top-up charge to a maximum voltage in case a minimum cell voltage is reached) 8. reactivation procedure after storage; 9. handling and cell connecting procedures and precautions; 10. cell and battery safety related information; 11. transportation requirements. | Requirement | - | - | - | >> | >> | // | >> | >> | // | - | - | - | - | - | - | - | - | - | E | - | - | - | - | - | - | - | - |
| Verification points: SRR: System requirements review PDR: Preliminary design review CDR: Critical design review TRR: Test readiness review TRB: Test review board DRB: Delivery review board AR: Acceptance review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Verification methods: RoD: Review of design T: Test A: Analysis INS: Inspection NOTES: RoD includes review of documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

(normative)
EMC control plan - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-E-ST-20, requirement 6.2.2a.

Purpose and objective

The EMC control plan defines the approach, methods, procedures, resources and organization to design, produce and verify a product to operate within its specified electromagnetic environment and performance characteristics.

It provides the instruction for conducting all activities related to the management, the design requirements and the verification of the electromagnetic compatibility of all items of equipment and subsystems of a project.

Expected response

Scope and content

ECSS-E-ST-20_0020365

          - The EMC control plan shall contain a description of the purpose, objective, content and the reason of prompting its preparation.

ECSS-E-ST-20_0020366

The EMC control plan shall list the applicable and reference documents to support the generation of the document.

ECSS-E-ST-20_0020367

The EMC control plan shall include any additional definition, abbreviation or symbol used.

ECSS-E-ST-20_0020368

The EMC control plan shall list the EMC requirements to be verified, covering at least the following areas:

The EMC programme management:

responsibilities of customer and supplier at all levels, lines and protocols of communication, control of design changes;

planning of the EMC control program: facilities and personnel required for successful implementation of the EMC control program; methods and procedures of accomplishing EMC design reviews and coordination;

programme schedules: Integration of EMC program schedule and milestones within the program development master schedule.

System level performance and design requirements:

definition of electromagnetic and related environments;

definition of critical circuits;

allocation of design responses at system and subsystem and equipment levels;

antenna–to–antenna interference reduction analysis and technique;

magnetic moment upper limit required for AOCS;

magnetic cleanliness control plan (spacecraft with specific payloads);

magnetic budget;

establishment of a controlled grounding scheme;

assessment of possible fault currents;

wiring (including shielding and shield termination and categorization) practises;

electrical bonding;

material properties, effects of corrosion prevention and similar concerns on bonding and general EMC issues;

design criteria for alleviating effects of spacecraft charging and other electrification issues.

Subsystem and equipment EMI performance requirements and verification:

allocated EMI performance at the equipment level, including tailored equipment level requirements. The control plan is the vehicle for tailoring limits and test methods;

Conducted emission on power leads in the frequency domain

Inrush current peak(s) and profile on power leads

Common mode conducted emission on power and signal leads

Conducted emission on antenna ports

DC magnetic field emission

Radiated magnetic field emission in the low frequency range (scientific spacecraft)

Radiated electric field emission in the low frequency range (scientific spacecraft)

Radiated emission of RF electric field

Conducted susceptibility on power leads in differential mode

Conducted susceptibility on power and signal leads in common mode

Conducted susceptibility to transients on power leads

Radiated susceptibility to low frequency magnetic fields

Radiated susceptibility to RF electric fields

Susceptibility to electrostatic discharges

Summary of test results from subsystem and equipment level EMI tests. Any specification non–compliances judged to be acceptable is described in detail and the justifying rationale presented.

Electro–Explosive Devices (EED):

appropriate requirements (ECSS-E-ST-33-11 and ECSS-E-ST-20-07);

design techniques;

verification.

EMC analysis:

predictions of intra–system EMI and EMC based on expected or actual equipment and subsystem EMI characteristics;

design of solutions for predicted or actual interference situations;

Spacecraft level EMC verification, including outline of system–level EMC test plan, including rationale for selection of critical circuits for safety margin demonstration and instrumentation techniques for both critical and EED circuit sensitisation.

Special remarks

None.

(normative)
Electromagnetic effects verification plan (EMEVP) - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-E-ST-20, requirement 6.4.1b.

Purpose and objective

The electromagnetic effects verification plan (EMEVP) defines the approach, methods, procedures to verify electromagnetic effects.

The EMEVP provides the instruction for conducting all activities required to verify that the effects of the electromagnetic environment are compatible with the requirements of the project.

Expected response

Scope and content

ECSS-E-ST-20_0020369

          - The EMEVP shall contain a description of the purpose, objective, content and the reason of prompting its preparation.

ECSS-E-ST-20_0020370

          - The EMEVP shall list the applicable and reference documents to support the generation of the document.

ECSS-E-ST-20_0020371

          - The EMEVP shall include any additional definition, abbreviation or symbol used.

ECSS-E-ST-20_0020372

          - The EMEVP shall list the requirements of the plan, including:

methods to be used to select critical circuits, used to monitor conformance to degradation criteria and safety margins, including the definition of the method of selection;

procedures used for developing failure criteria and limits;

test conditions and procedures for all electronic and electrical equipment installed in or associated with spacecraft and sequence for operations during tests, including switching;

specific tolerance for particular measurement;

implementation and application of test procedures, including modes of operation and monitoring points for each subsystem or equipment;

use of approved results from laboratory interference tests on subsystems and equipment;

methods and procedures for data readout and analysis;

means of verifying design adequacy of spacecraft electrification;

means of simulating and testing electro–explosive subsystems and devices (EEDs);

verifying electrical power quality, and methods for monitoring DC and AC power busses;

test locations and descriptions of arrangements for simulating operational performance in cases where actual operation is impractical;

configuration of equipment and subsystems modes of operation to ensure victim equipment and subsystems are tested in most sensitive modes, while culprit equipment and subsystems are tested in noisiest mode(s);

details concerning frequency ranges, channels, and combinations to be specifically tested such as image frequencies, intermediate frequencies, local oscillator, transmitter fundamental and harmonically related frequencies, and including subsystem susceptibility frequencies identified during laboratory testing;

to precise parallel or series injection for conducted susceptibility test;

personnel to perform the test, including customer and supplier personnel at all levels, and quality representatives;

list of all test equipment to use, including a description of unique EMC instrumentation for stimulating and measuring electrical, electronic, and mechanical outputs of equipment and subsystems to be monitored during the test programme;

description of cables attached to the equipment under test;

definition of the line impedance stabilization network (values of internal components);

need for calibration and check of the measurement setup;

antennas to use for RF emission and susceptibility tests;

Method of switching ON for inrush current testing.

to item 4 “specific tolerance for particular measurement”: See also B.2.1e.and f.

ECSS-E-ST-20_0020373

An intra–system compatibility culprit/victim test matrix shall be included in the EMEVP, showing all combinations of individual equipment/subsystems to be tested in order to verify overall intra–system compatibility;

ECSS-E-ST-20_0020374

          - The description of the Step–by–step test procedures for operation of all matrix equipment shall be included in the EMEVP to support test execution.

Special remarks

None.

(normative)
Electromagnetic effects verification report (EMEVR) - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-E-ST-20, requirement 6.4.1c.

Purpose and objective

The electromagnetic effects verification report (EMEVR) provides reporting of all activities in relation with the verification of the effects of the electromagnetic environment.

The document is prepared for each project, based on the electromagnetic effects verification plan.

It then applies to every item of equipment and subsystem in the project.

Expected response

Scope and content

ECSS-E-ST-20_0020375

          - The EMEVR shall contain a description of the purpose, objective, content and the reason of prompting its preparation.

ECSS-E-ST-20_0020376

          - The EMEVR shall list the applicable and reference documents to support the generation of the document.

ECSS-E-ST-20_0020377

          - The EMEVR shall include any additional definition, abbreviation or symbol used.

ECSS-E-ST-20_0020378

          - The EMEVR shall include:

identification of specific objectives, including applicable requirements and EMEVP references;

description of test article (e.g. configuration and drawings and photographs);

description of any fixes or configuration changes to article resulting from verification failures;

description of changes to cables attached to the equipment under test with respect to the EMEVP

summary of results including an executive summary stating degree of conformance to requirements;

description of any deviations from test facilities, analysis techniques or tools, and inspection aids in EMEVP;

description of any deviations from step–by–step procedures in EMEVP;

test set–up diagrams/photographs as appropriate;

list of test equipment, including calibration information;

recorded data or logs, including instrument readings, correction factors, and reduced results; methods of data reduction.

If value of data has been compromised due to test conditions, the reason and impact on results;

description of ambient and other test conditions.

Special remarks

None.

(normative)
Battery user manual - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-E-ST-20, requirement 5.6.4b.

Purpose and objective

The battery user manual is a document generated by the manufacturer, that can be used by the customer for the procurement of cells and batteries.

Expected response

Scope and content

ECSS-E-ST-20_0020379

          - The battery user manual shall contain the following information:

maximum ground storage life (where applicable before and after activation);

maximum period of non–use without special “wake–up” cycling;

range of battery temperatures and maximum durations during pre–launch and operational phases;

battery maintenance procedures during integration and pre–launch phases including case of launch delay;

storage procedure, range of storage temperature, cell discharge requirements before storage;

humidity and packaging constraints for storage;

maximum and minimum state of charge to be maintained during storage, requirements on individual shorting of cells, details of any trickle charge or periodic maintenance (e.g. minimum voltage checks and top-up charge to a maximum voltage in case a minimum cell voltage is reached)

reactivation procedure after storage;

handling and cell connecting procedures and precautions;

cell and battery safety related information;

transportation requirements.

Special remarks

None.

Bibliography

| ECSS-S-ST-00 | ECSS system – Description, implementation and general requirements |
| --- | --- |
| ECSS-E-ST-10-02 | Space engineering – Verification |
| ECSS-E-ST-10-04 | Space engineering – Space environment |
| ECSS-E-ST-20-01 | Space engineering – Multipaction design and test |
| ECSS-E-ST-20-08 | Space engineering – Photovoltaic assemblies and components |
| ECSS-E-ST-20-20 | Space engineering – Electrical design and interface requirements for power supply |
| ECSS-E-ST-32 | Space engineering – Structural general requirements |
| ECSS-E-HB-20-02 | Space engineering – Li-ion battery testing handbook |
| ECSS-E-HB-20-05 | Space engineering – High voltage engineering and design handbook |
| ECSS-E-HB-20-20 | Space engineering – Guidelines for electrical design and interface requirements for power supply |
| ECSS-Q-ST-30 | Space product assurance – Dependability |
| ECSS-Q-ST-40 | Space product assurance – Safety |
| ECSS-Q-ST-70 | Space product assurance – Materials, mechanical parts and processes |
| ECSS-U-AS-10 | Adoption Notice of ISO 24113: Space systems - Space debris mitigation requirements |
| NASA-STD-8739.4 | Crimping, interconnecting cables, harnesses, and wiring |
| RNC-CNES-Q-70-511 | Spécification de conception et de contrôle des interconnexions filaires |
| JSC-20793 Rev B April 06 | Crew vehicle battery safety requirements |