Space product assurance

ECSS Secretariat

ESA-ESTEC

Requirements & Standards Division

Noordwijk, The Netherlands

Materials, mechanical parts and processes

Foreword

This Standard is one of the series of ECSS Standards intended to be applied together for the management, engineering, product assurance and sustainability in space projects and applications. ECSS is a cooperative effort of the European Space Agency, national space agencies and European industry associations for the purpose of developing and maintaining common standards. Requirements in this Standard are defined in terms of what shall be accomplished, rather than in terms of how to organize and perform the necessary work. This allows existing organizational structures and methods to be applied where they are effective, and for the structures and methods to evolve as necessary without rewriting the standards.

This Standard has been prepared by the ECSS Executive Secretariat, reviewed and approved by the ECSS Technical Authority.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Standard, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Requirements and Standards Division

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2019© by the European Space Agency for the members of ECSS

Change log

| ECSS-Q-70A 19 April 1996 | First issue |
| --- | --- |
| ECSS-Q-70B 14 December 2004 | Second issue |
| ECSS-Q-ST-70C 6 March 2009 | Third issue |
| ECSS-Q-ST-70C Rev.1 15 October 2014 | Third issue Revision 1 - |
| ECSS-Q-ST-70C Rev.2 15 October 2019 | Third issue Revision 2 Major changes with respect to ECSS-Q-ST-70C Rev.1 (15 October 2014) are: - Update of the Standard to include the European Regulation "Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH)" leading to the modification of requirement 4.1.3i by adding item 6 - Addition of term “REACH” in clause 3.3 “Abbreviated terms” Editorial changes: - Tables A-2, A-3 and Table B-2moved after the requirement. - Autonumbering in clause E.4 corrected. |

Table of contents

Change log	3

1 Scope	9

2 Normative references	10

3 Terms, definitions and abbreviated terms	12

3.1	Terms from other standards	12

3.2	Terms specific to the present standard	12

3.3	Abbreviated terms	13

3.4	Nomenclature	14

4 General requirements	15

4.1	MMPP management requirements	15

4.1.1	Overview	15

4.1.2	MMPP plan	15

4.1.3	MMPP Manager and MPCB	18

4.2	Management and consolidation of the activities	20

4.2.1	Overview	20

4.2.2	Establishing and processing of lists	20

4.2.3	Management of the lists	21

4.2.4	Supplier role and responsibilities	23

4.3	Technical constraints	23

4.4	Cleanliness and contamination control	24

4.5	Safety hazardous mechanical parts and materials	24

4.6	Optical, mechanical or electrical GSE hardware	25

4.7	Selection of space materials and processes	25

5 Materials control	26

5.1	Technical criteria for selection of materials	26

5.1.1	General	26

5.1.2	Temperature	26

5.1.3	Thermal cycling and thermo optical	27

5.1.4	Vacuum	27

5.1.5	Offgassing and toxicity	28

5.1.6	Flammability	28

5.1.7	Radiation	29

5.1.8	Electrical charge and discharge	29

5.1.9	Corrosion	29

5.1.10	Stress­corrosion	30

5.1.11	Fluid compatibility	30

5.1.12	Galvanic compatibility	31

5.1.13	Atomic oxygen	31

5.1.14	Micrometeoroids and debris	31

5.1.15	Moisture absorption and desorption	32

5.1.16	Mechanical contact surface effects: cold welding, fretting, wear	34

5.1.17	Life	34

5.1.18	Bacterial and fungus growth	34

5.1.19	Hydrogen embrittlement	35

5.2	Selection	35

5.2.1	General	35

5.2.2	Constraints	37

5.3	Declared materials list (DML)	38

5.4	Criticality analysis	38

5.4.1	Overview	38

5.4.2	Requirements	38

5.5	Evaluation and validation phases	38

5.5.1	General	38

5.5.2	Evaluation phase	39

5.5.3	Validation phase	39

5.5.4	Approval phase	40

5.5.5	Deviation request	40

5.6	Procurement of materials	40

5.6.1	Procurement specifications	40

5.6.2	Incoming inspection procedure	41

5.7	Use of materials	41

5.7.1	Validation status of materials	41

5.7.2	Traceability of materials	42

5.7.3	Packaging, storage, removal from storage	42

5.7.4	Limited­life materials before implementation	42

5.7.5	Limited­life materials after implementation	43

5.7.6	Materials nonconformances and alerts	43

5.7.7	Health and safety	43

6 Mechanical parts control	44

6.1	Selection of mechanical parts	44

6.2	Selection	44

6.3	Declared mechanical parts list (DMPL)	45

6.4	Criticality analysis	45

6.4.1	Overview	45

6.4.2	Requirements	45

6.5	Evaluation and qualification phases	45

6.5.1	General	45

6.5.2	Evaluation phase	46

6.5.3	Qualification phase	46

6.5.4	Approval phase	47

6.5.5	Deviation request	47

6.6	Procurement of mechanical parts	48

6.6.1	General	48

6.6.2	Procurement specification	48

6.6.3	Source inspection	48

6.6.4	Incoming inspection procedure	49

6.7	Use of mechanical parts	49

6.7.1	Qualification status of mechanical parts	49

6.7.2	Traceability of mechanical parts	49

6.7.3	Packaging, storage, removal from storage	49

6.7.4	Limited­life mechanical parts or parts subject to wearout	50

6.7.5	Mechanical parts nonconformances and alerts	50

7 Process control	51

7.1	Specifications or procedures	51

7.2	<< deleted >>	51

7.3	Process selection and training	51

7.4	Declared processes list (DPL)	53

7.5	Criticality analysis	53

7.5.1	Overview	53

7.5.2	Requirements	53

7.6	Evaluation and verification phase	54

7.6.1	General	54

7.6.2	Evaluation phase	54

7.6.3	Verification phase	55

7.6.4	Approval phase	55

7.6.5	Deviation request	56

7.7	Use of a process	56

7.7.1	Verification status of a process	56

7.7.2	Re­verification of a process	56

7.7.3	Implementation of a process	56

7.7.4	Traceability of processes	57

7.7.5	Process nonconformances and alerts	57

7.7.6	Mandatory inspection points (MIP)	57

7.7.7	Packaging, storage, removal from storage	57

Annex A (normative) Declared materials list (DML) - DRD	58

A.1	DRD identification	58

A.2	Expected response	58

Annex B (normative) Declared mechanical parts list (DMPL) - DRD	66

B.1	DRD identification	66

B.2	Expected response	66

Annex C (normative) Declared process list (DPL) - DRD	72

C.1	DRD identification	72

C.2	Expected response	72

Annex D (normative) Request for approval (RFA) - DRD	77

D.1	DRD identification	77

D.2	Expected response	77

Annex E (informative)  Relationship between materials, mechanical parts, processes activities and programme phases	82

E.1	Feasibility phase (phase A)	82

E.2	Preliminary definition phase (phase B)	82

E.3	Detailed definition and production phase (phase C or D)	82

E.4	Utilization phase (phase E)	83

Annex F (informative) MMPP documents delivery with respect to milestones	84

Bibliography	86

Figures

Figure 4-1: Materials, mechanical parts and processes flow chart	16

Figure A-1 : Example of a realized DML	65

Figure B-1 : Example of a realized DMPL	71

Figure C-1 : Example of realized DPL	76

Figure D-1 : Example of RFA (Page 1 of 2)	80

Figure D-1 : Example of RFA (Page 2 of 2)	81

Tables

Table 4-1: Steps to be taken to get approval for materials, mechanical parts and processes (MMPP)	18

Table 5-1 Compatible couples for bimetallic contacts	33

Table A-1 : Material group numbers	59

Table A-2 : Environmental code	62

Table A-3 : Size code	63

Table A-4 : Approval status	63

Table B-1 : Mechanical part group numbers	67

Table B-2 : Environmental code	69

Table B-3 : Approval status	70

Table C-1 : Process group numbers	73

Table C-2 : Approval status	75

Table F-1 : MMPP documents delivery w.r.t. milestones	85

# Scope

This Standard specifies the requirements and statements applicable to materials, mechanical parts and processes to satisfy the mission performance requirements.

This Standard also specifies the documentation requirements and the procedures relevant to obtaining approval for the use of materials, mechanical parts and processes in the fabrication of space systems and associated equipment.

This Standard covers the following:

management, including organization, reviews, acceptance status and documentation control;

selection criteria and rules;

evaluation, validation and qualification, or verification testing;

procurement and receiving inspection;

utilization criteria and rules.

The relationship between activities and programme phases is defined in Annex E.

The provisions of this Standard apply to all actors involved at all levels in the production of space systems. These can include manned and unmanned spacecraft, launchers, satellites, payloads, experiments, electrical ground support equipment, mechanical ground support equipment, and their corresponding organizations.

This standard may be tailored for the specific characteristics and constraints of a space project in conformance with ECSS-S-ST-00.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revisions of any of these publications do not apply. However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. For undated references the latest edition of the publication referred to applies.

| ECSS-S-ST-00-01 | ECSS system - Glossary of terms |
| --- | --- |
| ECSS-E-ST-20-06 | Space engineering - Spacecraft charging |
| ECSS-M-ST-80 | Space product assurance - Risk management |
| ECSS-Q-ST-10 | Space product assurance - Product assurance management |
| ECSS-Q-ST-10-04 | Space product assurance - Critical-item control |
| ECSS-Q-ST-10-09 | Space product assurance - Nonconformance control system |
| ECSS-Q-ST-20 | Space product assurance - Quality assurance |
| ECSS-Q-ST-40 | Space product assurance - Safety |
| ECSS-Q-ST-70-01 | Space product assurance - Cleanliness and contamination and control |
| ECSS-Q-ST-70-02 | Space product assurance - Thermal vacuum outgassing test for the screening of space materials |
| ECSS-Q-ST-70-04 | Space product assurance - Thermal testing for the evaluation of space materials, processes, mechanical parts and assemblies |
| ECSS-Q-ST-70-06 | Space product assurance - Particle and UV radiation testing of space materials |
| ECSS-Q-ST-70-09 | Space product assurance - Measurements of thermo-optical properties of thermal control materials |
| ECSS-Q-ST-70-21 | Space product assurance - Flammability testing for the screening of space materials |
| ECSS-Q-ST-70-22 | Space product assurance - Control of limited shelf­life materials |
| ECSS-Q-ST-70-29 | Space product assurance - Determination of offgassing products from materials and assembled articles to be used in a manned space vehicle crew compartment |
| ECSS-Q-ST-70-36 | Space product assurance - Material selection for controlling stress­corrosion cracking |
| ECSS-Q-ST-70-37 | Space product assurance - Determination of the susceptibility of metals to stress­corrosion cracking |
| ECSS-Q-ST-70-71 | Space product assurance - Materials, processes and their data selection |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this Standard, the terms and definitions from ECSS-S-ST-00-01 apply, in particular for the following terms:

material

## Terms specific to the present standard

critical mechanical part

mechanical part that requires specific attention or control due to fracture mechanics aspects and limited­life aspects, or with which the supplier has no previous experience of using the mechanical part in the specific application and environment or that are new or non­qualified, or that has caused problems during previous use that remain unresolved

critical process

process new to an individual company or non­verified for the application in question or has caused problems during previous use that remain unresolved

critical material

material that is new to an individual company or non­validated for the particular application and environment, or that has caused problems during previous use that remain unresolved

mechanical part

piece of hardware which is not electrical, electronic or electromechanical, and which performs a simple elementary function or part of a function in such a way that it can be evaluated as a whole against expected requirements of performance and cannot be disassembled without destroying this capability

process

set of inter­related resources and activities which transforms a material or semi­finished product into a semi­finished product or final product

request for approval (RFA)

document with which the supplier or user asks the competent body for permission to use a critical material, part or process

special process

process where quality cannot be completely ensured by visual inspection of the end article only

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| AA | Aluminium Association |
| AOCS | attitude and orbit control system |
| ATOX | atomic oxygen |
| AISI | American Iron and Steel Institute |
| CDA | Copper Development Association |
| CDR | critical design review |
| CFRP | carbon fibre reinforced polymer |
| CI | configuration item number (as per project definition) |
| DML | declared material list |
| DMPL | declared mechanical part list |
| DPL | declared process list |
| DRD | document requirements definition |
| EEE | electrical, electronic and electromechanical |
| ESA | European Space Agency |
| ESMDB | European Space Materials Database |
| GOX | gaseous oxygen |
| GSE | ground support equipment |
| LEO | low Earth orbit |
| LOX | liquid oxygen |
| MIP | mandatory inspection point |
| MMPP | Materials, mechanical parts and processes |
| MMPDS | Metallic Materials Properties Development and Standardisation |
| MPCB | Material, Mechanical Parts and Processes Control Board |
| NASA | National Aeronautics and Space Administration |
| NCR | nonconformance report |
| NRB | nonconformance review board |
| PA | product assurance |
| PDR | preliminary design review |
| PID | process identification document |
| PMP | parts, materials, processes |
| QR | qualification review |
| QRR | qualification review report |
| REACH | Registration, Evaluation, Authorisation and Restriction of Chemicals (European Regulation) |
| RFA | request for approval |
| RFD | request for deviation |
| SCC | stress­corrosion cracking |

## Nomenclature

The following nomenclature applies throughout this document:

- The word “shall” is used in this Standard to express requirements. All the requirements are expressed with the word “shall”.

The word “should” is used in this Standard to express recommendations. All the recommendations are expressed with the word “should”.

It is expected that, during tailoring, recommendations in this document are either converted into requirements or tailored out.

The words “may” and “need not” are used in this Standard to express positive and negative permissions, respectively. All the positive permissions are expressed with the word “may”. All the negative permissions are expressed with the words “need not”.

The word “can” is used in this Standard to express capabilities or possibilities, and therefore, if not accompanied by one of the previous words, it implies descriptive text.

In ECSS “may” and “can” have completely different meanings: “may” is normative (permission), and “can” is descriptive.

The present and past tenses are used in this Standard to express statements of fact, and therefore they imply descriptive text.

# General requirements

## MMPP management requirements

### Overview

The general MMPP activity within the framework of a project is summarized in Figure 4-1.

### MMPP plan

ECSS-Q-ST-70_0490001

The supplier shall prepare, maintain and implement a MMPP plan, as part of the overall PA plan in conformance with DRD from Annex A of ECSS-Q-ST-10 and this Standard or as a separate document.

ECSS-Q-ST-70_0490002

The MMPP plan shall be submitted to the customer for approval.

ECSS-Q-ST-70_0490185

Figure 4-1: Materials, mechanical parts and processes flow chart

Figure 4-1: Materials, mechanical parts and processes flow chart (continued)

ECSS-Q-ST-70_0490186

Table 4-1: Steps to be taken to get approval for materials, mechanical parts and processes (MMPP)

| Phase | Materials |  | Mechanical parts |  | Processes |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Step | Comments | Step | Comments | Step | Comments |
| Critical Analysis | 1 |  | 1 |  | 1 |  |
| Evaluation (usually by test methods defined by ECSS standards) | 2 | Critical materials are tested, e.g. outgassing, SCC, flammability. | 2 | Mechanical parts are tested by, for example, vibration, thermal analysis, off­gassing and life test. | 2 | Critical processes are evaluated by testing “technology samples” including all, for example, electrical interconnection processes and painting, adhesive bonding, welding. |
| Verification | Not applicable |  | Not applicable |  | 3 | Verification tests usually defined in ECSS standards |
| Validation | 3 |  | Not applicable |  | Not applicable |  |
| Qualification | Not applicable |  | 3 |  | Not applicable |  |
| Approval |  | By RFA (Annex D) or DML |  | By RFA (Annex D) or DMPL/DPL |  | By RFA (Annex D) or DPL |
| NOTE 1 Project approval is always by means of the request for approval (RFA) form and the projects’ declared materials list (DML), declared mechanical parts list (DMPL) and declared processes list (DPL). NOTE 2 The details for approvals of MMPP lists are contained in this Standard. NOTE 3 To summarize: Materials are validated. Mechanical parts are qualified. Processes are verified. In addition: Skills training schools are ESA-certified. Outside test or evaluation laboratories are certified by agency or company audits. Operators for special processes are trained, certified and monitored. |  |  |  |  |  |  |

### MMPP Manager and MPCB

ECSS-Q-ST-70_0490003

<< deleted >>

ECSS-Q-ST-70_0490004

<< deleted >>

ECSS-Q-ST-70_0490194

The supplier shall appoint a MMPP manager.

ECSS-Q-ST-70_0490195

The MMPP Manager shall ensure that the Materials, Mechanical Parts and Processes used to manufacture a spacecraft or a launcher satisfy both the ground and flight functional requirements and constraints of the project.

ECSS-Q-ST-70_0490196

To obtain the validation status for materials and qualification status for parts and verification status for processes, the MMPP manager shall present to the customer activities which were performed in conformance with requirement 4.1.3d together with the results obtained.

ECSS-Q-ST-70_0490197

The MMPP manager shall organize Materials, Mechanical Parts and Processes Control Board (MPCB) at all suppliers with the task to review and approve the Materials, Mechanical Parts and Processes as specified in the Clauses 5, 6, and 7.

ECSS-Q-ST-70_0490198

The MPCB activity shall start not later than at PDR.

ECSS-Q-ST-70_0490199

The MMPP Manager shall agree with the customer on the MPCB’s activities at PDR.

ECSS-Q-ST-70_0490200

Minimum tasks of the MPCB shall be as follows:

Coordination of the initiation and approval of RFA’s in conformance with DRD from the Annex D by involving the relevant technical discipline.

Review and approval of test programme and related results.

Review of preliminary Declared Materials, Mechanical Parts and Processes Lists and of any available evidence to support the approval, by the PDR.

Review and approval of Declared Materials, Mechanical Parts and Processes Lists and of the evidence for the approval by the CDR.

Review and approval of any change to the approved Declared Materials, Mechanical Parts and Processes Lists.

            - Identify and mitigate the risks linked to obsolescence of materials, processes, or mechanical parts at all levels of the customer-supplier chain.

1:	REACH legislation is an example of a source for obsolescence

2:	Guidelines for obsolescence management for materials, mechanical parts and processes are provided in ECSS-Q-HB-70-23.

## Management and consolidation of the activities

### Overview

The relationship between materials and processes activities and programme phases is shown in Annex E.

### Establishing and processing of lists

ECSS-Q-ST-70_0490005

Each supplier and sub­supplier shall establish, collect, review and deliver the declared materials, mechanical parts and processes lists including all the items intended for use in the flight equipment.

ECSS-Q-ST-70_0490006

The lists specified in 4.2.2a shall reflect the current design at the time of issue.

ECSS-Q-ST-70_0490007

The lists specified in 4.2.2a shall contain the materials, mechanical parts and processes used in the current design to:

demonstrate compliance with all requirements of the programme;

verify the results of equipment supplier activities;

control and monitor the status of materials, mechanical parts and processes in conformance with programme milestones.

See Annex F.

ECSS-Q-ST-70_0490269

The following constraints should apply:

maximum use of the materials and processes specified in approved data sources and items already approved on similar projects;

use of project related preferred lists, if available.

For approved data sources see example of ESMDB database in the Annex A of the ECSS-Q-ST-70-71.

ECSS-Q-ST-70_0490009

The following constraints shall be assessed:

requirements originating from the initial technical specification.

programmatic project requirements and conditions.

ECSS-Q-ST-70_0490010

An analysis of the criticality of preliminary lists specified in the 4.2.2a shall be performed such that, after checking the conformity of the materials, mechanical parts and processes, against all the project requirements, allows them to be classified into three categories:

Critical items, subject to evaluation, validation, qualification, or verification programmes.

Items that are not critical but which do not conform to one or more project requirements.

Non­critical items.

ECSS-Q-ST-70_0490011

For items classified as critical in conformance with 4.2.2f.1 a request for approval shall be submitted in conformance with Annex D.

ECSS-Q-ST-70_0490270

For items classified as not critical but which are not in conformance with one or more project requirements in accordance with 4.2.2f.2 a justified deviation request should be drafted.

### Management of the lists

ECSS-Q-ST-70_0490012

The supplier shall document all materials in the Declared materials list in conformance with Annex A.

ECSS-Q-ST-70_0490013

The supplier shall document all mechanical parts in the Declared mechanical parts list in conformance with Annex B.

ECSS-Q-ST-70_0490014

The supplier shall document all processes in the Declared process list in conformance with Annex C.

ECSS-Q-ST-70_0490015

The supplier shall process the lists of lower level suppliers to ensure exchangeabililty, traceability searchability, sortability, storability and retrievability for that set of lists, before submitting it to the customer.

ECSS-Q-ST-70_0490016

The lists specified in the requirement 4.2.3a, 4.2.3b and 4.2.3c shall be updated during the course of the project.

ECSS-Q-ST-70_0490017

The preliminary lists specified in the requirement 4.2.3a, 4.2.3b and 4.2.3c shall include the items from suppliers’ preliminary needs.

They are used to identify critical items  (available for the PDR).

ECSS-Q-ST-70_0490018

The as­designed lists shall include the items from the baselines various design files, available for CDR.

ECSS-Q-ST-70_0490019

Any change after CDR or QR shall be reflected in the list and shall be in accordance with Figure 4-1 (Part 2).

1	The MMPP manager is responsible within the programme to ensure that all the information needed is given and that the approval status is consistent with technical and scheduling objectives and data is exchangeable.

2	Where no project requirements exist for a separate DMPL, the mechanical parts can be entered into a separate section of the DML.

3	The materials of, for example, screws and nuts that are made up of a few materials can be listed in the DMPL. The materials (metals and plastics) of complex parts can be listed in the DML with, for example, outgassing, toxicity, flammability, corrosion and stress corrosion values and reference to the DMPL item.

ECSS-Q-ST-70_0490201

The supplier shall establish, collect, review and deliver the declared materials, mechanical parts and processes lists in an electronic format in conformance with Annex A, Annex B, and Annex C.

ECSS-Q-ST-70_0490202

The supplier shall demonstrate that the lists specified in the requirements 4.2.3a, 4.2.3b and 4.2.3c have been formally approved prior to their delivery to the customer.

### Supplier role and responsibilities

ECSS-Q-ST-70_0490020

The supplier shall perform the following tasks:

obtaining the correct and complete lists from lower level suppliers;

providing provisional and, later, definitive approval for each list;

submitting the project declared lists for approval prior to initiation of the hardware phase, before CDR.

ECSS-Q-ST-70_0490021

The lists specified in the requirement 4.2.3i by the suppliers shall include all the information described in this Standard.

ECSS-Q-ST-70_0490022

Amendments to the lists specified in the requirement 4.2.3 a, b and c shall be implemented through established change procedures.

ECSS-Q-ST-70_0490023

The following documentation shall be delivered to the customer upon request:

RFA with reference and issue in conformance with DRD in Annex D;

evaluation reports;

deviation requests.

ECSS-Q-ST-70_0490024

The material, mechanical parts or process justification files shall be made available to the customer upon request either on the supplier site, or by any other process agreed by both parties.

For example, by non-disclosure agreement.

## Technical constraints

ECSS-Q-ST-70_0490025

Mechanical parts, and materials and processes shall satisfy the mission’s functional requirements and constraints.

ECSS-Q-ST-70_0490026

Mechanical parts, and materials and processes shall satisfy both ground environment and flight requirements and constraints.

Examples are:

Ground environment constraints: manufacture, tests, storage, maintenance, transport and integration

Flight constraints: launch and orbit.

ECSS-Q-ST-70_0490027

<< deleted >>

ECSS-Q-ST-70_0490028

The estimated availability of the parts and products obtained from materials and processes used shall be compatible with the space system’s life cycle.

Examples of space system’s life cycle are tests, storage and mission.

## Cleanliness and contamination control

ECSS-Q-ST-70_0490029

The supplier shall establish and maintain a contamination and cleanliness control programme including, as a minimum:

cleaning procedures, and

cleanliness monitoring procedures or methods.

ECSS-Q-ST-70_0490030

The risks of chemical or particle pollution generated by parts, materials or processes used shall be identified and reduced as specified in ECSS-M-ST-80 clause 7.2, in conformance with mission requirements.

Cleanliness or contamination analysis is example of mission requirements.

ECSS-Q-ST-70_0490031

For cleanliness­ or contamination­critical applications, a chemical and particle requirement specification and a specific cleanliness control plan shall be established in conformance with DRDs from Annex A and Annex B of ECSS-Q-ST-70-01.

## Safety hazardous mechanical parts and materials

ECSS-Q-ST-70_0490032

Mechanical parts and materials with hazardous characteristics shall be identified, managed and processed in conformance with requirements from clause 5 to clause 8 of ECSS-Q-ST-40.

## Optical, mechanical or electrical GSE hardware

ECSS-Q-ST-70_0490033

When optical, mechanical or electrical GSE materials are used in thermal vacuum or interfacing with flight hardware, degradation shall be assessed.

For example, contamination, surface degradation, electro­mechanical and chemical effects.

## Selection of space materials and processes

ECSS-Q-ST-70_0490203

          - For the selection of space materials and processes, ECSS-Q-ST-70-71 shall be applied.

# Materials control

## Technical criteria for selection of materials

### General

ECSS-Q-ST-70_0490204

Material design data shall be generated for the intended service conditions using test and analysis procedures approved by the customer.

ECSS-Q-ST-70_0490205

Material properties shall be compatible with the environments to which they are exposed during terrestrial testing prior to launch, and during the mission.

The requirements of clause 5 apply when the environmental conditions of the mission require their application. The specific requirements, test methods and acceptance or rejection criteria are presented in the ECSS-Q-ST-70 series of documents.

### Temperature

ECSS-Q-ST-70_0490034

Material properties shall be compatible with the thermal environment to which they are exposed.

Examples of thermal environment are also the passage through transition temperatures, e.g. phase transitions, ductile-brittle transition temperatures for metals, glass transition (Tg) for polymer materials, and environmental factors which affect these properties, such as moisture.

### Thermal cycling and thermo optical

ECSS-Q-ST-70_0490035

Materials subject to thermal cycling shall be assessed for their ability to withstand induced thermal stress.

ECSS-Q-ST-70_0490036

The following materials shall be tested in conformance with requirements from clause 5 of ECSS-Q-ST-70-04:

Materials susceptible to thermal vacuum effect.

Materials of unknown characteristics in respect to thermal vacuum.

For a non-exhaustive list of materials specified in the requirement 5.1.3b.1, see ECSS-Q-ST-70-04 Clause 1.

ECSS-Q-ST-70_0490037

Materials subject to thermal cycling except the case specified in the requirement 5.1.3b, shall be tested in conformance with a procedure approved by the customer.

ECSS-Q-ST-70_0490206

Thermo optical properties shall be evaluated in conformance with requirement from clause 4 of ECSS-Q-ST-70-09.

ECSS-Q-ST-70_0490207

Directional effects due to manufacturing or processing shall be evaluated via dedicated testing to be agreed with the customer.

### Vacuum

ECSS-Q-ST-70_0490038

<< deleted >>

ECSS-Q-ST-70_0490039

Outgassing screening tests shall be carried out in conformance with requirements from clause 5 of ECSS-Q-ST-70-02.

The screening process applied depends on the application

ECSS-Q-ST-70_0490208

All organic materials for use in space systems shall be evaluated to determine their outgassing characteristics.

ECSS-Q-ST-70_0490271

The need for retest outgassing characteristics of materials used for an extended period of time at a temperature higher than 50 C should be mutually agreed with the customer.

When dynamic outgassing tests are required see ECSS-Q-TM-70-52.

### Offgassing and toxicity

ECSS-Q-ST-70_0490040

Spacecraft and associated equipment shall be manufactured from materials and by processes that do not cause hazard to personnel or hardware, either on the ground or in space.

ECSS-Q-ST-70_0490041

Requirements from clause 4 of ECSS-Q-ST-70-29 shall apply for the characterization of offgassing products.

MIL-HDBK-454 test method is not space relevant.

ECSS-Q-ST-70_0490209

For materials for the use in manned compartments of a spacecraft or space segment elements, offgassing and toxicity analysis shall be performed.

ECSS-Q-ST-70_0490210

The levels of offgassing and toxicity specified in the requirement 5.1.5c shall be agreed with the customer.

### Flammability

ECSS-Q-ST-70_0490042

The materials flammability resistance, shall be evaluated in conformance with requirements from the clause 5 of ECSS-Q-ST-70-21 for the most hazardous environment envisaged for their use, and be applicable to:

unmanned spacecraft or space segment elements launched by a manned space transportation system when powered on launch, and

manned spacecraft and space segment elements.

See also NASA STD-6001.

### Radiation

ECSS-Q-ST-70_0490043

Materials exposed to radiation shall be assessed in conformance with requirements from clause 5 of ECSS-Q-ST-70-06 to determine their resistance to the radiation dosage expected during the mission.

ECSS-Q-ST-70_0490211

Evaluation of materials resistance to radiation shall include the combined effects of particle radiation and ultraviolet radiation in the normal space environment, along with any mission-specific radiation levels.

ECSS-Q-ST-70_0490212

The effect of bleaching due to vacuum or air recovery shall be evaluated, in case ex-situ measurements are performed.

ECSS-Q-ST-70_0490213

In case synergistic testing is not possible it shall be proven that synergistic effects caused by radiation and temperature are not degrading the materials properties.

ECSS-Q-ST-70_0490214

In case technical limits prevent the synergistic testing approach, it shall be so justified, and the justification for sequential testing shall be provided.

For example when different acceleration factors are required for particle and UV radiation.

### Electrical charge and discharge

ECSS-Q-ST-70_0490044

External surfaces of the spacecraft shall be conductive, interconnected and grounded to the spacecraft structure to avoid the build­up of differential charges in conformance with requirements from clause 6 to clause 10 of ECSS-E-ST-20-06.

### Corrosion

ECSS-Q-ST-70_0490045

For all materials that come into contact with atmospheric gases, cleaning fluids or other chemicals, it shall be demonstrated that the degradation of properties during their anticipated service­life does not prevent to meet the performance and integrity requirements.

ECSS-Q-ST-70_0490215

          - All mechanical parts, assemblies and equipment, including spares, shall be finished to provide protection from corrosion.

This applies equally to fasteners and other fixing devices, such as insert systems.

### Stress­corrosion

ECSS-Q-ST-70_0490046

Materials used for structural and load bearing applications (i.e. subject to tensile stress) shall be selected in accordance with requirements of clauses 5.2.1, 5.2.3 and 5.2.4 of ECSS-Q-ST-70-36.

ECSS-Q-ST-70_0490047

Any material not covered by cases specified in requirement 5.1.10a shall be tested in conformance with requirements from clause 5 of ECSS-Q-ST-70-37.

### Fluid compatibility

ECSS-Q-ST-70_0490048

Materials within the system exposed to reactive fluids, both directly and as a result of single point failures when failure propagation effects cause hazardous operation of interfacing hardware shall be compatible with that fluid in their application.

Example of reactive fluids are liquid oxygen (LOX) and gaseous oxygen (GOX).

ECSS-Q-ST-70_0490049

<< deleted >>

ECSS-Q-ST-70_0490216

The compatibility of materials which are or can come into contact with LOX or GOX shall be evaluated except the case specified in the requirement 5.1.11d.

See NASA-STD-6001 Tests No. 13 and No. 14.

ECSS-Q-ST-70_0490217

In case no compatibility data are available, tests shall be performed for reactive fluids other than oxygen.

See NASA-STD-6001 Test No. 15.

### Galvanic compatibility

ECSS-Q-ST-70_0490050

When bimetallic contacts are used, the choice of the pair of metallic materials used shall be agreed with the customer.

This also includes metal­to­conductive fibre­reinforced materials contacts.

ECSS-Q-ST-70_0490051

Galvanic compatibilities shall be selected in conformance with Table 5-1.

ECSS-Q-ST-70_0490052

Materials not listed in Table 5-1 shall be evaluated in a flight­simulated configuration using an accelerated environment to be agreed by the customer.

### Atomic oxygen

ECSS-Q-ST-70_0490053

All materials for use on the external surfaces of spacecraft for use in Low Earth Orbit (LEO) altitudes, between 200 km and 700 km, shall be evaluated for their resistance to atomic oxygen (ATOX).

The flux level varies with altitude, velocity vector and solar activity. Fluence levels vary with the duration of exposure.

ECSS-Q-ST-70_0490054

Test procedures shall be subject to the approval by the customer.

ECSS-Q-ST-70_0490218

The effect of ATOX on thermo optical properties including specularity shall be evaluated.

Data related to ATOX are presented in ECSS-E-ST-10-04.

### Micrometeoroids and debris

ECSS-Q-ST-70_0490055

The effect of impacts by micrometeoroids and debris on materials shall be reviewed and assessed on a case by case basis.

ECSS-Q-ST-70_0490219

Use of materials shall comply with safety evaluation and assessment results concerning design and application criteria or details.

ECSS-Q-ST-70_0490220

          - Micrometeoroids and debris analysis and test procedures shall be subject to the approval by the customer.

### Moisture absorption and desorption

ECSS-Q-ST-70_0490056

Moisture absorption shall be prevented during manufacture and storage of hygroscopic materials.

ECSS-Q-ST-70_0490187

Table 5-1 Compatible couples for bimetallic contacts

| Pure metals and alloys in alphabetical order (including carbon) | Aluminium­Copper alloys | Al (pure), Al­Zinc alloys | Cadmium | Cast iron (austenic) | Chromium | Copper, Brasses | Cupro­Nickel, Al­bronzes, Si­bronzes | Gold, Platinum, Carbon, Rhodium | Gun­metal (CuZn10 alloy), P­bronzes, Sn­bronzes | Magnesium | Nickel, Monel, Inconel, Nickel/Molybdenum­alloys | Silver | Sn­Pb alloys (all), Tin, Lead | Stainless steel 18/8 (300 series) | Stainless steel 13Cr (400 series) | Steel (carbon, low alloy), Cast iron | Titanium and Ti­alloys | Zinc, Beryllium |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aluminium­Copper alloys |  | 1 | 1 | 3 | 3 | 3 | 3 | 3 | 3 | 2 | 2 | 3 | 1 | 2 | 2 | 3 | 2 | 2 |
| Al (pure) Al­Zinc alloys |  |  | 1 | 3 | 3 | 3 | 3 | 3 | 3 | 2 | 3 | 3 | 2 | 3 | 3 | 3 | 3 | 2 |
| Cadmium |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 0 | 1 | 1 | 2 | 2 | 2 |
| Cast iron (austenic) |  |  |  |  | 1 | 1 | 1 | 2 | 1 | 3 | 1 | 2 | 1 | 1 | 1 | 2 | 1 | 3 |
| Chromium |  |  |  |  |  | 1 | 0 | 0 | 1 | 3 | 1 | 0 | 2 | 0 | 0 | 2 | 0 | 3 |
| Copper, Brasses |  |  |  |  |  |  | 0 | 2 | 0 | 3 | 1 | 1 | 2 | 1 | 1 | 3 | 0 | 3 |
| Cupro­Nickel Al­bronzes Si­bronzes |  |  |  |  |  |  |  | 2 | 0 | 3 | 1 | 1 | 2 | 2 | 1 | 3 | 0 | 3 |
| Gold Platinum, Carbon Rhodium |  |  |  |  |  |  |  |  | 2 | 3 | 2 | 0 | 3 | 0 | 1 | 3 | 0 | 3 |
| Gun­metal(CuZn10 alloy) P­bronzes Sn­bronzes |  |  |  |  |  |  |  |  |  | 3 | 1 | 1 | 1 | 0 | 0 | 3 | 0 | 3 |
| Magnesium |  |  |  |  |  |  |  |  |  |  | 3 | 3 | 2 | 3 | 3 | 3 | 3 | 3 |
| Nickel Monel Inconel Nickel/Molybdenum­alloys |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 | 1 | 0 | 2 | 1 | 3 |
| Silver |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 0 | 0 | 3 | 0 | 3 |
| Sn­Pb alloys (all) Tin, Lead |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 3 | 1 |
| Stainless steel 18/8 (300 series) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 3 | 0 | 3 |
| Stainless steel 13Cr (400 series) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 0 | 3 |
| Steel (carbon, low alloy) Cast iron |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 3 |
| Titanium and Ti­alloys |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
| Zinc Beryllium |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Key: 0 - Can be used without restriction. 1 - Can be used in a non­controlled environment (e.g. assembly area and general non­clean room environment). 2 - Can be used in a clean room environment. 3 - Needs specific measures to avoid galvanic corrosion when these combinations are selected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Mechanical contact surface effects: cold welding, fretting, wear

ECSS-Q-ST-70_0490057

For all solid surfaces in moving contact with other solid surfaces, it shall be demonstrated that the degradation of surface properties over the complete mission does not prevent to meet the performance requirements.

ECSS-Q-ST-70_0490221

For all solid surfaces, moving or in static contact with other solid surfaces, and intended to be separated it shall be demonstrated that the increase in separation force during the physical contact does not exceed the specified limit.

### Life

ECSS-Q-ST-70_0490058

Materials shall be selected to ensure that they will meet the material performance requirements during all their specified lifetime. .

### Bacterial and fungus growth

ECSS-Q-ST-70_0490222

          - Materials selected for manned or fluid systems shall:

not support bacterial or fungus growth,

be sterilizable.

ECSS-Q-ST-70_0490223

          - The extent of the degradation by the sterilization process shall be determined to define margins for design implementation.

ECSS-Q-ST-70_0490224

          - The design and qualification of an equipment to be sterilised shall implement the defined margins for sterilisation degradation.

ECSS-Q-ST-70_0490225

The level of bacterial growth and fungus contamination shall be determined on the assembled hardware.

ECSS-Q-ST-70_0490226

Evaluation of organic materials used in the pressurized environment of long-term, manned spacecraft shall be performed prior to selection and verification.

### Hydrogen embrittlement

ECSS-Q-ST-70_0490227

The possibility of hydrogen embrittlement occurring during component manufacture or use shall be assessed.

ECSS-Q-ST-70_0490228

The material evaluation specified in 5.1.19a shall be performed including the assessment of a protection and control.

ECSS-Q-ST-70_0490229

Based on the assessment specified in the requirement 5.1.19a and 5.1.19b, protection and control measures shall be implemented to avoid hydrogen embrittlement during both mechanical parts manufacturing and use.

Hydrogen embrittlement can be introduced for example during e.g. plating, welding.

ECSS-Q-ST-70_0490230

          - Mechanical parts subject to fatigue or sustained loading stresses, which are made of material susceptible to hydrogen embrittlement, shall be heat treated after coating.

## Selection

### General

ECSS-Q-ST-70_0490059

Materials shall be chosen as follows:

if an identical application in other space programmes similar with respect to environment constraints and lifetime to the proposed application exist, use materials used in such an application;

otherwise:

if materials exist for which satisfactory evaluation results are obtained on samples representative of the application with margin above the specified ones as regards conditions of use, use such materials;

otherwise, use materials included in approved data sources.

For example: ESMDB (see Annex A of ECSS-Q-ST-70-71) ESA and NASA data banks.

ECSS-Q-ST-70_0490060

Whether the materials are already validated or remain to be validated, their selection shall ensure that the following criteria are met:

continuity of supply;

reproducibility of characteristics;

mechanical properties of materials processed according to a specified technique;

environmental-stability properties under space conditions, together with mission- or application-specific requirements.

ECSS-Q-ST-70_0490231

Each critical material shall be validated for the specific application.

Critical material includes the ones of approved data sources.

ECSS-Q-ST-70_0490232

Space-proven materials with heritage covering the specific mission requirements shall be selected at the earliest design stage.

ECSS-Q-ST-70_0490233

          - The supplier shall be responsible for the selection of materials that are capable of meeting the requirements of his business agreement.

ECSS-Q-ST-70_0490234

          - The supplier shall be responsible for storing and maintain all materials data in an internal database.

ECSS-Q-ST-70_0490235

          - The access to the data, specified in requirement 5.2.1f, shall be granted to the customer.

ECSS-Q-ST-70_0490236

          - To achieve high reliability and good performance, the use of a material shall be restricted to within its maximum qualified range of physical and mechanical properties.

ECSS-Q-ST-70_0490237

          - All test methods and inspection techniques used to verify material characteristics and final products shall conform to standards approved by the customer.

ECSS-Q-ST-70_0490238

          - All test methods and inspection techniques specified in the requirement 5.2.1i shall not be used before approval by the customer.

### Constraints

ECSS-Q-ST-70_0490061

          - Pure tin finish with more than 97 % purity shall not be used.

This is due to the possibility of whisker growth and transformation to grey tin powder at low temperatures.

ECSS-Q-ST-70_0490062

          - The incoming inspection of each EEE component batch shall include the verification of the termination composition to avoid assembly of pure tin finish.

ECSS-Q-ST-70_0490239

          - Cadmium and Zinc shall not be used as raw material or surface treatment for flight hardware.

ECSS-Q-ST-70_0490240

          - Cadmium and Zinc shall not be used for ground support equipment exposed to vacuum or when in contact with the flight hardware.

For example during thermal vacuum testing phases on ground.

ECSS-Q-ST-70_0490241

          - The incoming inspection of each non-EEE component batch shall include the verification of the metallic surface treatment to avoid assembly of pure tin, cadmium or zinc finish.

ECSS-Q-ST-70_0490242

The materials presented in the following shall not be used:

Beryllium oxide

Mercury and its compounds

Polyvinyl chloride (PVC)

Radioactive material

This list is not exhaustive.

ECSS-Q-ST-70_0490243

Beryllium shall not be used for structures.

ECSS-Q-ST-70_0490244

Beryllium shall not be used in applications other than structures, unless:

approved by the customer , and

all safety requirements are implemented by the supplier.

## Declared materials list (DML)

ECSS-Q-ST-70_0490063

The supplier shall establish and maintain a declared materials list in conformance with Annex A.

## Criticality analysis

### Overview

To conform to mission requirements, the objective of the analysis is to identify whether further data are required.

### Requirements

ECSS-Q-ST-70_0490064

The supplier shall analyse all the materials contained in his preliminary lists with respect to criticality and in correlation with the risk analysis performed.

ECSS-Q-ST-70_0490065

Any material not meeting the project requirements shall be subject to a RFD for customer approval.

ECSS-Q-ST-70_0490066

Any critical material shall be subject to a RFA in conformance with Annex D to be submitted to the customer.

ECSS-Q-ST-70_0490067

<< deleted >>

## Evaluation and validation phases

### General

ECSS-Q-ST-70_0490068

If so identified by the criticality analysis, the supplier shall perform an evaluation phase before the validation phase for all critical materials with unknown characteristics (new materials) or with major changes in the use or in the configuration

ECSS-Q-ST-70_0490272

In case of an extension of an existing application, the evaluation specified in requirement 5.5.1a need not be performed if so agreed with the customer.

ECSS-Q-ST-70_0490070

Guarantied characteristics of materials and material supplier inspection methods, together with associated documents, shall be available for review at the supplier’s premises before the start of evaluation or qualification phases.

### Evaluation phase

ECSS-Q-ST-70_0490071

The evaluation specified in 5.5.1a shall assess as a minimum the following:

the limits of use;

the materials physical, chemical or functional characteristics along with their values and tolerances;

behavioural tendencies and degradation processes depending on environmental parameters including sensitivity to pollution;

acceptance criteria.

ECSS-Q-ST-70_0490072

When evaluation is performed, the supplier shall provide:

an evaluation programme at PDR, according to Figure 4-1, and

an evaluation report before CDR, according to Figure 4-1 and Table 4-1.

### Validation phase

ECSS-Q-ST-70_0490073

For all critical materials, a validation programme, available at PDR, shall be drawn up by the supplier and then implemented to check or confirm that the materials satisfy the mission requirements with the specified margins to obtain validation status.

ECSS-Q-ST-70_0490074

Validation status shall be decided at CDR in view of the available results obtained written in validation report, and the review of corresponding documentation.

ECSS-Q-ST-70_0490245

The validation programme and report shall be approved by the customer.

### Approval phase

ECSS-Q-ST-70_0490075

The material shall not receive an approval identification in the declared material list for the project unless the requirements specified in the clauses 5.5.2 and 5.5.3 are satisfied.

ECSS-Q-ST-70_0490076

If approval is not granted, the supplier in charge of the item shall either:

select another material, or

propose a modified evaluation programme and resubmit for approval, or

if actions specified in the requirements 5.5.4b.1 and 5.5.4b.2 fail to achieve positive results, initiate a deviation procedure.

### Deviation request

ECSS-Q-ST-70_0490077

For materials not conforming to project requirements, whether at the end of criticality analysis or of evaluation and validation tests, the supplier shall submit a request for deviation in conformance with requirements from the clause 5.2.3.5 of ECSS-Q-ST-10-09.

## Procurement of materials

### Procurement specifications

ECSS-Q-ST-70_0490078

All materials shall be procured to an internationally or nationally specification approved by the supplier quality system, or an in­house fully configured procurement specification which defines the materials properties, the materials requirements, the test methods, the acceptance criteria for the specific applications, source inspection (if any) and material supplier inspection.

ECSS-Q-ST-70_0490079

Where material suppliers do not accept specifications and procurement is by means of a datasheet the supplier shall introduce internal, in­house receipt inspection to ensure that the validation status of the material is maintained during the subsequent procurements.

ECSS-Q-ST-70_0490080

Materials with long lead times or long procurement delays, versus the project schedule, shall be identified before the formal subsystem PDR.

ECSS-Q-ST-70_0490081

Procurement shall be planned, documented and implemented to obtain reliable product assurance provision at CDR.

ECSS-Q-ST-70_0490082

Back­up plans shall be prepared and initiated whenever there is evidence of delays or technical problems.

ECSS-Q-ST-70_0490083

The material requirements shall be accepted by the material supplier or manufacturer.

### Incoming inspection procedure

ECSS-Q-ST-70_0490084

All materials shall be submitted to an incoming inspection.

ECSS-Q-ST-70_0490085

An incoming inspection procedure shall define the inspections and tests to be carried out, particularly for materials that are known to be variable in their final properties.

## Use of materials

### Validation status of materials

ECSS-Q-ST-70_0490086

The supplier shall verify that all critical materials are validated before being used in the manufacture of qualification or flight products.

ECSS-Q-ST-70_0490087

Any modification, change of condition or configuration of application shall lead to a re-evaluation in conformance with the process shown in Figure 4-1.

### Traceability of materials

ECSS-Q-ST-70_0490088

The supplier shall apply the traceability rules specified in the requirements from clause 5.2.5 of ECSS-Q-ST-20 to all materials.

ECSS-Q-ST-70_0490246

Materials shall be identified by a unique reference number, code or a lot number to provide traceability.

The traceability allow to reconstruct the materials history, either individually (individual traceability) or by the manufacturing lot of which it was a part (lot traceability) in case of an incident or nonconformance, or a need for a technical investigation following failure or damage.

### Packaging, storage, removal from storage

ECSS-Q-ST-70_0490090

The supplier shall define provisions for packaging, storage and removal from storage for materials.

ECSS-Q-ST-70_0490091

Measurements and inspections used to guarantee the material integrity and monitoring during storage and removal from storage shall be identified.

### Limited­life materials before implementation

ECSS-Q-ST-70_0490092

The supplier shall ensure that all materials which have limited­life characteristics have their date of manufacture or date of delivery and shelf­life expiry date identified and clearly marked on each lot or batch.

ECSS-Q-ST-70_0490247

Materials which have exceeded their shelf­life expiry date shall not be re­certified until the physical and chemical characteristics are inspected and the parameters, subject to deterioration, are evaluated for continued acceptability according to the acceptance and rejection criteria in conformance with requirements from the clause 4 of ECSS-Q-ST-70-22.

### Limited­life materials after implementation

ECSS-Q-ST-70_0490094

Materials with limited­life after implementation shall be identified and controlled in conformance with requirements from the clause 5 of ECSS-Q-ST-10-04.

1	Propellant is an example of material with limited-life after implementation

2	Storage and mission life are criteria for the assessment and control of those materials.

### Materials nonconformances and alerts

ECSS-Q-ST-70_0490095

Nonconformances and alerts shall be managed in conformance with requirements from the clause 5 and clause 6 of ECSS-Q-ST-10-09 and clause 5.2 of ECSS-Q-ST-10.

### Health and safety

ECSS-Q-ST-70_0490096

Material safety data sheet or equivalent shall be available for all materials.

# Mechanical parts control

## Selection of mechanical parts

ECSS-Q-ST-70_0490097

The supplier shall verify that all materials and processes used in the manufacture of mechanical parts satisfy the mission technical requirements.

## Selection

ECSS-Q-ST-70_0490098

Mechanical parts shall be chosen from used for an identical application in other space programmes similar with respect to environment constraints and lifetime.

ECSS-Q-ST-70_0490099

Type reduction actions shall be implemented at all levels of the programme.

ECSS-Q-ST-70_0490100

Whether the mechanical parts are already qualified or remain to be qualified, their selection shall ensure that the following criteria are met:

durability of supply;

reproducibility of characteristics.

ECSS-Q-ST-70_0490248

The supplier shall be responsible for storing and maintain all mechanical parts data in an internal database.

ECSS-Q-ST-70_0490249

The access to the data, specified in requirement 6.2d, shall be granted to the customer.

## Declared mechanical parts list (DMPL)

ECSS-Q-ST-70_0490101

The supplier shall establish and maintain a declared mechanical parts list in conformance with Annex B.

## Criticality analysis

### Overview

To conform to mission requirements, the objective of the analysis is to identify whether further data are required.

### Requirements

ECSS-Q-ST-70_0490102

The supplier shall analyse all the mechanical parts contained in their preliminary lists with respect to criticality and in correlation with the risk analyses performed in conformance with requirements from the clause 5 of ECSS-Q-ST-10-04.

ECSS-Q-ST-70_0490103

Critical parts shall be identified in the DMPL and included in the critical items list.

ECSS-Q-ST-70_0490104

Any critical part shall be the subject of a RFA in conformance with Annex D submitted to customer approval.

## Evaluation and qualification phases

### General

ECSS-Q-ST-70_0490105

If so identified by the criticality analysis, the supplier shall perform an evaluation phase before the qualification phase for all critical parts with unknown characteristics or with major changes in the use or in the configuration.

ECSS-Q-ST-70_0490273

In case of an extension of an existing application, the evaluation indicated in 6.5.1a need not be performed if so agreed with the customer.

ECSS-Q-ST-70_0490107

Guarantied characteristics of materials and material supplier inspection methods, together with associated documents, shall be available for review at the suppliers premises before the start of evaluation or qualification phases.

Refer to Table 4-1 for an explanation of the steps involved.

### Evaluation phase

ECSS-Q-ST-70_0490108

The evaluation specified in 6.5.1a shall include the following, as a minimum, for each critical part:

the limits of use,

the part’s physical or functional characteristics, along with its values and tolerances,

behavioural tendencies and degradation processes depending on environment parameters, including sensitivity to pollution, and

acceptance criteria.

ECSS-Q-ST-70_0490109

When an evaluation is performed the supplier shall provide:

an evaluation programme at PDR, and

an evaluation report before CDR.

ECSS-Q-ST-70_0490110

The behaviour of the parameters to be monitored which were also recorded during the evaluation programme tests, shall serve as a reference for the analysis of qualification test results.

Example of such behaviour is variation and change over time.

### Qualification phase

ECSS-Q-ST-70_0490111

For each critical part a qualification programme shall be drawn up by the supplier of the equipment using the critical part and then implemented to check or confirm whether the parts satisfy mission requirements with the specified margins.

ECSS-Q-ST-70_0490112

Qualification status shall be decided at CDR in view of the available results obtained, written in qualification report, and the reviews of corresponding documentation .

ECSS-Q-ST-70_0490250

The qualification programme and report shall be submitted to the customer for approval.

### Approval phase

ECSS-Q-ST-70_0490113

The mechanical parts shall not receive an approval identification in the declared mechanical parts list for the project unless the requirements from clauses 6.5.2 and 6.5.3 are satisfied.

ECSS-Q-ST-70_0490114

If approval is not granted, the supplier in charge of the item shall either:

select another mechanical part, or

propose a modified evaluation programme and resubmit for approval, or

if actions specified in the requirements 6.5.4b.1 and 6.5.4b.2 fail to achieve positive results, initiate a deviation procedure.

### Deviation request

ECSS-Q-ST-70_0490115

For mechanical parts not conforming to project requirements, whether at the end of criticality analysis or of evaluation and qualification tests, the supplier shall submit a request for deviation in conformance with requirements from the clause 5.2.3.5 of ECSS-Q-ST-10-09.

## Procurement of mechanical parts

### General

ECSS-Q-ST-70_0490116

Mechanical parts with long lead times or procurement delays, versus the project schedule, shall be identified before the subsystem PDR.

ECSS-Q-ST-70_0490117

Procurement shall be planned, documented and implemented to obtain reliable product assurance provision at CDR.

ECSS-Q-ST-70_0490118

Back­up plans shall be prepared and initiated whenever there is evidence of possible delays or technical problems.

### Procurement specification

ECSS-Q-ST-70_0490119

Each part shall be covered by a procurement specification or a standard approved by the supplier quality system.

ECSS-Q-ST-70_0490120

The procurement specifications shall define the part characteristics, requirements, tests methods, acceptance criteria, lot acceptance testing, source inspection and material supplier inspection.

ECSS-Q-ST-70_0490121

The procurement specifications shall be explicitly accepted by the mechanical part supplier or manufacturer.

### Source inspection

ECSS-Q-ST-70_0490122

For complex mechanical parts related to a specific project development, each supplier shall define the nature and frequency of their own source inspection points.

ECSS-Q-ST-70_0490123

Source inspection shall be carried out by the customer on the premises of the supplier, mechanical part manufacturer, in conformance with requirements from the clauses 5.4.3 and 5.4.4 of ECSS-Q-ST-20.

### Incoming inspection procedure

ECSS-Q-ST-70_0490124

Each part or batch of mechanical parts shall be submitted to an incoming inspection.

ECSS-Q-ST-70_0490125

An incoming inspection procedure shall be established defining the inspections and tests to be carried out.

## Use of mechanical parts

### Qualification status of mechanical parts

ECSS-Q-ST-70_0490126

The supplier shall ensure that all critical parts are qualified before being used in the manufacture of qualification or flight products.

ECSS-Q-ST-70_0490127

Any modification, change in condition or configuration of application shall lead to a re-evaluation in conformance with the process shown in Figure 4-1.

### Traceability of mechanical parts

ECSS-Q-ST-70_0490128

The supplier shall apply the traceability rules specified in clause 5.2.5 of ECSS-Q-ST-20 to his mechanical parts.

ECSS-Q-ST-70_0490251

Parts shall be identified by a unique reference number or code and a lot number to provide traceability - where there is an incident or nonconformance, or for the purposes of technical investigations following failure or damage - to reconstruct the mechanical parts history, either individually (individual traceability) or by the manufacturing lot it was part of (lot traceability).

### Packaging, storage, removal from storage

ECSS-Q-ST-70_0490130

The supplier shall define provisions for packaging, storage and removal from storage for mechanical parts.

ECSS-Q-ST-70_0490131

Measurements and inspections used to guarantee the part integrity and monitoring during storage and removal from storage shall be identified.

### Limited­life mechanical parts or parts subject to wearout

ECSS-Q-ST-70_0490132

Limited­life mechanical parts after implementation or subject to wear out shall be identified and controlled, including storage and mission life.

Examples of such mechanical parts are mechanisms, pyro initiators and O­rings.

ECSS-Q-ST-70_0490133

Limited­life mechanical parts shall be assessed as candidates to the critical items list in conformance with requirements from clause 5 of ECSS-Q-ST-10-04.

### Mechanical parts nonconformances and alerts

ECSS-Q-ST-70_0490134

Management of mechanical parts nonconformances and alerts shall be in conformance with requirements from clause 5 to clause 6 of ECSS-Q-ST-10-09 and clause 5.2 of ECSS-Q-ST-10.

# Process control

## Specifications or procedures

ECSS-Q-ST-70_0490135

Each process to be used in the manufacturing or assembly of a product shall be identified by a specification or procedure.

ECSS-Q-ST-70_0490136

Reference shall be made to accept and reject criteria.

## << deleted >>

ECSS-Q-ST-70_0490137

<< deleted >>

## Process selection and training

ECSS-Q-ST-70_0490138

Processes shall be chosen from already verified processes according to the following order of preference and priority:

processes covered by space agencies or other governmental organization certification for identical conditions of use;

processes for which satisfactory evaluation and verification results are obtained on samples representative of the application with a sufficient margin as regards conditions of use;

processes already used by the same supplier for other space programmes in the same conditions of use.

ECSS-Q-ST-70_0490139

Whether the processes are already verified or remain to be verified, their selection shall ensure that the following criteria are met:

reliability;

inspectability;

re­workability of the process item;

reproducibility.

ECSS-Q-ST-70_0490252

The supplier shall store and maintain all processes data in an internal database.

ECSS-Q-ST-70_0490253

The access to the data, from requirement 7.3c, shall be granted to the customer.

ECSS-Q-ST-70_0490254

Processes using limited-life materials shall be in conformance with the requirements from clause 4 of ECSS-Q-ST-70-22.

ECSS-Q-ST-70_0490255

Operators shall be trained for all processes.

ECSS-Q-ST-70_0490256

Operators performing special processes shall be certified in conformance with the requirements of the relevant standard.

ECSS-Q-ST-70_0490257

Inspectors shall be trained and certified for all processes.

ECSS-Q-ST-70_0490258

Certification shall be reassessed at least every two years.

ECSS-Q-ST-70_0490259

All training and certification shall only be performed at a school or by certified instructors in conformance with customer requirements.

ECSS-Q-ST-70_0490260

Retraining and recertification of operators shall be applied in the event of a new process, modification to an existing process or a change of the equipment used.

ECSS-Q-ST-70_0490261

Retraining and recertification of inspectors shall be applied in the event of a new inspection procedure, modification to an existing inspection procedure or a change of an existing one.

ECSS-Q-ST-70_0490262

In the case of a process suspension for four months continuously, retraining and recertification shall be performed.

ECSS-Q-ST-70_0490263

Standards relating to the occupational health of operators working with processes resulting in exposure to vapours, dust or debris shall be implemented and controlled.

## Declared processes list (DPL)

ECSS-Q-ST-70_0490140

The supplier shall establish and maintain a declared processes list in conformance with Annex C.

## Criticality analysis

### Overview

To conform to mission requirements, the objective of the analysis is to identify whether further data is required.

### Requirements

ECSS-Q-ST-70_0490141

The supplier shall analyse all the processes contained in their preliminary lists with respect to criticality and in correlation with the risk analyses performed.

ECSS-Q-ST-70_0490142

Critical processes shall be identified in the DPL and included in the list of critical items.

ECSS-Q-ST-70_0490143

Any critical process shall be the subject of an RFA in conformance with Annex D submitted to customer approval.

ECSS-Q-ST-70_0490144

Special processes shall be identified and controlled.

ECSS-Q-ST-70_0490145

Process control shall be ensured by means of procedures available for customer review.

Best practice is to implement a statistical process control.

ECSS-Q-ST-70_0490274

<< deleted >>

## Evaluation and verification phase

### General

ECSS-Q-ST-70_0490147

If so identified by the criticality analysis, the supplier shall perform an evaluation phase before the verification phase for all critical processes which are new or with major changes in the use or in the configuration.

ECSS-Q-ST-70_0490275

In case of an extension of an existing application, the evaluation specified in requirement 7.6.1a need not be performed if so agreed with the customer.

ECSS-Q-ST-70_0490149

For confidential processes, the supplier shall prove that the process has been verified.

1	For example, by presenting a verification certificate from space agencies or other governmental organization responsible to check the applicability of this verification.

2	Refer to Table 4-1 for an explanation of the steps involved.

### Evaluation phase

ECSS-Q-ST-70_0490150

The evaluation specified in 7.6.1a shall include the following as a minimum for each critical process:

the limits of use,

the values, determined by test samples or technology samples, of relevant parameters and their tolerances, and

acceptance criteria.

ECSS-Q-ST-70_0490151

When an evaluation is performed, the supplier shall provide

an evaluation plan at PDR, and

an evaluation report before CDR.

### Verification phase

ECSS-Q-ST-70_0490152

For each critical process, the supplier shall implement a verification programme.

ECSS-Q-ST-70_0490153

The verification programme shall be defined in conformance with ECSS-E-ST-10-02.

ECSS-Q-ST-70_0490154

The supplier shall ensure that the processes satisfy the mission requirements and that the parameters needed for the product design are defined so as to obtain verification status.

ECSS-Q-ST-70_0490155

Verification status shall be decided at CDR in view of the available results obtained, written in verification report, and the review of corresponding documentation.

ECSS-Q-ST-70_0490264

The verification programme and report shall be approved by the customer.

### Approval phase

ECSS-Q-ST-70_0490156

A processes shall not receive an approval identification in the declared processes list unless requirements from clauses 7.6.2 and 7.6.3 are satisfied.

ECSS-Q-ST-70_0490265

In case approval specified in the requirement 7.6.4a is not granted, the supplier in charge of the item shall either:

select other processes, or

propose a modified evaluation programme and resubmit for approval, or

if actions specified in 7.6.4b.1 and 7.6.4b.2 fail to achieve positive results, initiate a deviation procedure.

### Deviation request

ECSS-Q-ST-70_0490157

For processes not conforming to project requirements, whether at the end of criticality analysis or of evaluation and verification tests, the supplier shall submit a request for deviation in conformance with requirements from clause 5.2.3.5 of ECSS-Q-ST-10-09.

## Use of a process

### Verification status of a process

ECSS-Q-ST-70_0490158

The supplier shall confirm that all critical processes have been verified before being used in the manufacture of qualification or flight products.

ECSS-Q-ST-70_0490159

Any modification, change of condition or configuration of application shall lead to a re-evaluation in conformance with the process.

See Figure 4-1.

### Re­verification of a process

ECSS-Q-ST-70_0490160

When a process needs to be re­verified, a request for approval (RFA) in conformance with DRD from Annex D shall be established and a re­verification programme shall be implemented.

Any prolonged stoppage in manufacturing, any major change of the facilities or procedures or any transfer of production to another entity can invalidate partially or completely the initial verification of a process.

### Implementation of a process

ECSS-Q-ST-70_0490161

Before implementation of a process, the supplier shall ensure that personnel are trained in accordance with the training and certification programme specified in 7.3f, 7.3g and 7.3h and that environment, means and documentation are adequate.

ECSS-Q-ST-70_0490162

This verification shall ensure that:

manufacturing and quality control tools associated with the process are adequate, calibrated and properly maintained are used under environmental and cleanliness conditions, in conformance with requirements from clause 4.4,

personnel is trained and certified in accordance with the training and certification programme specified in 7.3f, 7.3g and 7.3h, and

the processes specifications, manufacturing and inspection procedures and workmanship standards including definition of manufacturing operations and acceptance criteria exist.

1	Photographically documented if possible for visual acceptance criteria at the appropriate work and inspection stations.

2	For planning of manufacturing, assembly and integration operation and inspection see ECSS-Q-ST-20.

### Traceability of processes

ECSS-Q-ST-70_0490163

Traceability of processes shall be ensured in conformance with requirements from clause 5.2.5 of ECSS-Q-ST-20.

### Process nonconformances and alerts

ECSS-Q-ST-70_0490164

Process nonconformances and alerts shall be processed in conformance with requirements from clause 5 to clause 6 of ECSS-Q-ST-10-09 and clause 5.2.9 of ECSS-Q-ST-10.

### Mandatory inspection points (MIP)

ECSS-Q-ST-70_0490165

MIPs shall be defined in conformance with requirements from clause 5.5.8 of ECSS-Q-ST-20.

### Packaging, storage, removal from storage

ECSS-Q-ST-70_0490166

The supplier shall define provisions for packaging, storage, and removal from storage for products or semi­finished products before and after implementation of processes.

(normative)
Declared materials list (DML) - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-Q-ST-70, requirements 4.2.3a and 5.3a.

Purpose and objective

The purpose of the DML is to have a detailed record of all the materials used to produce the products of a project or programme.

The data in the DML make possible to assess whether the materials are suitable for a specific application, at the supplier and the customer levels (in the approval status column).

The DML is prepared for each “Configuration item” at the relevant stages (e.g. at the start, PDR, CDR and QR) as defined in the flow chart given in Figure 4-1.

The following documents are linked to the DML:

the declared process list (DPL);

request for approval (RFA) materials.

Expected response

Scope and content

Materials groups

ECSS-Q-ST-70_0490167

          - The DML shall contain the following statements:

“Materials are classified into 20 groups depending on their type or their main use, see Table A-1”.

“Primers are classified in the group of their associated component.”

“Where no project requirement exists for a separate DMPL, mechanical parts are entered on the DML as a separate group with the corresponding numbers.”

ECSS-Q-ST-70_0490168

          - If new groups are created, for a given project, these shall have numbers over 21.

ECSS-Q-ST-70_0490188

: Material group numbers

| Group number | Description |
| --- | --- |
| 1 | Aluminium and aluminium alloys |
| 2 | Copper and copper alloys |
| 3 | Nickel and nickel alloys |
| 4 | Titanium and titanium alloys |
| 5 | Steels |
| 6 | Stainless steels |
| 7 | Filler metals: welding, brazing soldering |
| 8 | Miscellaneous metallic materials |
| 9 | Optical materials |
| 10 | Adhesives, coatings, varnishes |
| 11 | Adhesive tapes |
| 12 | Paints and inks |
| 13 | Lubricants |
| 14 | Potting compounds, sealants, foams |
| 15 | Reinforced plastics (including PCBs) |
| 16 | Rubbers and elastomers |
| 17 | Thermoplastics |
| 18 | Thermoset plastics (including PCBs) |
| 19 | Material aspects of wires and cables |
| 20 | Miscellaneous non­metallic materials, e.g. ceramics |

Contents of the DML

ECSS-Q-ST-70_0490169

          - The DML shall include the information stated in Figure A-1, where the header information identifies the list as the declared materials list and includes the issue number and date of issue, as follows:

Item number (applicable to equipment manufacturer level only)

This consists of the material group identifier and the user code. It takes the form of:

<group number>.<identifier within the group>.<running number>.<user code>

For example: 11.5.1.KOF.

Characteristics of the item number are:

The user, identified by an agreed user code for the project.

One only per material type.

Does not change during the life of the materials list (sub­items are permitted when deemed necessary).

Commercial identification or standardized designation

Enter the correct and standard designation, such as the trade name plus number.

If no trade name exists, enter the manufacturer’s name plus number.

For metal alloys, the Aluminium Association (AA) system is recommended for aluminium alloys, and the American Iron and Steel Institute (AISI) system for steel. For other metals or alloys, the main constituent is entered first except in the case of a traditional name.

For example: brass or bronze.

For each material, as designated in A.2.1<2>a2(a) to (c), use a unique item number. If several lines are used for different applications or processing, add sub­item numbers.

Chemical nature and product type

For metallic materials, add the condition as procured if applicable.

Examples of such conditions are rolled and heat treatment.

Where a semi­finished product is procured, give the relevant state.

Examples of such state are form, plate and sheet.

Give the thickness of the material, that can be an important parameter.

Examples of chemical nature are: epoxy resin, polyurethane adhesive, Ti6Al4V.

Procurement information

Manufacturer or distributor: name of the manufacturer and name of the distributor if different.

Procurement specification: provide reference to one of the following:

the procurement specification with issue, revision and date

a national or international specification or standard, if this exists, and identifies the source of procurement (if relevant), with issue, revision and date

datasheets, in which case indication of issue or date is not applicable.

Processing parameters

List here a summary of the process parameters applied by the user of the process.

E.g. mixture proportions, cure temperature, special cleaning agent, surface treatment, thermal treatment and temperature, and reference to specification number.

Use and location

Use codes able to define the location of the material with respect to the:

subsystem;

particular piece of equipment (box or item);

use of the equipment, e.g. a structural element, thermal control, electrical insulation.

If the CI number is not included in the list header, then a suitable abbreviation of the relevant subsystem is included.

Include any restrictions that apply to the use of a particular material in the corresponding comment column.

Environmental code

The environmental code is defined using Table A-2.

Size code

The size code is indicated by an alphanumeric combination, such as A5, V2 or M3, as in Table A-3.

Validation references, justification for approval and prime comments and prime approval

Make reference to relevant test data that demonstrates the acceptability of the material under the environmental conditions and the application relevant to the particular project concerned. Specifically, in column 9.1, corrosion (CORR), stress corrosion (SCC), flammability (FLAM), offgassing (OFFG) and outgassing (OUTG) data or report­references are entered.

Use standard abbreviations to summarize the acceptance status of a material for a particular property.

Use the justification for approval (column 9.2) and prime approval (column 9.3) columns for any additional information to obtain customer approval.

Standard abbreviations are used to summarize the acceptability or otherwise of a material for a specific property. These are defined for the project.

Select the supplier approval status code from Table A-4.

Customer approval status code and comments

Select this code from Table A-4.

Include additional comments where appropriate.

ECSS-Q-ST-70_0490279

: Environmental code

| Radiation/UV/ATOX (R)a |  | Ambience (A) | Temperature (T)b,c |
| --- | --- | --- | --- |
| G: Geostationary | S: Outside shadow | V: Vacuum | 1: 0 K to 100 K |
| L: Low orbit | L: Outside light | H: Hermetic | 2: 101 K to 200 K |
| B: Radiation belt |  | M: Manned | 3: 201 K to 300 K |
| I: Interplanetary |  | E: Elevated pressure | … |
| P: Planetary |  |  |  |
| a For all materials, a letter is selected from the left­hand column. For materials on the surface of the spacecraft, the letter “L” or “S” is added. b Thermal cycling to be indicated by two values, e.g. 3/5. c “RT” (room temperature) can be accepted as a code between 283 K (10 °C) and 313 K (40 °C). |  |  |  |
| NOTE: The materials that are at a boundary between environments are described by two sets of codes. |  |  |  |

ECSS-Q-ST-70_0490280

: Size code

| Size code | Value |
| --- | --- |
| 0 | 0 < A or V or M  1 |
| 1 | 1 < A or V or M  10 |
| 2 | 10 < A or V or M  100 |
| 3 | 100 < A or V or M  1 000 |
| 4 | … |
| where: A is the area, in cm2 V is the volume, in cm3 M is the mass, in g |  |

ECSS-Q-ST-70_0490266

If heritage is used as justification for approval it shall cover the current hardware configuration and mission profile.

ECSS-Q-ST-70_0490281

: Approval status

| Code | Description |
| --- | --- |
| A | Approved. All materials classified “A” may be used without restriction. |
| X | Approved with an RFA. These materials are subjected to an evaluation or validation programme. The RFA number is entered as a comment. |
| W | Approved with a concession. These materials do not meet the requirements but are used for functional reasons. The concession number is entered as a comment. |
| P | Pending a decision. Materials for which an evaluation report or a concession is waiting for the customer’s provisional or definitive approval. |
| O | Open. New materials or materials for which investigations and validations are in progress. |
| R | Rejected. |
| D | Deleted. This classification is used for a material that is no longer used. |
| NOTE If approval cannot be given and one of the other codes is entered, enter the comments in the appropriate column. |  |

Special remarks

ECSS-Q-ST-70_0490170

          - The change record shall list the successive issues and their release dates since the first formal issue of the document.

ECSS-Q-ST-70_0490171

The change record shall include a brief description of the updates which contributed to each issue or revision.

Figure A-1 of this DRD proposes an example of the format and defines the content within the framework of a project or a programme.

| DECLARED MATERIALS LIST (DML) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Programme name: ABCDEFG |  |  |  | CI no.: 12345676890 Group (Title): abcdefg |  | Doc no.: 001 Issue/Revision: 1/4 |  |  |  | Date: 01.10.2000 Page: 1 |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  |  | 10 |
|  |  |  |  |  |  |  |  | 9.1 | 9.2 |  | 9.3 |  |
| Item no. and user code | Commercial identification or standardized designation | 1) Chemical nature 2) Product type | 1) Manufacturer/ supplier name 2) Procurement spec. Issue/RevDate | Summary of process parameters | 1) Subsystem 2) Equipment 3) Use | 1) R 2) A 3) T | 1) A 2) V 3) M | Acronym/ rating/ Validation Ref. for applicable properties | 1) Justification for approval 2) Prime comments |  | Prime approval status | Customer approval status/ comments |
| 1.2.1.TXES | AZ5GU | 1) Al.Zn5.6 Mg2.5 Cdu1.6, Cr0.3 eq. AA7075 2) Plate | 1) Almet Pechiney 2) CRB 527 01/02/01.02.1996 | T7351 and Iridit 14 heat treatment | 1) PL 2) E4 package 3) Structure | 1) LS 2) V 3) 3 | 1) 2) 3) M3 |  | 1) Used on ETS2 2) |  | A | Give evidence of ETS2 specific usage |
| 10.1.1.ETCA | DC93500 | 1) Silicon 2) Two parts | 1) Dow Corning 2) E3846MC10S 02/02/1984 | Mixture: 10/1 in g Curing: 4h/65 ºC | 1) PCU 2) Experiment tray 3) Part potting | 1) G 2) V 3) 3-4 | 1) 2) 3) M3 |  | 1) ECSS­Q-ST-70­01 2) |  | A |  |
| 11.5.1.KOF | ECCOFOAM EPH | 1) Polyurethane 2) Resin/Catalyst 1202H | 1) Emerson and Cuming 2) SP/FOK/05/684 03/01/25.06.1992 | Resin/ Cat: 100/65g 4h/40 C +48h/100 C | 1) GP 2) Platform 3) Package potting | 1) LS 2) M 3) 3-4 | 1) 2) V3 3) |  | 1) DU–96–352 2) Used at T > 100 C (Risk of distortion beyond) |  | A | A |

ECSS-Q-ST-70_0490189

: Example of a realized DML

(normative)
Declared mechanical parts list (DMPL) - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-Q-ST-70, requirements 4.2.3b and 6.3a.

Purpose and objective

The purpose of the DMPL is to have detailed record of all the mechanical parts used to produce the products of a project or programme.

The data in the DMPL make possible to assess whether the mechanical parts are suitable for a specific application, at the supplier and customer levels (in the approval status column).

The DMPL is prepared for each “Configuration item” at the relevant stages (e.g. Start, PDR, CDR and QRR) as defined in the flow chart given in Figure 4-1 of ECSS-Q-ST-70.

The following document is linked to the DMPL: request for approval (RFA) mechanical parts.

Expected response

Scope and content

Mechanical parts groups

ECSS-Q-ST-70_0490172

          - The DMPL shall contain the following statements:

“Mechanical parts are classified into 11 groups depending on their type or their main use” (see Table B-1).

ECSS-Q-ST-70_0490173

If, for a given project it is considered necessary to create new groups, these shall have numbers over 61.

ECSS-Q-ST-70_0490276

Items that appear in the EEE parts list should not be repeated here.

For example, heaters, some valves, thermostats, relays, transformer coils and solenoids.

ECSS-Q-ST-70_0490190

: Mechanical part group numbers

| Group number | Description |
| --- | --- |
| 51 | Spacing parts (e.g. washers and spacers) |
| 52 | Connecting parts (e.g. bolts, nuts, rivets, inserts and clips) |
| 53 | Bearing parts (e.g. ball­bearings and needle bearings) |
| 54 | Separating parts (e.g. pyrotechnics, springs and cutters) |
| 55 | Control parts (e.g. gears) |
| 56 | Fluid handling parts (e.g. diffusers) |
| 57 | Heating parts |
| 58 | Measuring instruments (e.g. gauges and thermocouples) |
| 59 | Optical passive equipment |
| 60 | Magnetic parts |
| 61 | Other parts |

Contents of the DMPL

ECSS-Q-ST-70_0490175

          - The DMPL shall include the information in Figure B-1, where the header information identifies the list as the declared mechanical parts  list and includes the issue number and date of issue, as follows:

Item number (applicable at equipment supplier level only)

This consists of the mechanical part identifier and the user code. It takes the form of:

<group number>.<identifier within the group>.<running number>.<user code>

For example: 7.2.1.ACSA.

Characteristics of the item number are:

Identify the subcontractor by an agreed user code for the project.

One only per mechanical part type.

Does not change during the life of the mechanical parts list.

Commercial identification

Enter the correct and standard designation, such as trade name plus number.

If no trade name exists then enter the manufacturers’ name and number.

Type of part

Describe the material and surface treatment (if applicable).

Procurement information

Manufacturer or distributor: name of the manufacturer and name of the distributor if different.

Procurement specification: provide reference to one of the following:

the procurement specification with issue, revision and date

a national or international specification or standard, if this exists, and identifies the source of procurement (if relevant), with issue, revision and date.

Elementary function, main characteristics

Enter the function of the mechanical part.

Enter the main characteristics of the mechanical part.

For example, number of revolutions per minute for a ball bearing.

Use and location

Use codes able to define the location of the mechanical part with respect to the:

subsystem;

particular piece of equipment (box or item);

use of the equipment.

Environmental code

The environmental code is defined using Table B-2.

Criticality

Enter “C” for critical or “N” for non­critical. If a mechanical part is considered critical, describe the reason for the criticality and methods of control.

Supplier reference, prime comments and prime approval status

Use the supplier reference, prime comments and approval columns to enter all additional information that are necessary in order to obtain customer approval.

This information comprises reference and issue of the RFA or approval, mechanical parts justification file, evaluation reports and deviation requests.

Make reference to the relevant test data that demonstrates acceptability of the mechanical part under the environment conditions and the application relevant to the particular project concerned.

Use standard abbreviations, defined by the customer, to summarize the acceptance status of a mechanical part for a particular property.

In order to justify the use of a material for flammability resistance, list here the material thickness and height of oxygen share.

Select the prime approval status code from Table B-3.

The customer approval status code and comments

Select this code from Table B-3.

Include additional comments where appropriate.

ECSS-Q-ST-70_0490282

: Environmental code

| Radiation/UV/ATOX (R)a |  | Ambience (A) | Temperature (T)b,c |
| --- | --- | --- | --- |
| G: Geostationary | S: Outside shadow | V: Vacuum | 1: 0 K to 100 K |
| L: Low orbit | L: Outside light | H: Hermetic | 2: 101 K to 200 K |
| B: Radiation belt |  | M: Manned | 3: 201 K to 300 K |
| I: Interplanetary |  | E: Elevated pressure |  |
| P: Planetary |  |  |  |
| a For all mechanical parts, a letter is selected from the left­hand column. For mechanical parts on the surface of the spacecraft, the letter “L” or “S” is added. b Thermal cycling to be indicated by two values, e.g. 3/5. c “RT” (room temperature) can be accepted as a code between 283 K (10 °C) and 313 K (40 °C). |  |  |  |

ECSS-Q-ST-70_0490283

: Approval status

| Code | Description |
| --- | --- |
| A | Approved All mechanical parts classified “A” may be used without restriction. |
| X | Approved with an RFA These mechanical parts are subjected to an evaluation or validation programme. The RFA number is entered as a comment. |
| W | Approved with a concession These mechanical parts do not meet the requirements but are used for functional reasons. The use of such mechanical parts is approved by the customer. The concession number is entered as a comment. |
| P | Pending a decision Mechanical parts for which an evaluation report or a concession is waiting for the customer’s provisional or definitive approval. |
| O | Open New mechanical parts or mechanical parts for which investigations and validations are in progress. |
| R | Rejected. |
| D | Deleted This classification is used for a mechanical part that is no longer used. |
| NOTE: If approval cannot be given and one of the other codes is entered, enter the comments in the appropriate column. |  |

Special remarks

ECSS-Q-ST-70_0490176

          - The change record shall list the successive issues and their release dates since the first formal issue of the document.

ECSS-Q-ST-70_0490177

This record shall include a brief description of the updates which contributed to each issue or revision.

Figure B-1 of this DRD proposes an example of the format and defines the content within the framework of a project or a programme.

| DECLARED MECHANICAL PARTS LIST (DMPL) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Programme name: ABCDEFG |  |  |  | CI no.: 12345676890 Group (Title): abcdefg |  | Doc no.: 001 Issue/Revision: 1/4 |  |  | Date: 01.10.2000 Page: 1 |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  | 10 |
|  |  |  |  |  |  |  |  | 9.1 |  | 9.2 |  |
| Item no. and user code | Commercial identification | Type of part | 1) Procurement specification 2) Issue/Revision/ Date | 1) Elementary function 2) Main characteristics | 1) Subsystem 2) Equipment 3) Use | 1) R 2) A 3) T | 1) Criticality 2) Reason and method of control | 1) Supplier Reference 2) prime comments |  | Prime approval status | Customer approval status/comments |
| 51.2.1.ACSA | ESA003521000120 | Copper/AL bimetal ring | 1) AIEV 2) From catalogue | 1) Separator ring 2) Heat conductor | 1) TC 2) Plate interface 3) Spacing and heat inspection | 1) G 2) V 3) 3-4 | 1) N 2) | 1) Used on all projects 2) |  | A | A |
| 52.2.1.ASAD | A0090TX...XA | Ti6Al4V screws > M4 | 1) White areo 2) ASNA0090 DSN2413 | 1) assembly 2) | 1) PTANK 2) plate 3) fixing | 1) G 2) V 3) 3-4 | 1) N 2) | 1) Used on TC2 2) |  | A | A |
| 60.1.1.ACSA | 42908TC/F | Ferrite cores magnetic | 1) Magnetics, Data sheet 2) SP/MAGN/003 01.02/03.06.1999 | 1) Coil core of transformer 2) Magnetic component | 1) TC 2) South face 3) Heat regulation | 1) G 2) V 3) 3-4 | 1) C 2) to be qualified | 1) 2) |  |  |  |

ECSS-Q-ST-70_0490267

: Example of a realized DMPL

(normative)
Declared process list (DPL) - DRD

DRD identification

Requirement identification and source document

This DRD is called from ECSS-Q-ST-70, requirements 4.2.3c and 7.4a.

Purpose and objective

The purpose of the DPL is to have a detailed record of all the processes used to produce the products of a project or programme.

The data in the DPL make possible to assess whether the processes are suitable for a specific application, at the supplier and customer levels (in the approval status column).

The DPL is prepared for each “Configuration item” at the relevant stages (e.g. Start, PDR, CDR and QRR) as defined in the flow chart given in Figure 4-1 of ECSS-Q-ST-70.

The following documents are linked to the DPL:

declared material list (DPL),

request for approval (RFA) processes.

Expected response

Scope and content

Process groups

ECSS-Q-ST-70_0490178

          - The DPL shall contain the following statements:

Processes are classified into 17 groups depending on their type or their main use, see Table C-1.

ECSS-Q-ST-70_0490179

If, for a given project it is considered necessary to create new groups, these shall have numbers over 17.

ECSS-Q-ST-70_0490192

: Process group numbers

| Group number | Description |
| --- | --- |
| 1 | Adhesive bonding |
| 2 | Composite manufacture |
| 3 | Encapsulation/moulding |
| 4 | Painting/coating |
| 5 | Cleaning |
| 6 | Welding/brazing |
| 7 | Crimping/stripping/wire wrapping |
| 8 | Soldering |
| 9 | Surface treatments |
| 10 | Plating |
| 11 | Machining |
| 12 | Forming |
| 13 | Heat treatment |
| 14 | Special fabrication: processes developed specifically for the programme |
| 15 | Marking |
| 16 | Miscellaneous processes |
| 17 | Inspection procedures |

Contents of the DPL

ECSS-Q-ST-70_0490180

          - The DPL shall include the information in Figure C-1, where the header information identifies the list as the declared processes list and includes the issue number and date of issue, as follows

Item number (applicable to equipment supplier level only)

This consists of the process identifier and the user code. It takes the form of

<group number>.<identifier within the group>.<running number>.<user code>

For example: 1.2.1.SSEX

Characteristics of the item number are:

Identify the subcontractor by an agreed user code for the project.

One only per process type.

Does not change during the life of the process list.

Process identification

Indicate the correct and standard identification of the process, e.g. the process name or title: bonding, coating or soldering.

Specification

Identify the name or abbreviation of the process executor.

Make a reference to the associated procedure, together with the issue, revision and date.

For example, national, international, EN, ISO, ECSS or company in­house.

Process description

Enter here a short description of the process.

Use and location

Use codes able to define the location of the process with respect to the:

subsystem,

particular piece of equipment (box or item),

use of the equipment.

For example, a structural element, thermal control, electrical insulation).

This column number is not used.

Associated item numbers

Enter the associated material list (DML) or mechanical parts list (DMPL) with the process.

Criticality

Enter “C” for critical or “N” for non­critical.

If a process is considered to be critical, Enter the references to the relevant RFA.

Supplier reference, prime comments and approval

Use the supplier reference and approval columns to enter all additional information that are necessary to obtain customer approval.

Select the supplier approval status code from Table C-2.

Give reference to qualification programme performed for critical processes.

The customer approval status code and comments

Select this code from Table C-2.

Include additional comments where appropriate.

ECSS-Q-ST-70_0490268

If heritage is used as justification for approval it shall cover the current hardware configuration and mission profile.

ECSS-Q-ST-70_0490284

: Approval status

| Code | Description |
| --- | --- |
| A | Approved All processes classified “A” may be used without restriction. |
| X | Approved with an RFA These processes are subjected to an evaluation or validation programme. The RFA number is entered as a comment. |
| W | Approved with a concession These processes do not meet the requirements but are used for functional reasons. It is important to reduce the use of such processes to a minimum. All deviation requests are approved by the customer. The concession number is entered as a comment. |
| P | Pending a decision Processes for which an evaluation report or a concession is waiting for the customer’s provisional or definitive approval. |
| O | Open New processes or processes for which investigations and validations are in progress. |
| R | Rejected. |
| D | Deleted This classification is used for a process that is no longer used. |
| If approval cannot be given and one of the other codes is entered, enter the comments in the appropriate column. |  |

Special remarks

ECSS-Q-ST-70_0490181

          - The change record shall list the successive issues and their release dates since the first formal issue of the document.

ECSS-Q-ST-70_0490182

This record shall include a brief description of the updates which contributed to each issue or revision.

Figure C-1 of this DRD proposes an example of the format and define the content within the framework of a project or a programme.

| DECLARED PROCESS LIST (DPL) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Programme name: ABCDEFG |  |  | CI no.: 1234567890 Group (Title): abcdefg |  | Doc no.: 001 Issue/Revision: 1/5 |  |  | Date: 14.05.2000 Page: 1 |  |  |
| 1 | 2 | 3 | 4 | 5 | 7 | 8 | 9 |  |  | 10 |
|  |  |  |  |  |  |  | 9.1 |  | 9.2 |  |
| Item no. and user code | Process identification | 1) User name 2) Associated procedure issue/revision/ date | Process description | 1) Subsystem code 2) Equipment code 3) Use | Associated DML or DMPL item number | 1) Criticality 2) Reason for criticality | 1) Supplier Reference 2) Prime comments |  | Prime approval status | Customer approval status/ comments |
| 1.2.1.SSEX | Bonding | 1) EREMS 2) E/SQ/PI/012 02/01/02.08.1984 | Applying a spot of glue with a stainless steel dispenser | 1) BE3 2) C5 board 3) To fix parts | 6.1.2.ETC | 1) N 2) | 1) Used on ANTARES 2) |  | A | Give evidence of ANTARES specific usage |
| 4.3.1.KOF | Coating | 1) CERCO 2) E/SQ/PI/023 02/01/08.12.1985 | Coating by paintbrush or by immersion in the resin | 1) BE3 2) C1 C2 boards 3) Protection of CI and EEE parts | 2.1.1.KOF | 1) N 2) | 1) Used on PASTEC, ANTARES 2) |  | A | Give evidence of PASTEC ANTARES specific usage |
| 8.3.1.KOF | Vapour phase soldering of SMDs | 1) EREMS 2) E/SQ/PI/026 01/02/09.09.1997 | ECSS-Q-ST-70-38 | 1) BE3 2) C3 3) | 15.1.1.AST | 1) C 2) | 1) QM/04L123/ BD/MH Table 1 2) |  | A | A |

ECSS-Q-ST-70_0490193

: Example of realized DPL

(normative)
Request for approval (RFA) - DRD

DRD identification

Requirement identification and source document

This DRD is called up from ECSS-Q-ST-70, requirements 5.4.2c, 6.4.2c and 7.5.2c.

Purpose and objective

The objective of an RFA is to enable the supplier to request from the customer permission to use a critical mechanical part, material or process.

The information provided by the supplier makes possible for the customer to assess whether the critical mechanical part, material or process is suitable for a specific application.

The RFA is prepared for each critical mechanical part, material or process at the relevant stages as defined in the flow chart given in Figure 4-1 of ECSS-Q-ST-70.

The following documents are linked to the RFA:

declared mechanical parts list;

declared materials list;

declared process list.

Expected response

Scope and content

ECSS-Q-ST-70_0490277

          - The RFA should contain the information in Figure D-1 (pages 1 and 2), as follows:

The header information, identifying the document as a request for approval together with the project logo and project name, RFA reference, issue revision and date.

Originator

Originator’s name and reference.

Location

Subsystem and equipment codes.

Item description

Brief description of the item.

PMP information

The DML, DMPL or DPL item number and list reference.

Item status, including the following information:

manufacturers’ name and qualification reference;

suppliers’ name and qualification status;

product or material specification;

procurement specification;

process or handling specification;

other related process or handling specifications;

verification or qualification specification;

report on verification or qualification.

Reason for RFA

Enter the reason for the RFA.

Application and location details

Enter here the details of the application and exact location of the item.

Give the reference in the CIL.

Evaluation and validation programme

Reference and details of main tests.

Subcontractor supplier approval for the first issue of the RFA.

Customer initial decision, to be entered on first issue of the RFA providing:

the decision concerning the proposed material,

the requirement to perform tests (deviation request as necessary), and

the decision concerning the proposed test programme.

Customer’s and final customer’s (if applicable) signature

Justification results obtained with reference to the supplier’s validation report and conclusion.

Subcontractor supplier approval on RFA final issue.

Final approval status of the RFA by the customer.

Special remarks

ECSS-Q-ST-70_0490184

          - The RFA shall be completed by the supplier (parts 1 to 10, 13 and 14) and the customer (parts 11, 12 and 15).

| 1 | Company: | Project: |  |  | Reference: RFA- |  |  |  |  | Page 1 of 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Issue |  |  |  |  |  |  |  |
|  |  |  |  |  | Revision |  |  |  |  |  |  |  |
|  |  |  |  |  | Date |  |  |  |  |  |  |  |
| Request for approval (RFA) |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Originator: |  | 3 | Subsystem: |  |  |  |  |  |  |  |  |
|  | Originator reference: |  |  | Equipment: |  |  |  |  |  |  |  |  |
| 4 | Item description: |  | 5 | PMP list item number: |  |  |  |  |  |  |  |  |
|  |  |  |  | PMP list reference: |  |  |  |  |  |  |  |  |
| 6 | Item status |  |  |  |  |  |  |  |  |  |  |  |
| Manufacturer: |  |  | Manufacturer qualification reference: |  |  |  |  |  |  |  |  |  |
| Supplier: |  |  | Qualification status: |  |  |  |  |  |  |  |  |  |
| Product/material specification: |  |  | Procurement specification: |  |  |  |  |  |  |  |  |  |
| Process/handling specification: |  |  | Related specification: |  |  |  |  |  |  |  |  |  |
| Verification/qualification specification: |  |  | Report: |  |  |  |  |  |  |  |  |  |
| 7 | Reason for RFA |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Application/location details: |  | CIL Reference: |  |  |  |  |  |  |  |  |  |
| 9 | Evaluation/validation programme (title, reference) |  |  |  |  |  |  |  |  |  |  |  |
|  | Tests |  |  |  |  |  |  |  |  |  |  |  |
|  | Plan, procedures, schedule to be attached |  |  |  |  |  |  |  |  |  |  |  |

ECSS-Q-ST-70_0490278

: Example of RFA (Page 1 of 2)

| 1 | Company: |  | Project: |  | Reference: RFA- |  |  |  |  |  | Page 2 of 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Issue |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Revision |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Date |  |  |  |  |  |  |  |  |
| Request for approval (RFA) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  | Materials and processes responsible |  | PA responsible |  | Project responsible |  |  |  |  |  |  |  |
|  | Supplier approval on RFA first issue |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Decision on RFA first issue: |  |  | Comments: |  |  |  |  |  |  |  |  |  |
|  | - Request refused: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Submit deviation: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Proceed with validation programme: |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Decision | Materials and processes responsible |  | PA responsible |  | Project responsible |  |  |  |  |  |  |  |
|  | Customer agree/disagree |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Final customer (if applicable) agree/disagree |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Justification results |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Validation report (title and reference) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conclusion: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Validation report to be attached |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Supplier approval on RFA final issue | Materials and processes responsible |  | PA responsible |  | Project responsible |  |  |  |  |  |  |  |
| 15 | Decision on RFA final issue | Materials and processes responsible |  | PA responsible |  | Project responsible |  |  |  |  |  |  |  |
|  | Customer agree/disagree |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Higher level customers (as necessary) agree/disagree |  |  |  |  |  |  |  |  |  |  |  |  |

              - : Example of RFA (Page 2 of 2)

(informative) 
Relationship between materials, mechanical parts, processes activities and programme phases

Feasibility phase (phase A)

In phase A, the materials, mechanical parts and processes product assurance tasks are carried out in order to:

- identify main programme constraints on materials, mechanical parts and processes,

define the policy, and

plan the product assurance tasks for the project definition phase.

Preliminary definition phase (phase B)

In phase B, the materials, mechanical parts and processes product assurance tasks are carried out in order to:

- define or identify requirements,

identify main new items needed and plan corresponding necessary actions for phase C,

plan the product assurance tasks for the detailed design, development, manufacturing, integration and test phase and prepare the materials, mechanical parts and processes plan as part of the PA plan, and

support preliminary design review.

Detailed definition and production phase (phase C or D)

In phase C or D, the materials, mechanical parts and processes product assurance tasks are carried out in order to:

- identify materials, mechanical parts and processes;

issue preliminary lists;

identify critical items;

establish or review RFA;

support mandatory inspection points identification;

establish evaluation programme, perform test or review test results;

establish validation, qualification, or verification programmes (e.g. perform tests or review test results);

support nonconformance processing (NRB, failure review board);

establish the as­designed lists;

support the critical design review;

support the qualification review;

establish the final as­designed (updated) lists;

support release of manufacture of flight hardware;

support final acceptance review.

Utilization phase (phase E)

In phase E, the materials, mechanical parts and processes product assurance tasks are carried out in order to:

- support the series manufacturing of recurring products,

support the investigation of operational phase anomalies, and

update the as­flown materials lists to incorporate the new materials that have been added or changed as a result of NCR activities. In particular, the PMP lists should include the actual materials flown on manned and reusable spacecraft and their payloads.

(informative)
MMPP documents delivery with respect to milestones

Scope of the Table F-1 is to present relation of documents associated to mechanical parts, material and processes activities to support project review objectives as specified in ECSS-M-ST-10.

This table constitutes a first indication for the data package content at various reviews. The full content of such data package is established as part of the business agreement, which also defines the delivery of the document between reviews.

The table lists the documents necessary for the project reviews (identified by “+”).

The various crosses in a row indicate the increased levels of maturity progressively expected versus reviews. The last cross in a row indicates that at that review the document is expected to be completed and finalized.

All documents, even when not marked as deliverables in Table F-1 are expected to be available and maintained under configuration management as per ECSS-M-ST-40 (e.g. to allow for backtracking in case of changes).

Table F-1 presents the reviews at which the different issues of the ECSS-Q-ST-70 and ECSS-Q-ST-70-01 documents are expected.

Other MMPP deliverable document requirements definitions (DRDs) are contained in the ECSS-Q-ST-70-XX documents.

: MMPP documents delivery w.r.t. milestones

| Document Title | Reviews |  |  |  |  |  |  |  |  |  |  |  | DRD Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MDR | PRR | SRR | PDR | CDR | QR | AR | ORR | FRR | LRR | CRR | ELR |  |
| Declared materials list (DML) |  |  |  | + | + | + | + |  |  |  |  |  | ECSS-Q-ST-70, Annex A |
| Declared mechanical parts list (DMPL) |  |  |  | + | + | + | + |  |  |  |  |  | ECSS-Q-ST-70, Annex B |
| Declared process list (DPL) |  |  |  | + | + | + | + |  |  |  |  |  | ECSS-Q-ST-70, Annex C |
| Request for approval (RFA) |  |  |  | + | + | + | + |  |  |  |  |  | ECSS-Q-ST-70, Annex D |
| Cleanliness Requirement Specifications (CRS) |  |  | + | + | + | + | + |  |  |  |  |  | ECSS-Q-ST-70-01, Annex A |
| Cleanliness and contamination control plan (C&CCP) |  |  |  | + | + | + | + |  |  |  |  |  | ECSS-Q-ST-70-01, Annex B |

Bibliography

| ECSS-S-ST-00 | ECSS system - Description, implementation and general requirements |
| --- | --- |
| ECSS-M-ST-10 | Space project management - Project planning and implementation |
| ECSS-M-ST-40 | Space project management - Configuration and information management |
| ECSS-Q-TM-70-52 | Space product assurance - Kinetic outgassing of materials for space |
| MIL-HDBK-454 | General Guidelines for Electronic Equipment |
| NASA STD-6001 | Flammability, odour, offgassing and compatibility requirements and test procedures for materials in environments that support combustion (previously NHB 8060 1C) |